3GPP TSG-RAN WG4 Meeting #116	R4-2511650

Bengaluru, IN, August 25 – 29, 2025

Agenda Item:	7.14.3.2

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on remaining issues for the first objective, particularly on optimizing CSSF outside gap in CA/DC scenarios for L3 measurement delay reduction.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 further discussed the following issues but did not conclude as shown below from WF [2]:

Issue 1-1-2: Clarify the network indication solution in solution 1 - Option 1 (OPPO): RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update. - Option 2 (Apple, CATT): network indication to enable/disable this CSSF enhancement of solution 1 can be semi-static while network indication of a specific SCC per-band for measurement can be dynamic. - (CATT, Apple)Network indication of a specific SCC per-band for measurement is dynamic indication based on MAC CE. Issue 1-1-3: Expand solution 1 to FR1 case - Option 1 (vivo, Apple, Nokia, CTC, Ericsson(only limit to serving cell measurement), CMCC, ZTE, MTK): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous. - (Nokia) Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - (Ericsson): limit the applicability to the serving cell measurements - (Nokia) RAN4 to discuss how to handle the neighbor cell measurement when network indicates to enable solution 1 for FR1 bands. - Option 2 (OPPO, CATT, HW, MTK): Not to expand solution 1 to FR1 band

For Issue 1-1-2, Solution#1 may or may not need two indications. The two indications are:

- One indication is for the NW to enable/disable this CSSF enhancement (i.e., measure 1 CC per band).

- Another one is for NW to indicate specific SCC per-band to be measured.

However, in our view both indications can be provided in one signaling and this signaling is preferred to be semi static such as RRC. This is also to avoid any additional processing delay might be needed if this indication was dynamic (i.e., activating/deactivating different CC every time if the indication was dynamic)

Proposal 1: On NW indication for solution#1, RAN4 to consider single semi static signalling which can include both: 1) indication for enable/disable this CSSF enhancement and 2) indicate which SCC per-band to be measured.

For Issue 1-1-3, if RAN4 decide to expand solution#1 to FR1, we support to limit the applicability of solution#1 to FR1 intra-band contiguous. Otherwise, we support not to expand solution#1 to FR1 band.

Proposal 2: Either not to support expanding solution#1 to FR1 or limit the applicability of solution#1 to FR1 intra-band contiguous CA (i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous).

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: On NW indication for solution#1, RAN4 to consider single semi static signalling which can include both: 1) indication for enable/disable this CSSF enhancement and 2) indicate which SCC per-band to be measured.

Proposal 2: Either not to support expanding solution#1 to FR1 or limit the applicability of solution#1 to FR1 intra-band contiguous CA (i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous).

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.