3GPP TSG-RAN WG4 Meeting #116	R4-2511714

Bengaluru, India, 25th – 29th August 2025

Certification Program Working Group

Liaison Statement

| To | 3GPP TSG RAN WG4 |
| --- | --- |
| CC | 3GPP TSG RAN WG5, GCF CAG, GCF PAG, GSMA, CCSA |
| Source | CTIA Certification Over-the Air (OTA) Working Group |
| Subject | LS on NB-IoT NTN OTA Testing |
| Date | August 18th, 2025 |

# Introduction

The CTIA Certification OTA Working Group would like to update relevant industry organizations on the NB-IoT NTN OTA test development.

# Discussion

The CTIA Certification OTA Working Group recently finalized the inclusion of NB-IoT NTN OTA testing for NTN bands 255 and 256 as informative in Version 9 of the CTIA OTA Test Plan.

CTIA Certification has approved TRP and TIS OTA test cases for NB-IoT NTN using the 3GPP TS 36.521-4 test procedures for maximum output power and reference sensitivity as the baseline for TRP and TIS, respectively. To address excessive test time concerns, especially the sensitivity search for TIS, the approved OTA test procedures deviate from current 3GPP TS 36.521-4 test procedures as follows:

- For both TRP and TIS, implementation of a co-located configuration was selected where the UE location is the same as the satellite location, and the kOffset is set to 0ms.

- For TRP and TIS, P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT transmits at maximum TX power

- For TIS, the uplink is control only, i.e. no user plane PUSCH data shall be configured.

The CTIA Certification OTA Working Group elected to include NB-IoT NTN OTA testing as only informative in Version 9 of the CTIA Certification OTA Test Plan to allow time for system provider validation of the deviations listed above. The CTIA Certification OTA Working Group expects to transition the NB-IoT NTN OTA testing to normative in Version 10 of the CTIA Certification OTA Test Plan pending successful authorization of test equipment; additionally, CTIA Certification plans to incorporate NR NTN OTA test cases in Version 10. Version 9 of the CTIA Certification OTA Test Plan is planned for release by the end of this year. Version 10 of the CTIA Certification OTA Test Plan is planned for release in the second half of 2026.

The CTIA Certification OTA Working Group kindly requests that RAN4 and RAN5 consider incorporation of the deviations listed above when considering the development of corresponding NB-IoT NTN OTA test cases.

The CTIA Certification OTA Working Group will provide further updates as the validation progresses and as the updates for Version 10 of the CTIA Certification OTA Test Plan are finalized.

# Contact Info

- Thorsten Hertel, Co-Chair, CTIA Certification OTA Working Group, Keysight

Technologies

Thorsten.Hertel@keysight.com

- Andrew Youtz, Co-Chair, CTIA Certification OTA Working Group, Verizon  andrew.youtz@verizonwireless.com

- Jose M. Fortes, Secretary, CTIA Certification OTA Working Group, Rohde & Schwarz

jose.fortes@rohde-schwarz.com

Date of the next CTIA Certification OTA Working Group meeting:

- September 16th, 2025, 11am-1pm EDT (Conference Call)

- October 27th, 2025, 1pm – 3pm CDT (F2F Meeting)