3GPP TSG-RAN WG4 Meeting #116	R4-2511751

# Bengaluru, IN, 25th – 29th Aug, 2025

Agenda item:			6.1

Source:	Moderator (Aalyria)

Title:	WF on NR IOT-NTN band emission requirements

Document for:	Agreement

# Introduction

Summary of contributions under the following Agenda Items:

| WI | Description | AI |
| --- | --- | --- |
| NR_NTN_combinedLband-Core | Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) | 6.13 |

# Way Forward: NR_NTN_combinedLband

#### Issue 1-1-2: Handling of UE requirements based on ETSI EN standards

- Way Forward:

  - Capture two sets of additional UE emission requirements and associated A-MPR

    - Set 1 (NS_09N, NS_10N): covers emission requirements in accordance with EN 301 681 v2.1.2

    - Set 2 (NS_13N, NS_14N): aligned with the information in the LS from ETSI TC SES SCN (R4-2509023, SES(25)000071) and should be used to ensure UE support for the new requirements is in place, when appropriate (i.e. if/when the new ETSI requirements get ratified).

      - NS_13N and NS_14N will be captured without applicable bands, for the moment (as per below), until further revision

    - These requirements and corresponding A-MPR may be later revised in the maintenance phase, as necessary

  - Include NOTE5 in the spec, as follows, to clarify requirement applicability:

| NS_13N5 | 6.5.3.3.10 |  | 5 |  |  |
| --- | --- | --- | --- | --- | --- |
| NS_14N5 | 6.5.3.3.10 |  | 5, 10, 15, 20 |  | Clause 6.2.3.9 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation NOTE5: NS_09N (n253, n250), NS_10N (n251) are currently in force. NS_13N (n253, n250), NS_14N (n251) may supersede NS_09N, NS_10N when the new ETSI requirements get ratified. |  |  |  |  |  |

#### Issue 1-1-3: Cross-device GNSS Protection

- Way Forward:

  - It is RAN4’s understanding that requirements for GNSS protection are already covered by the additional ETSI out-of-band emission requirements.

#### Issue 1-1-4: Additional TX OFF Measurement

- Way Forward:

  - Any additional requirements for Tx OFF measurement in the radio astronomy frequency range, if necessary, will be handled in the maintenance phase.