3GPP TSG-RAN4 Meeting #116	R4-2511772

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0823 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on FR2 RF requirements for NR_MIMO_Ph5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Sony, Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5 |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Feature CR for FR2 UE RF requirement, per agreement in R4-2505111: ‘Introduce the beam correspondence requirement when the IE unifiedTCI-StateType is configured as ‘separate’ and different QCL-TypeD reference signals are configured across the TCI states.’ The motivation is to give the UE the ability to transmit UL in a direction different from the one from which it receives PDSCH. The aim is to improved UL budget by increasing LOS probability and reducing range distances. The compromised UL condition is expected mainly for PC3 UEs, accordingly the requirement is written for PC3. FWAs are expected to be professionally installed and less dependent on UL budget improvement and are therefore not considered in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new beam correspondence requirement under ‘N’. ‘M is used for LPWUS per agreement in RAN4#113 The requirement is intended to be tested with the legacy single AoA test setup. The requirement is limited to PC3 UEs in this release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MIMO_Ph5 will have no UE RF feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2511344 |  |  |  |  |  |  |  |  |

/* Begin changes*/

4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| K | Simultaneous reception or transmission in multiple directions |
| N | Non-simultaneous reception or transmission in multiple directions |
| NOTE 1: Suffix D in this specification represents either polarized UL MIMO or spatial UL MIMO. RF requirements are same. If UE supports both kinds of UL MIMO, then RF requirements only need to be verified under either polarized or spatial UL MIMO. NOTE 2: Suffix K applies to simultaneous reception or transmission with different TCI states and different QCL-TypeD reference signals across the TCI states NOTE 3: Suffix N applies to reception and transmission with reference signalassociated with the UL TCI state is different from the QCL-TypeD associated with the DL TCI state |  |

/* End changes*/

/* Begin changes*/

6.6A	Beam correspondence for CA

For intra-band CA in FR2, the same beam correspondence relationship for beam management is supported across CCs in this release of the specification and no requirement is specified. Beam correspondence performance for intra-band CA is fulfilled if the beam correspondence requirements defined in clause 6.6 is met for non-CA case.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, with all CCs active with non-zero UL RB allocation, the following beam correspondence requirements apply for each CC:

1	The minimum peak EIRP requirement specified for UL inter-band CA in 6.2A, based on test conditions in clause 6.6. Reference signal power adjustments by MBS,n are replaced by ΔRIB,S,n, where referenced.

2	The common spherical coverage requirement specified for UL inter-band CA in 6.2A, based on test conditions in clause 6.6. Reference signal power adjustments by MBS,n are replaced by ΔRIB,S,n, where referenced.

6.6N	Beam correspondence for non-simultaneous reception or transmission in multiple directions

6.6N.1	General

Beam correspondence for non-simultaneous reception or transmission in multiple directions is the ability of the UE to select a suitable beam for UL transmission without relying on UL beam sweeping under the following conditions:

- the IE unifiedTCI-StateType is configured as ‘separate’

- the IE pl-Offset for separate DL/UL TCI state(s) is not configured

- the IE referenceSignal-r17 associated with the UL TCI state differs from the qcl-Type set to ‘typeD’ associated with the DL TCI state.

6.6N.2	(reserved)

6.6N.3	(void)

6.6N.4	Beam correspondence for power class 3

The beam correspondence requirement for power class 3 UEs in RRC_CONNECTED consists of three components: UE minimum peak EIRP (as defined in Clause 6.2.1.3) and UE spherical coverage (as defined in Clause 6.2.1.3). The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions:

-	If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6N.4.3.2.

6.6N.4.2	(reserved)

6.6N.4.3	Side Conditions

6.6N.4.3.1	Void

6.6N.4.3.2	Side Condition for SSB based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on SSB are only applied under the following side conditions for each TCI state:

-	The downlink reference signal SSB is provided, and CSI-RS is not provided.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.4.3.1-1.

6.6N.4.3.3	Void

/* End changes*/