# 3GPP TSG-RAN WG4 Meeting #116	                                                           R4-2511788
Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0090 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.102 for section 7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce IoT-NTN HPUE Rx requirements to the TS |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the Reference Sensitivity Degradation from PC3 to PC2 and PC1 for FDD UE category M1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rx requirement for IoT NTN HPUE is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

7.3A	Reference sensitivity power level for UE category M1

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 7.3A-1 and Table 7.3A-2 for category M1.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 253 | -102.7 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | -103 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 253 | -103.5 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

For power class 2 FDD UE category M1, certain degradation of the reference sensitivity in Table 7.3A-1 is allowed. The maximum amount of degradation is specified in Table 7.3A-1a.

Table 7.3A-1a Reference Sensitivity Degradation from PC3 to PC2 for FDD UE category M1

| NTN Band | Reference Sensitivity Degradation (dB) | Duplex Mode |
| --- | --- | --- |
| 256 | 0.5 | FDD |
| 255 | 1 | FDD |
| 254 | 0 | FDD |

The reference sensitivity (REFSENS) requirement specified in Table 7.3A-1, Table 7.3A-2 and 7.3A-1a shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3A-3 and with default TX-RX carrier center frequency separation except for cases specified in Table 7.3A-4.

NOTE:	Table 7.3A-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G] (informative).

Table 7.3A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 256 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 254 | 61 | FDD and HD-FDD |
| 253 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 1.4 MHz | 161.4 MHz, 218.6 MHz |
| 255 | 1.4 MHz | -68.9 MHz, -134.1 MHz |
| 254 | 1.4 MHz | 858.4 MHz, 888.6 MHz |
| 253 | 1.4 MHz | -144.4 MHz, -155.6 MHz |

<End of change>