3GPP TSG-RAN4 Meeting #116	R4-2511811

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2983 | rev | 2 | Current version: | 16.23.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 Rel-16. To add indication of modified MPR behavior [PHS_System_NRonly] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PHS system in Japan is now closed but the DECT service in the same range 1884.5-1915.7 MHz must still be protected. Introduce a bit in the table modified MPR behaviour allowing a UE indicate support of the corresponding NS and A-MPR changes made in the later (Rel-19). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex L.1: introduce a bit in the modified MPR behaviour -- (shall be set by UEs compliant with Rel-19) -- may be set by UEs compliant with earlier releases back to Rel-16 In this way the NW is also aware of the reduced A-MPR if the bit is set. Isolated impact: optional support of an MPR change that is specified in a later release |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs of this release would not be able to indicate support of the modified emission limits that require lower power back-off (A-MPR), which would unnecessarily reduce available UE maximum output power. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

Annex L (normative):
ModifiedMPR-Behavior

# L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v19.3.0 | - This bit may be set to 1 by a UE supporting this version of the specification. |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | - EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |

Annex M (informative):
Change history

< end of changes >