# 3GPP TSG-RAN WG4 Meeting #116 　　　　　　　　　　　　　　　　　	R4-2511834

Bengaluru, India, 25th  – 29th August, 2025

Source: 	NTT DOCOMO INC., Skyworks, Qualcomm, MediaTek

Title:	TP for TR 38.746 adding HPUE CA_n1A-n78A-n79A with 2UL

Agenda item:	6.8.3

Document for:	Approval

- Introduction

This paper provides TP for HPUE CA_n1A-n78A-n79A with dual ul for TR 38.786. All PC2 fallbacks have been specified.

2 Text Proposal

< Start of Text Proposal>

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.X	CA_n1-n78-n79

### 6.X.1	Configurations

Table 6.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A-n79A | CA_n1A-n78A7 CA_n1A-n79A7 CA_n78A-n79A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.X.2	Maximum output power

Table 6.X.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n1-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.X.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.X.3.1	Power class 2 case a, b, c, d

#### Based on calculation,

- IMD3/IMD5 due to UL CA_n1A-n78A falls into n79 DL. The IMD3 value is calculated by adding 9dB to the PC3 MSD value, resulting in 24.9dB.

- IMD5 due to UL CA_n1A-n79A falls into n78 DL. The IMD5 value is calculated by adding 12.4dB to the PC3 MSD value, resulting in 17dB.

- IMD3/IMD4/IMD5 due to UL CA_n78A-n79A falls into n1 DL. The IMD3 value is calculated by adding 9dB to the PC3 MSD value, resulting in 24.6dB. PC2 IMD4 is 3.8dB.

#### The MSD exception is defined as below.

Table 6.x.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4870 | 24.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3490 | 17 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 24.6 | FDD | IMD31 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 3.8 | FDD | IMD41 |
|  | n78 | 3590 | 10 | 50 | 3590 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |

### 6.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

< End of Text Proposal>

3	References

[1] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone". V19.2.0

[2] 3GPP TR 38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s)”. V0.4.0

[3] R4-2507935, WF on new MSD rules