# 3GPP TSG-RAN WG4 Meeting #116				R4-2511859

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.22.1

Source:	Moderator (CMCC)

Title:	WF on A-IoT device RF requirements and OTA test methods

Document for:	Approval

- System parameters

## Topic 2-1: R2D bandwidth

Issue 2-1-1: R2D transmission bandwidth

Agreement:

Define asymmetric guard band as below

Table : Minimum guardband (kHz)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

## Topic 2-2: D2R bandwidth

Issue 2-2-1: D2R bandwidth

Agreement:

- Following equations will be captured as informative Annex in TS.

For BS D2R CBW: D2R CBW for BS (kHz) = ceiling ((2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9) =ceiling ( (2+2R)/Tb × (1+∣SFO∣)/0.9) =ceiling ( (1+R)/ (Tc ×R) × (1+∣SFO∣)/0.9) (Eq. 4) The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band).

For device D2R CBW D2R CBW for device (kHz) =ceiling (2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO) =ceiling ( (2+2R)/Tb × (1+∣SFO∣)) =ceiling ( (1+R)/ (Tc ×R) × (1+∣SFO∣)) (Eq. x)

  - Capture the following channel bandwidth tables in TS:

| Device D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Norminal D2R transmission Bandwidth without SFO(kHz) | Norminal Small frequency shift without SFO(kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 17 | 25 | 42 | 75 | 141 | 273 | 534 | 1065 |  |
| 30 |  | 33 | 50 | 83 | 149 | 281 | 545 | 1073 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

| BS D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Norminal D2R transmission Bandwidth without SFO (kHz) | Norminal Small frequency shift without SFO(kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 19 | 28 | 46 | 83 | 156 | 303 | 596 | 1183 |  |
| 30 |  | 37 | 55 | 92 | 165 | 312 | 605 | 1192 |  |
| 60 |  |  | 74 | 110 | 184 | 330 | 624 | 1210 |  |
| 120 |  |  |  | 147 | 220 | 367 | 660 | 1247 |  |
| 240 |  |  |  |  | 294 | 440 | 734 | 1320 |  |
| 480 |  |  |  |  |  | 587 | 880 | 1467 |  |
| 960 |  |  |  |  |  |  | 1174 | 1760 |  |
| 2880 |  |  |  |  |  |  |  |  | 3520 |

## Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement:

- Define 10kHz channel raster for both D2R and R2D.

  - Note: channel raster for D2R will only be captured into BS spec 38.194

- No channel raster offset

## Topic 2-4: Others

Issue 2-4-1: whether channel spacing is needed or not

Agreement:

- No channel spacing.

Issue 2-4-2: maximum number of devices allowed to communicate simultaneously with one A-IoT BS

Agreement:

- No discussion is needed.

- Device RF requirements

## Topic 3-1: Transmit output power

Issue 3-1-1: Transmit output power

Agreement:

- Define requirements only at the peak antenna gain direction.

  - When input CW level is -27dBm, the backscatter loss<=10dB for OOK and <=6dB for BPSK

  - When input CW level is-10dBm, the backscatter power is larger than -25dBm for OOK, and -21dBm for BPSK respectively.

## Topic 3-2: Modulation quality

Issue 3-2-1: SFO requirement

Agreement:

No explicitly define SFO requirement in RF spec. Define D2R CBW bandwidth calculation equations in Annex.

## Topic 3-3: Emission requirements

Issue 3-3-1: SEM requirements

Agreement:

Define flat SEM requirements, i.e. 10dBc, The RBW is 1SB

  - The carrier power includes the two sidebands centred at +/-SFS, but excludes the spectrum around the carrier frequency.

Issue 3-3-1: spurious emission requirements

Agreement:

Table 3: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 5 GHz | -30 dBm | 1 MHz |  |
| 5 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 1 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 1 GHz and less than or equal to 2.55 GHz. |  |  |  |

- OOB boundary:

  - For CBW<1.4 MHz: max of 500kHz and 10 times NB where NB equals to D2R CBW

    - For CBW>=1.4 MHz: 7.5MHz assuming 3M CBW

- For testing:

  - Only using limited DSB and SFS combination for testing with -5 dBm CW input power.

    - SFS = 480kHz with 15kHz, 2SB transmission bandwidth need to be tested

## Topic 3-4: Reference sensitivity

Issue 3-4-1: Reference sensitivity

Agreement:

- EIS at peak antenna gain direction is -34dBm. Additional peak EIS levels can be added in the future if needed

- EIS spherical coverage requirement is defined as sensitivity over partial sphere, i.e. XdBm at Y solid angle range partial sphere . Y is suggested to be +-45 degree with respect to the bore sight direction (or UE declared direction),  X is suggested to be 5.5dB worse than peak EIS.

  - For testing

    - 90% success rate.

    - Use CFRA for REFSENS test procedure

    - Allow set time + SFO for the device response time.

    - Leave number of repeats to reach 90% success rate for the RAN5 to determine.

    - Further discuss if false alarm test can be considered in demod.

- FRC is listed as below (note: it does not mean UE should be tested for all specified FRC)

|  | SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- | --- |
|  | PRB | 1 | 2 | 3 | 4 |
| SIP | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | Depending on the size of the MAC PDU of A-IoT paging message indicating CFA |  |  |  |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | OOK |  |  |  |  |
|  | M | 6 | 6 | 6 | 6 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
| chip number except for SIP, padding |  | 228 | 228 | 228 | 228 |
| Padding |  | 6 | 12 | 2 | 2 |

## Topic 3-5: Others

Issue 3-5-1: Maximum input power

Agreement:

Max input level is 30dB higher than peak EIS, detailed value is based on conclusion of min sensitivity.

- OTA

Performance metric

Agreement:

- Performance metric of Tx requirements:

EIRPθ,ϕ=EIRPθθ,ϕ|cwθθ,ϕ+EIRPϕθ,ϕ|cwθθ,ϕ+EIRPθθ,ϕ|cwϕθ,ϕ+EIRPϕθ,ϕ|cwϕθ,ϕ

  - Where EIRPθ and EIRPϕ are the EIRP in the corresponding θ and ϕ polarizations, CWθ and CWϕ are the incident CW in the corresponding θ and ϕ polarizations,

  - For backscatter power measurement, the EIRP only contains the power of 1st sidebands within D2R channel bandwidth and excludes power of CW.

- Performance metric of Rx requirements

EISθ,ϕ=1(1EISθθ,ϕ+1EISϕθ,ϕ)

  - Where EISθ and EISϕ are the EIS in the corresponding θ and ϕ polarizations.

  - The EIS partial sphere coverage metric is defined as the maximum R2D EIS radiated in the Theta and Phi range from partial surface within ±45° angular width degrees.

Visualization of Partial sphere within ±45° angular range

Device positioning guidelines

Agreement:

| Test condition | DUT orientation | Diagram |
| --- | --- | --- |
| Free space DUT | α = 0º; β = -90º; γ = 0º |  |