# 3GPP TSG-RAN WG4 Meeting #116	R4-2511890

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.17.1

Source: 	Qualcomm Incorporated

Title: 	WF on AI/ML for air interface for [116][131]

Document for:	Approval

# Introduction

This WF captures the agreements for the discussion carried out on AI/ML under the [116][131]NR_AIML_air thread.

# Agreements

## CSI reporting requirement framework for CSI prediction

### Agreements in main session:

Issue 1-5: Scheduling delay

Agreement:

  - The delay from when the UE prediction is send until the time the TE applies is n+4 (slots) for FDD

### Agreements in ad-hoc session (R4-2511891)

Issue 1-1: Performance monitoring

Agreement:

RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction

FSS which requirements are to be introduced.

Issue 1-2: Requirement baseline for monitoring

Agreement:

Introduce the following requirements for CSI performance monitoring:

- Reporting delay

  - FFS how the delay is defined

- Reporting accuracy

  - FFS on whether accuracy requirement can be defined/checked

- Mapping table – To be checked whether RAN1 captures or RAN4

- If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

Issue 1-4: Reporting delay requirement

Agreement:

wait for RAN1 decision

## RRM core requirement and testing framework for beam management

### Agreements in main session:

Issue 2-2: Prediction report delay

Agreement:

  - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay

    - The reporting delay includes the inference delay

  - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

Issue 2-3: TCI State Handling

Agreement:

  - On Detectability and SNR conditions:

    - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command

    - The TCI state remains detectable during the TCI state switching period

    - The SSB associated with the TCI state remain detectable during the TCI switching period

      - SNR of the TCI state ≥ -3 dB

  - Time conditions:

    - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement

  - Agreement on QCL relationship

    - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

    - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

Issue 2-9: Metrics/KPIs for beam ID prediction

Agreement:

    - If RSRP prediction is reported, absolute/relative RSRP requirements applies to

      - reported beams where SNR side condition is met

      - The exact RSRP reporting including both absolute and relative will be defined in RAN1

      - The related test can only apply to the top-K, where the value of K is FFS, beams.

        - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K

        - The feasibility of the test will be further discussed and decided.

### Agreements in ad-hoc session (R4-2511891)

Issue 2-1: Measurement period for inference

Agreement:

observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report)

For case 1 reuse M,N,P from legacy measurement requirements

For case 2, use T*M, N P  (M,N,P same as legacy measurement requirement)

FFS on T value, T can also be 1. T can also be based on capability

This observation period is to be used for the prediction delay requirement

Issue 2-4: Activation delay

Agreement:

- UE has to be ready to start measurements for inference after sending RRC reconfiguration complete

- Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay

- This applies to all reporting schemes

Issue 2-6: Test system setup

Agreement:

Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing)

Single AoA tests can also be performed in the multi AoA based RRM test setup

Further analyze what spatial channel models can be emulated with this test setup

  - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup

Single AoA tests are not precluded

Issue 2-7: Test system channel model

Agreement:

Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber.

Take CDL-C Umi as one example

Other channel models can be further discussed

  - RRM core requirement and testing framework for Positioning accuracy enhancement

### Agreements in ad-hoc session (R4-25011891)

Issue 3-1: Reporting Delay Requirements for case 1

Agreement:

Take the framework of the existing reporting delay requirements

- Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

- Check until next meeting on the number of samples needed for the measurements

Issue 3-3: Report mapping for UL SRS-TDCP measurement

Agreement:

Adopt the mapping for UL SRS-TDCP as proposed in R4-2510759

Issue 3-4: Report mapping for UL SRS-TDCT and UL SRS-TDCP measurements

Agreement:

Technically endorse CR in R4-2510760, to be later merge with the rest of positioning requirements