# 3GPP TSG-RAN WG4 Meeting #116	R4-2511892

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.17.1

Source: 	vivo, Qualcomm Incorporated

Title: 	AH minutes for BM simulation assumption in AI/ML for air interface [116][131]

Document for:	Approval

- Introduction

The discussion on the AI/ML study is organized in [131] in RAN4#116. The ad-hoc meeting will discuss some issues from the moderator summary in [1] and the issues observed from summary of simulation results for BM [2].

- Discussion

# Topic #2: RRM core requirement and testing framework for beam management

    - Sub-topic 2-5

Issue 1: How to proceed with defining metrics based on Simulation results

- Step 1: Continue aligning on the following aspects

  - Dataset

    - Alt 1: Companies should perform simulations based on the reference dataset and submit results in the next meeting.

    - Alt 2: Agree on dataset-related parameter settings in this meeting, and companies should submit results based on the aligned dataset parameters in the next meeting. The dataset-related parameters potentially include:

      - Dataset size

  - Data distribution of

    - ground truth RSRP corresponding to each top-K predicted beams and ground truth RSRP of the strongest genie-aided beam

  - RF error modeling

    - Alt 1: Two RX ports (corresponding to V and H) with independent RF error values for each RX port

    - Alt 2: One RX port

  - RX beam assumption

    - Alt 1: Do not define RX beam pattern. Each company selects RX beam pattern on its own.

    - Alt 2: Assume DFT RX beam pattern

      - FFS Number of RX beams and peak direction of DFT RX beams

  - Target test scenario to define accuracy requirement

    - Alt 1: Use the worst-case performance of two test datasets where datasets come from following scenarios:

      - Dense urban macro

      - CDL-based channel model (details are TBD)

        - Note: The UMi CDL-C channel of Table D.1-1 of 38.151 can be used as a starting point (shown in Table 1)

    - Alt 2: Use the performance of one test dataset where dataset is based on dense urban macro.

      - Note: This is currently under consideration and captured in the recently agreed simulation assumption of R4-2508081

    - Alt 3: Use the performance of one test dataset where dataset is based on CDL based channel model.

      - Note: The UMi CDL-C channel of Table D.1-1 of 38.151 can be used as a starting point (shown in Table 1)

- Step 2: Based on the aligned assumption in Step 1, in the next meeting, decide the value of metric based on the simulation results submitted by companies

  - The source for the metric value can be determined by directly taking the average of the results from companies

    - applying certain criteria to exclude results with excessively large deviations

Discussion:

Vivo: we uploaded dataset but other companies have not. We know dataset has big impact on the simulation results

QC: are you suggesting to create a mix dataset or just to align the size?

MTK: how to test? The requirements will be used for the test metric? We will need to redo the sims to decide the test requirements.

QC: mixed dataset is not agreeable to us. Each company would use their own Rx beam codebook for training.

MTK: we used vivo dataset and compared the results to ours, prediction metric of top-1+x dB and results were good.

QC: RSRP distribution cannot be aligned because it has dependence on the UE Rx codebook, gain, etc.

QC: for Rf error, alternative 1 with independent error pre Rx branch

Vivo: it’s still implementation independent

Nokia: RF error should be clarified

QC: we should also simulate CDL channels since the test is likely to be some variation of CDL

Agreement:

Dataset alignment: align the size of the training dataset for the simulation

RF error modeling: 2Rx chains with independent RF error per chain(same distribution for each chain)

Issue 2: The target scenarios to be defined

Background:

Scenario 1: CSI-RS predicts CSI-RS, narrow-to-narrow prediction

Scenario 2: SSB predicts CSI-RS, wide-to-narrow prediction

- Option 1: Only to consider scenario 1

- Option 2: both scenario 1 and scenario 2 are considered

Discussion:

QC: We are ok with option 2

CMCC: we support option 2

Nokia: we are also ok

Agreement:

Target both scenarios 1 and 2 for requirement definition

Issue 3: Use which simulated case for the metric definition

Background:

Table Clarification of Simulation Cases

|  | Training dataset | Model input for Inference | Ground-truth for training | Ground-truth for inference |
| --- | --- | --- | --- | --- |
| Case 1 | No error | No error | No error | No error |
| Case 2a | No error | With error | No error | No error |
| Case 2b | No error | With error | No error | With error |
| Case 3 | With error | With error | With error | With error |

- Option 1: Use case 3

- Option 2: Use 2b

Discussion:

QC: we are fine with case 3

Agreement:

Use case 3 to define requirements

# Reference

[1] R4-2509098, Topic summary for [116][131] NR_AIML_air_part1, Moderator (Qualcomm Incorporated), RAN4#116

[2] R4-2511865, Summary of beam management simulation results