3GPP TSG-RAN4 Meeting #116	R4-2511896

# Bengaluru, IN, Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to 38.101-1, LPWUS ACS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Nokia, Murata |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS |  |  |  |  |  | Date: |  |  | 2025-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ‘Feature’ CR component: ACS requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce ACS requirements are introduced for LP-WUR/WUS feature. UE is expected to withstand the same jammers whether it is receivng PDSCH or detecting LPWUS. The LP-WUR/WUS ACS requirement is specified with a 5 MHz interferer for all bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE RF feature is not complete to support LP-WUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* Begin changes*/

## 7.5M	Adjacent channel selectivity for LP-WUR/WUS

The adjacent channel selectivity (ACS) is a measure of a UE’s ability to detect an LP-WUS signal at its configured RBs inside the configured channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the configured channel. ACS is the ratio of the receive filter attenuation on the LP-WUS frequency to the receive filter attenuation on the adjacent channel(s).

The UE that indicates either [type1 LR] or [type2 LR] shall fulfil the minimum requirements specified in Table 7.5M-1.

These requirements apply for any SCS specified in table [5.3M.2-1]. The requirement in table 7.5M-1 shall be verified for each set of test parameters in Table 7.5M-2 and Table 7.5M-3, respectively. For these parameters, the MDR criterion in clause [7.1M] shall be met with the side conditions in clause [7.1M].

Table 7.5M-1: ACS requirement

|  | Units | Value |
| --- | --- | --- |
| ACS for [type1 UE] | dB | 27.0 + Z |
| ACS for [type2 UE] | dB | 32.0 |
| NOTE1: Z = 0.0 with 7 guard RBs for 15kHz SCS and Z = - 4.5 with 0 guard RB at each side of the LP-WUS signal |  |  |

Table 7.5M-2: Test parameters, case 1

| RX parameter | Units | Value |  |
| --- | --- | --- | --- |
| Power in LP-WUS transmission BW config | dBm | [REFSENSLP-WUS] 3 + 14.0 dB |  |
| Pinterferer4 | dBm | [REFSENSLP-WUS] 3 + ACS4 + 16.0 dB |  |
| BWinterferer | MHz | 3 | 5 |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to ¦Finterferer¦/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 3: REFSENSLP-WUS is the value specified in clause [7.3M.2] according to type 1 LR and type 2 LR respectively. NOTE 4: Defined in table 7.5M-1 according to type 1 LR and type 2 LR respectively. |  |  |  |

Table 7.5M-3: Test parameters for case 2

| RX parameter | Units | Value |  |
| --- | --- | --- | --- |
| Pinterferer | dBm | -25.0 |  |
| Power in LP-WUS transmission BW config | dBm | Pinterferer – ACS3 - 2.0 dB |  |
| BWinterferer | MHz | 3 | 5 |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to ¦Finterferer¦/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 3: Defined in table 7.5M-1 |  |  |  |

/* End changes*/