# 3GPP TSG-RAN WG4 Meeting #116                                                                  R4-2512119

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.3.1

Source:	Moderator (Apple)

Title:	WF on RRM requirements for NR_LBCA_Sw

Document for:	Approval

# Topic #1: Core part for R19 LB CA via switching (7.3.4)

### Issue 1-1: General

Issue 1-1-1: RRM requirement applicability for R19 LB CA via switching

- Agreement in online session:

  - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC:

  - a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

  - b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

  - c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Issue 1-1-2: switching pattern applicability for dormant SCell

- Proposal 1 (vivo): Switching pattern is not applied for dormant SCell.

- [Agreement]:

  - No requirement is defined for dormant SCell in LB-CA via switching.

### Issue 1-2: Interruption or scheduling restriction requirement

Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC

Agreement: RAN4 to confirm the agreement from the last RAN4 meeting

  - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered

Agreement:

  - According to the WID, RANA only defines requirements for CA with FDD PCC and one SDL SCC.

Issue 1-2-3: Interruption or scheduling restriction requirement (if needed) for carrier switching between FDD CC and SDL CC

Agreement:

  - This issue is closed.

### Issue 1-3: SDL SCell related requirement

#### Activated SDL SCell L3 measurement

Issue 1-3-1:  Requirement design of activated SDL SCell L3 measurement

Agreement in online session:

    - if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied.

    - reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA.

    - Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA.

Agreement in online session:

agree on the following approach, and further update on the wording is not precluded. Further discuss on the CR drafting.

    - if the activated SCC measurement is MG-less in LB-CA, Kp factor can be updated for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA.

    - Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following:

      - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

        - For a window W of duration max(SMTC period,  xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion:

          - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and

          - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W.

          - No requirement when Navailable = 0.

#### Deactivated SDL SCell L3 measurement

Issue 1-3-2: Interruption requirement design of deactivated SDL SCell measurement

[Agreement]:

- For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level),

    - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19

    - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19

- Missed ACK/NACK rate

For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length.

#### SDL SCell activation/deactivation

Issue 1-3-3: interruption or scheduling restriction during SDL SCell activation/deactivation

[Agreement]:

No requirement for interruption or scheduling restriction during SDL SCell activation/deactivation. This issue is closed.

Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA

- [Agreement]:

  - FFS this issue in maintenance stage.

Issue 1-3-5: Activation of Switching Pattern

[Agreement]:

- Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure:

  - For the SCell which is in activation procedure,

    - The RRC configured switching pattern for SDL SCell is applied.

    - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length

    - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length

      - Option 1: TLBCA =0ms

      - Option 2: TLBCA =1ms

      - Option 3: TLBCA =4ms

  - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options.

    - For MAC CE based SCell activation, according to the switching pattern,

      - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length.

      - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length.

      - Other options are not precluded.

    - For RRC based direct SCell activation, according to the switching pattern,

      - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length.

      - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length.

      - Other options are not precluded.

Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

Issue 1-3-5a: Deactivation of Switching Pattern

[Agreement]:

- The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length).

  - Option 1: TLBCA =0ms

  - Option 2: TLBCA =1ms

  - Option 3: TLBCA =4ms

- FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

#### SDL SCell L1 requirement

Issue 1-3-6: BFD/CBD requirements for SDL SCell

- Proposal 1 (CATT): The BFD/CBD measurement requirement for SDL SCell in LB CA via switching can be defined based on the legacy requirement with the number of measurement sample being updated. The new measurement sample equals to the legacy value plus X, where X is the number of SSB/CSI-RS occasions that are not covered by SDL duration in switch pattern.

- Proposal 2 (Apple, HW):

  - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell.

  - New P_LB factor is used to replace the P factor in the existing BFD/CBD requirement, where,

    - W window is max(TL1, xRP_max, switching pattern periodicity)

    - P_LB= Ntotal_LB/ Navailable_LB

    - Ntotal_LB is total number of BFD-RS or CBD-RS in W window.

    - Navailable_LB is number of available BFD-RS or CBD-RS in W window.

- Proposal 3 (LGE): RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggered.

- Proposal 4 (vivo, MTK): L1 measurement requirements for BFD/CBD are not defined for SDL SCell via switching.

- Proposal 5 (Ericsson): For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

[Agreement]:

  - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching

    - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell.

    - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where,

      - W window is max(TL1, xRP_max, switching pattern periodicity)

      - P = Ntotal/ Navailable

      - Ntotal is total number of BFD-RS or CBD-RS in W window.

      - Navailable is number of available BFD-RS or CBD-RS in W window.

      - No requirement when Navailable = 0.

  - FFS in maintenance stage for P3.

Issue 1-3-7: TCI state switch delay requirement for SDL SCell

[Agreement]:

  - RAN4 to define the TCI state switch delay requirement for LB-CA via switching

  - In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE.

Issue 1-3-8: L1-RSRP/SINR measurement requirements for SDL SCell

[Agreement]:

  - follow the principle in issue 1-1-1 to define the L1-RSRP/SINR requirement for SDL SCell.

  - P factor in the existing L1-RSRP/SINR measurement requirements can be updated for LB-CA via switching, where,

    - W window is max(TL1, xRP_max, switching pattern periodicity)

    - P = Ntotal / Navailable

    - Ntotal is total number of L1-RSRP/SINR RS of SCell in W window.

    - Navailable is number of available L1-RSRP/SINR RS overlapping with SCell duration according to the RRC configured switching pattern in W window.

    - No requirement when Navailable = 0.

#### Others for SDL SCell

Issue 1-3-9: BWP switching for SDL SCell

- Proposal 1 (Ofinno):

  - Clarify the UE behavior by adding the following text in clause 8.6.2:

    - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot refers to the first DL or UL slot, which:

      - is available after the completion of the BWP switching and

      - is assigned to the same serving cell on which the active BWP switching occurs.

  - Clarify the UE behavior by adding the following text in clause 8.6.3:

    - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell.

[Agreement]:

FFS this issue in maintenance stage.

### Issue 1-4: Impact on FDD PCell RRM

Issue 1-4-1: RLM requirement for PCell

[Agreement]:

  - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA.

  - P factor in the existing RLM measurement requirements can be updated, where,

    - W window is max(TL1, xRP_max, switching pattern periodicity)

    - P = Ntotal/ Navailable

    - Ntotal is total number of RLM-RS in W window.

    - Navailable is number of available RLM-RS in W window.

    - No requirement when Navailable = 0.

  - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching

[Agreement]:

- The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA.

# Topic #2: Performance part for R19 LB CA via switching (7.3.5)

### Issue 2-1: General

Issue 2-1-1: Accuracy requirement impact of LB-CA

[Agreement]:

Existing measurement accuracy requirements apply for LBCA operation through switching.

Issue 2-1-2: Work plan for performance part (TC design) for LB-CA via switching

[Agreement]:

- Work plan in R4-2509990 is agreed.