3GPP TSG-RAN WG4 Meeting #116	                                                	         R4-2512121

Bengaluru, India, August 25 - 29, 2025

Agenda item:			7.11.7

Source:	Apple

Title:		WF on RRM requirements for NR_NTN_Ku_bands

Document for:	approval

# Topic #1: RRM impact of NR NTN on Ku bands

Issue 1-2: satellite switch requirements

Issue 1-2-1: Hard satellite switching with re-sync

Agreement:

- The components in hard satellite switching delay requirements refer to legacy hard satellite switching delay and inter satellite handover delay.

Issue 1-2-2: soft satellite switching with re-sync

Agreement:

- Agree to introduce RRM requirements for soft satellite switching with re-sync for the VSAT UE with electronic beam steering:

Tsoft_switch = max(t-service-t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing

- The requirements in this clause do not apply if the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-ServiceStart-r18 [and ssb-TimeOffset-r18] is acquired by the UE for the first time to the first slot corresponding to t-ServiceStart-r18 is is less than [1] s.

  - Further refinement on the wording is not precluced.

  - Further discussion on the value of [1] is not precluded.

- Extend the period of the scheduling restriction by two sided TRX_switch, TRX_switch = 1 symbol.

Issue 1-3: inter-satellite cell reselection requirements (section 4.2C).

Agreement:

- No consensus to define inter-satellite cell reselection requirements in Rel-19.

- No consensus to define non-blind handover requirement in Rel-19.

Issue 1-4: RLM (section 8.1C)

Agreement:

- LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology.