3GPP TSG-RAN WG4 Meeting #116	R4-2512126

Bengaluru, India, August 25th – August 29th, 2025

Agenda item:			7.19.1

Source:	Nokia

Title:	Adhoc session minutes for FS_NR_AIML_Mob_Part1

Document for:	Approval

# Introduction

This document contains the ad-hoc meeting minutes for the discussion on AI/ML mobility SI. The ad-hoc meeting discussed some of the topics from the moderator’s summaries [1][2].

# Discussion.

## Topic#1: General Aspects

### Issue 1-1: Spatial domain prediction

- Background:

Following agreement was reached in RAN2#130:

Agreements RAN2#130: - The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). - The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

- Background:

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to study the specification impact of intra-frequency spatial domain prediction in beam dimension for UE sided models.

  - Proposal 2 (ZTE Corporation, Sanechips): RAN4 to study the RRM impact of intra-frequency spatial domain prediction (scenario 6) in beam dimension.

  - Proposal 3 (Apple): Spatial-domain prediction for mobility requires complex cross-cell modeling across diverse antenna configurations, frequency bands, and SINR regimes, and is highly sensitive to both measurement errors and generalization errors. Addressing these challenges demands large multi-cell training datasets and robust error-tolerant model designs. We propose postponing this study to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods.

  - Proposal 4 (ZTE Corporation, Sanechips): RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case in cell dimension and beam dimension.

  - Proposal 5 (CMCC): it is proposed to firstly focus on scenario 2, 3, 4.

- Candidate Options:

  - Option 1: RAN4 to study the specification impact of intra-frequency spatial domain prediction (Scenario 6) in beam dimension for UE sided models

  - Option 2: RAN4 to study the specification impact of intra-frequency spatial domain prediction (Scenario 6)  in beam dimension for UE sided models as second priority.

  - Option 2: RAN4 to postpone the study on the impact of intra-frequency spatial domain (Scenario 6) to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods

- Recommended WF:

RAN4 to discuss the above-listed candidate options.

Issue 1-1: Spatial domain prediction

Due to time limitation, RAN4 has not studied potential UE impact for spatial domain prediction from network side model.

## Topic#2: RAN4 requirements for RRM Measurement prediction

### Issue 2-1: Impacts of measurement error on prediction accuracy

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

Furthermore, following WF was suggested in RAN4#114bis meeting:

WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (vivo): On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

    - For BB error modeling:

      - Link-level simulation approach is considered

    - For RF error,

      - For error modeling, the following two modelings can be the candidates. Further down-selection can be done in WI phase

        - Alt 1: Uniform distribution

        - Alt 2: Truncated Gaussian distribution(μ=0, σ^2=[X]), value [X] can be further determined respectively for FR1 and FR2

    - RF error application, each of Tx beam is independent

  - Proposal 2 (vivo): The measurement error analysis can consider the impact on prediction performance

    - [Case 1]: when errors are only added to the model input during the prediction phase

    - [Case 2]: When errors are added to the model input both during both the training phase and prediction phase

    - Whether to consider adding errors to the ground truth depends on the discussion on the definition of ground truth

  - Proposal 3 (CMCC): for impact on measurement error on prediction accuracy, the discussion on beam prediction can be used as baseline. In detail, the BB error can be derived based on link level simulation assumption, and RF error can be modelled as truncated Gaussian distribution.

  - Proposal 4 (Xiaomi): RAN4 to study the factor and impact of measurement error on RRM prediction.

  - Proposal 5 (LG Electronics Inc.): RAN4 to consider only Gaussian error model for RF and BB error model.

  - Proposal 6 (Apple): RAN4 to adopt a measurement error with a compact four-term model. The first term is a slow UE-wide bias that is constant for one handset on a given carrier band. A second term adds a small residual unique to each transmit beam, whose correlation across beams is controlled by a single knob. The third term is fast thermal-and-estimator noise, redrawn for every measurement occasion with variance taken from the existing SINR-dependent link-level table or through LLS simulations. A fourth term introduces additional zero-mean variance proportional to a chosen neighbor-power metric, modelling interference.

  - Proposal 7 (CATT): RAN4 to evaluate the impact of baseband error using link-level simulation with different SNR levels considered. For RF error, a uniform or Gaussian distribution can be used.

  - Proposal 8 (Nokia): RAN4 to capture the following error modelling principles in the TR:

    - RF and baseband measurement error modelling may be considered.

    - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

    - Alternative modelling approaches (such as based on link-level simulations) are not precluded and can be explored in the Work Item phase.

  - Proposal 9 (MediaTek): For AI mobility, in FR1, measurement error should be added as:

    - BB error:

      - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

      - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

    - RF error:

      - Modelled as Gaussian distribution under ±3 dB RF error (μ=0, σ2=3).

      - A random RF error is generated for all the beams of the same frequency layer per SMTC.

  - Proposal 10 (Ericsson): Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

    - RF error in dB for FR1 is characterized by Gaussian distribution with μ=0, σ2= 2.5 truncated at ±2.5.

    - RF error in dB for FR2 is characterized by Gaussian distribution with μ=0, σ2= 4 truncated at ±4.

  - Proposal 11 (OPPO): Take the following option 1 as baseline to model the measurement error and the details can be left to WI when discussing the simulation:

    - Option 1:

      - BB error:

        - Use link level simulation assumptions to generate L3-RSRP difference as baseband error.

      - RF error:

        - Alt1: Modelled as uniform distribution

        - Alt2: Modelled as truncated Gaussian distribution Under ±[X] dB RF error (μ=0, σ^2=[X]).

  - Proposal 12 (OPPO): Further discuss during WI stage:

    - The detailed assumptions for system level and link level simulation

    - Whether and how to simulate different SINR values for deriving baseband error

    - Whether and how to align the procedure for adding different errors on the ideal dataset

  - Proposal 13 (Samsung): Regarding measurement error model on prediction accuracy in AI mobility:

    - For RF error, take the outcome of the error modelling approach in PHY AI/ML as a baseline

    - For BB error, LLS approach involving SNR/SINR is considered to determine the distribution

    - Different properties of channel propagation across neighbor cells would be captured as a factor

  - Proposal 14 (Huawei): Use LLS to generate the measurement error model for each SINR, where the error is calculated by comparing the L1-filtered and L3-filtered of the measured L1-RSRP versus L3-RSRP ground truth.

  - Proposal 15 (Huawei): Use Gaussian distribution to model RF error, the mean value and variance of which will be decided during WI.

- Recommended WF:

RAN4 to capture the following factors to be considered for error models to consider the impact of measurement errors on prediction accuracy:

- RF and baseband measurement error modeling may be considered.

- As a baseline:

  - For BB error, following AI/ML beam management approach, use link level simulation assumptions to generate L3-RSRP difference as baseband error.

  - For RF error model, use a uniform or a Gaussian distribution.

- Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

Discussion:

Apple: In principle, it looks fine. Some details may be missing. What method for link level simulation will be used. This can be further discussed during WI phase.

LG: For BB, we can simply use gaussian model.

OPPO: remove first bullet. LL may not follow random distribution.

MediaTek: How errors will be introduced should also be discussed. It may however be decided during WI phase as well.

Qualcomm: Let’s discuss it during the meeting.

Xiaomi: We need to consider other factors as well to consider impact of measurement error on prediction accuracy.

Nokia: either same or different RF error for different RF chains

Tentative Agreement:

RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy:

- As a baseline:

  - For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

  - For RF error model, use a uniform or a Gaussian/truncated Gaussian distribution. Either the same or different RF error may be considered for different RF chains.

- The values of mean and variance of these distributions for different use cases will be further discussed during WI phase.

### Issue 2-9: Inter-frequency prediction scenario

- Background:

The following agreement was reached in RAN4#115 meeting:

Agreement: RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

Furthermore, the following agreement was reached in RAN4#114bis meeting:

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- Proposals and Observations:

  - Proposal 1 (Qualcomm): RAN4 to consider the impact on MG sharing mechanism among different inter-frequency layers

  - Proposal 2 (Qualcomm): RAN4 to consider the impact on Carrier-Specific Scaling Factor (CSSF) in case of inter-frequency prediction

  - Proposal 3 (Samsung): RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

  - Proposal 4 (Huawei, HiSilicon): For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9.

  - Proposal 5 (Huawei, HiSilicon): For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss:

    - Specify RSRP prediction accuracy for cluster approach and single cell approach separately, or

    - Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach

  - Proposal 6 (CMCC): for cluster approach, it is proposed to follow RAN2 agreements that model takes measurements from more than one cell as inputs.  For cluster approach, not to consider the case that measurement from single cell in one carrier frequency is used as an model input.

  - Proposal 7 (Nokia): For inter-frequency prediction scenario, RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point.

  - Proposal 8 (Nokia): In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 should take the same assumption to define the requirements.

  - Proposal 9 (Nokia): To define correlation coefficient (CC) for inter-frequency predictions, RAN4 to consider the channel models in different frequency layers. In this case, one way to define the CC is the correlation between PDPs of different frequency layers.

  - Proposal 10 (Xiaomi): For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 11 (Tejas Network Limited): No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

  - Proposal 12 (Apple): RAN4 is invited to study the generalization capability of AI/ML-based inter-frequency measurement prediction models during the WI stage (Re-20). The study should evaluate how models trained on measurement data from specific inter-frequency deployment scenarios (e.g., macro–micro, FR1–FR1, FR1–FR2) perform when applied to unseen scenarios with different frequency layer combinations, cell layouts, and propagation environments. This includes assessing the impact of clustering multi-cell measurements as input features, the portability of models across varying network configurations, and the trade-off between prediction accuracy and measurement gap reduction.

  - Proposal 13 (Apple):RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction model

  - Proposal 14 (Apple): RAN4 is invited to study the impact of clustering on AI/ML-based inter-frequency measurement prediction performance. Multi-cell inputs provide richer spatial and frequency-domain information, improve robustness against anomalies in individual cell measurements, and enable better generalization across deployment scenarios. The study should assess clustering benefits under varying mobility conditions, network topologies, and correlation levels between cells, with the objective of determining whether clustering can reduce measurement overhead while maintaining or improving prediction accuracy

  - Proposal 15 (MediaTek):

    - For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not.

    - For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

- Candidate Options:

  - Correlation Coefficient:

    - Option 1: RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9.

    - Option 2: RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point.

    - Option 3: RAN4 to define the CC as the correlation between PDPs of different frequency layers by considering channel models in different frequency layers

    - Option 4: Further details on the impact of CC should be considered during the WI phase

Discussion:

MediaTek: It can be further discussed during simulation campaign and then different companies can perform simulations using different (suitable) CC values.

  - CSSF:

    - Option 1: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period

    - Option 2: Not needed

Discussion:

E//: We need to consider it anyway.

  - Impact of cluster approach:

    - Option 1: Specify RSRP prediction accuracy for cluster approach and single cell approach separately

    - Option 2: Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach

Discussion:

E//: Does it lie in the domain of spatial domain prediction?

CMCC: RAN2 focusses on cluster approach. RAN4 should also focus on cluster approach.

Qualcomm: No requirements for single approach then?

MediaTek: At this moment, it is not important to consider single cell or cluster-based approach.

Tentative Agreement:

RAN4 to consider during WI phase if same or different minimum RSRP prediction accuracy requirements are needed for cluster and single cell approach.

  - Impact on Measurement Gaps:

    - Option 1: RAN4 to consider the impact on MG sharing mechanism among different inter-frequency layers

    - Option 2: Not needed

- Recommended WF:

RAN4 to discuss the above-listed candidate options for Correlation Coefficient, CSSF, Cluster approach and measurement gaps one by one.

# References

[1] R4-2509057, “Topic summary for [116][214] FS_NR_AIML_Mob_Part1”, Moderator(Nokia), RAN4#116

[2] R4-2509058, “Topic summary for [116][215] FS_NR_AIML_Mob_Part2”, Moderator(OPPO), RAN4#116