3GPP TSG-RAN WG4 Meeting #116	R4-2512127

Bengaluru, India, August 25th – August 29th, 2025

Agenda item:			7.19.1

Source:	Moderator (Nokia)

Title:	WF on AI/ML Mobility Part 1

Document for:	Approval

# Introduction

- This WF captures the discussions and agreements carried out on AI/ML Mobility SI under [116][214] FS_NR_AIML_Mob Part 1 thread.

# Agreements.

## RAN4 requirements for RRM Measurement prediction

### Agreements in the RRM session

Issue 2-1: Impacts of measurement error on prediction accuracy

Agreement:

RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy:

- As a baseline:

  - For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

  - For RF error model, use a truncated Gaussian distribution.

    - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

Issue 2-2: Relative RSRP accuracy definition

Agreement:

Relative RSRP accuracy definition

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

- Note: It is not precluded to update the definition based on further RAN2 progress in WI phase.

## RAN4 requirements for measurement event prediction

### Agreements in the RRM session

Issue 3-1: Performance metric for indirect measurement event prediction

Agreement:

- On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase.

- Capture the candidate proposals in the WF for information purpose.

# Way Forward.

## General Aspects

### Discussions in the adhoc session

Issue 1-1: Spatial domain prediction

Due to time limitation, RAN4 has not studied potential UE impact for spatial domain prediction from network side model.

## RAN4 requirements for measurement event prediction

### Discussions in the RRM session

Issue 3-1: Performance metric for indirect measurement event prediction

WF:

For information purposes, here are proposals from different companies on the issue of performance metric for indirect measurement event prediction:

  - Proposal 1 (NTT Docomo):

    - The report doesn’t include the predicted RSRPs in case of indirect measurement event prediction use case.

    - No need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction.

  - Proposal 2 (Nokia):

    - RAN4 to consider following performance metrics for indirect measurement event prediction.

      - Event Prediction Accuracy: Captures both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence. It consists of two components:

        - Outcome Accuracy: whether the prediction correctly identifies event occurrence or non-occurrence.

        - Temporal Accuracy: Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance.

      - Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value.

    - For indirect measurement event prediction use case, for potential report that includes the predicted RSRPs, RAN4 to assume a similar RSRP accuracy definition considered for predicted measurements. The applicable tolerance or accuracy levels may be influenced by several key factors, including length of the observation window, the duration of the prediction window, etc

  - Proposal 3 (Ericsson):

    - RAN4 to reuse accuracy requirement for temporal prediction of RRM measurement for indirect event prediction.

  - Proposal 4 (OPPO):

    - For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

    - For indirect prediction, the following approaches can be considered as possible KPI to evaluate the performance:

      - Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

      - Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

      - Alt 3: The handover failure rate per UE under certain time durations.

    - Other alternatives are not precluded.

    - For indirect prediction, RAN4 to discuss how to define the performance metrics and requirements in work item based on further RAN2 progress.

  - Proposal 5 (Huawei):

    - RAN4 to discuss the following options for indirect event prediction

      - Option 1: Define accuracy for indirect event prediction

      - Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value.

      - Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

      - Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

  - Proposal 6 (CATT): Indirect measurement event prediction:

    - RAN4 do not define accuracy requirements when the final result report contains the expected occurrence time of a measurement event.

  - Proposal 7 (Qualcomm):

The performance metrics for AI/ML-based measurement event prediction include, but are not necessarily limited to, the following:

- L3-RSRP accuracy:

  - RAN4 to further clarify “In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP” agreed upon at RAN4#114 as follows:

    - For the absolute and/or relative accuracy of the predicted L3-RSRP, if the predicted event report includes predicted L3-RSRPs, the corresponding ground truth should be taken from the predicted event occurrence time, not the actual ground truth event occurrence time.

- Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted):

  - It is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event.

  - Note: The report time of the predicted event should precede the actual occurrence time of the ground truth event.

- Positive event prediction accuracy:

  - The below two metrics can be considered as event prediction requirements either separately or jointly (e.g., via F1 score).

    - Successful event detection rate: The proportion of actual positive ground truth events that were correctly predicted by the UE within a tolerable prediction window of a certain length.

    - False alarm rate: The proportion of predicted positive ground truth events that did not actually occur within a tolerable prediction window of a certain length.

  - The tolerable prediction window length is defined as the time gap between the predicted event occurrence time and the actual positive ground truth event occurrence time. The specific value of this window length is subject to further discussion in Rel-20.

- The actual ground truth event occurrence time is defined according to the ground truth definition used in L3-RSRP prediction.

- Note: All of the above performance metrics (L3-RSRP accuracy, Event prediction latency, Positive event prediction accuracy, Tolerable prediction window) may be interdependent.

  - Proposal 8 (CMCC):

    - For impact on core requirements for indirect measurement event prediction, it is proposed to reuse the conclusion on the absolute and/or relative accuracy of predicted RSRP for RRM measurement prediction.

    - For impact on core requirements for the direct measurement event prediction, it is proposed to discuss whether and how to define RRM requirements for the final output (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction).

  - Proposal 9 (ZTE):

    - For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction.

    - RAN4 should consider whether and how to define accuracy for direct event prediction and indirect event prediction based on RAN2’s KPIs, such as precision, recall, F1 score and ETD.

  - Proposal 10 (Apple):

    - RAN4 should set similar performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, but with different emphasis. Case A should focus more on prediction lead time and reliability to enable earlier, safe handover preparation. Case B should focus more on RSRP reconstruction accuracy and correctness of the derived event decision to ensure measurement-overhead reduction without mobility degradation

    - Case A and Case B shall use windowed precision over a lead-time window [L_min, L_max]. A prediction is counted true only if it arrives at least L_min and at most L_max before the ground-truth A3 trigger T0, ensuring the gNB has enough,but not excessive,time to prepare the handover. Replace the F1 score with two limits: (i)windowed precision ≥ P_min and (ii) every counted hit satisfies the lead-time constraint (L_min, L_max). Precision curbs false A3 alarms; the lead-time bound guarantees accepted hits are early enough to be actionable. This pair avoids periodic/event-based recall ambiguities and roughly halves test workload by eliminating the need to log every missed event.

    - For Case B, apply the same windowed-precision rule over [L_min, L_max] and the same lead-time constraint, but with a relaxed performance target to reflect the reduced (skip-aware) prediction interval pattern