3GPP TSG-RAN WG4 Meeting #116										     R4-2512131

Bengaluru, IN, August 25 – 29, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part2

Agenda Item:	7.20.1

Source: 	MediaTek inc.

Document for:	Approval

# <Topic 1: UE reporting enhancement for CJT calibration>

The simulation results for delay and frequency offset reporting for CJT calibration are collected from companies’ contributions and are captured in R4-2512159.

Issue 1-1: How to define measurement period and accuracy requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

Define core requirements based on 1 sample only and further discuss the accuracy and SNR in the performance part.

Companies are encouraged to discuss how to derive the accuracy performance based on the simulation results.

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- Reuse the legacy L1-RSPR scheduling restriction in FR1 as baseline for delay and frequency offset measurement.

  - There are no scheduling restrictions due to delay and frequency offset measurement performed on CSI-RS (tracking RS) with the same SCS as PDSCH/PDCCH in FR1.

  - For UEs which support simultaneousRxDataSSB-DiffNumerology, there are no restrictions on scheduling availability due to delay and frequency offset measurement based on CSI-RS (tracking RS).

  - For UEs which do not support simultaneousRxDataSSB-DiffNumerology, the following restrictions apply due to delay and frequency offset measurement based on CSI-RS (tracking RS).

    - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSIRS for CQI on symbols corresponding to the CSI-RS (tracking RS) indexes configured for delay and frequency offset measurement.

Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- Reuse the legacy CSI-RS based L1 measurement restriction as baseline in FR1 for delay and frequency offset measurement.

  - Measurement restriction does apply,

    - If UE does not support simultaneousRxDataSSB-DiffNumerology and

    - SSB for RLM, BFD, CBD or L1-RSRP with different SCS as CSI-RS (TRS) for delay and frequency offset measurement and they are fully or partially overlapped on symbols in FR1.

  - Measurement restriction does not apply,

    - If UE support simultaneousRxDataSSB-DiffNumerology or

    - SSB for RLM, BFD, CBD or L1-RSRP with the same SCS as CSI-RS (TRS)for delay and frequency offset measurement in FR1.

# <Topic 2: Enhancement for asymmetric DL sTRP/UL mTRP scenarios>

Moderator’s note on the scenarios’ definition discussed in this section:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

## Sub-topic 2-1 Scenario#1

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- For DL reference point, agree on the concept of proposal P1c. For scenario 1, it is proposed that, for both TAGs, the reference point for PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DL or Joint TCI state.

Agreement:

- The same offset for both of TRPs.

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

For scenario 1, same gradual timing adjustment applies for each TAG and shall be met for each TAG.

Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements?

Agreement:

- For scenario#1, no MRTD requirements.

- For MTTD requirements:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

## Sub-topic 2-2 Scenario#2

Issue 2-4: For Scenario#2, how to update PRACH and PUCCH/PUSCH/SRS transmission requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Background: In the last meeting, RAN4 agreed on how to define PRACH and PUCCH/PUSCH/SRS transmission requirements for 2TA as below, and this issue is to address the tentative wording between brackets:

RAN4#115: Agreement: For Scenario#2, inter-cell operation, PRACH reference point: - reference point for PRACH transmission is - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL [or UL] TCI state(s) of the reference cell associated with the second TAG For inter-cell multi-TRP, for PUCCH/PUSCH/SRS: - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

Agreement:

Confirm the previous agreement and remove the remaining brackets [].