3GPP TSG-RAN WG4 Meeting # 116					       			       			       R4-2512137

Bengaluru, India, August 25-29, 2025

Agenda item:			7.23.1

Source:	Huawei

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part2

Document for:	Approval

Topic #1: Adaptation of common signal/channel transmission

Sub-topic 1-1 SSB adaptation – General

Issue 1-1-1: Applicable scenario for SSB adaption.

Background:

TS 38.321 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

Agreement (on Thursday):

RAN4 not to define requirement for DCI based SSB adaption for deactivated SCell measurement and SCell activation.

- Note: this RAN4 agreement does not intend to impact the discussion and decision in other WGs.

Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-2: L1 requirements – Measurement restriction impact

Agreement:

- When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: Colliding with MG

Agreement:

- If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Issue 1-3-2: L3 requirements impacts

Proposals (for information) - Option 1: After UE receiving SSB adaptation indication, UE still follows legacy deactivated SCell measurement framework based on measCycleScell

As per the agreement of issue 1-1-1, the issue is closed.

Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

Proposals for information - Option 1: R18 Scell activation with L3 reporting - No requirements - Define requirements - Option 2: multiple SCells activation scenario with SSB adaptation - No requirements - Option 3: Direct SCell activation - No requirements

As per the agreement of issue 1-1-1, the issue is closed.

Issue 1-4-3: SCell activation requirement for SSB adaptation – Others

Proposals (for information) - Option 1: RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling, the latter in combination with normal SCell activation or direct SCell activation or PUCCH SCell activation or any further agreed SCell activation scenarios. - Option 2: RAN4 to define SCell activation requirements in case of concurrent SCell activation and SSB adaptation command in same RRC message, e.g. for known SCell in FR1, based on - - Tactivation_time = TFirstSSB + X, with X=6ms for measurement period ≤ 2400ms, - - Tactivation_time = TFirstSSB_MAX + Trs + X, with X=6ms for measurement period > 2400ms. - Option 3: According to the DCI 2_9 based mechanism for adaptation of SSB in time domain for SCell, the SCell timing information is known to the UE, blind detection is not needed. - Option 4: If to define SCell activation requirement for SSB adaptation when SSB adaptation command comes at the same time as SCell activation command, d and additional processing time need to be contained in TfirstSSB - Option 5: When to define SCell activation requirement for SSB adaptation when SSB adaptation command comes earlier than SCell activation command, it would be better to ensure the first possible SSB burst after SSB adaptation was acquired before SCell activation command reception - FFS: Condition related to slot m+d+∆ and slot n

As per the agreement of issue 1-1-1, the issue is closed.

Topic #2: OD-SIB1

Sub-topic 2-1 RRM requirements impacts for OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1

RAN4 agreements in R4-2508441 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

Agreement on Ad hoc:

- Keep current agreement and if companies think additional requirements are needed discuss in maintenance stage.