# 3GPP TSG-RAN WG4 Meeting #116	R4-2512241

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.3.4

Source:	Apple

Title:	TP to TR38.768 on RRM requirements for low band aggregation via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

This contribution provides a text proposal to TR38.768[1] with two changes: (1)create sub-clause structure based on  RRM requirement discussion aspects, and (2)add agreements based on agreed WF in [2][3][4].

# 3	References

[1]	RP‑251762, TR 38.768 v1.0.0 on Low NR band carrier aggregation via switching, Apple, RAN#108

[2]	R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #114

[3]	R4-2504895,	 WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #114bis

[4]	R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #115

# 2	Text Proposal

<< begin change 1 >>

# 6	RRM aspects

<Editor’s note: capture RRM aspects in this clause>

## 6.1	General

RAN4 has reached the following agreements on the switching pattern applicability for RRM requirement in R19 LB CA via switching in RAN4#114bis meeting:

For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133),

-	The switching pattern for SDL SCell is applied.

For the SCell which is in activation procedure,

-	The switching pattern for SDL SCell is applied.

-	RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation, where the RS occasions are FFS

For the SCell which is deactivated,

-	The switching pattern for SDL SCell is not applied.

-	If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirement for deactivated SDL SCell measurement.

Note: the above FFS parts will be discussed and decided in RAN4.

The necessity of requirement for SDL SCC was agreed in RAN4 #115 meeting, as summarized in Table 6.1-1.

Table 6.1-1. Necessity of requirement on SDL CC

| Requirement | Whether or not RAN4 will have requirement |
| --- | --- |
| interruption requirement for CC switching | No (can be revisited if not aligned with RAN1 further agreement.) |
| Activated SDL SCell L3 measurement | Yes |
| Deactivated SDL SCell measurement [and corresponding interruption] | Yes |
| SDL SCell activation/deactivation and corresponding interruption | Yes |
| [BFD/CBD] | Yes |
| TCI state switch delay | Yes |
| PL-RS switch delay | No |
| L1-RSRP/SINR | Yes |

Following SSB assumptions for R19 LB CA via switching are agreed in RAN4 #115:

The baseline assumption is:

-	SDL SCell is transmitting SSB

-	RAN4 to use SSB based requirement as baseline for Rel-19 low band CA via switching

-	requirement of PCell will still use the existing principle as baseline but the impact of carrier switching in LB CA will be considered

Following MRTD assumptions for R19 LB CA via switching is agreed in RAN4 #115:

-	The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

-	Whether or not above RTD can be used as side condition for SCell activation will be further discussed.

  - RAN4#116 meeting agreed that according to the WID, RAN4 only defines requirements for CA with FDD PCC and one SDL SCC.

## 6.2	Discussion of switching delay or interruption requirement

It was agreed in RAN4 #115 meeting that, for SDL to FDD switching and FDD to SDL switching, RAN4 will not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

## 6.3	Discussion of SDL SCell related requirement

### 6.3.1	RRM requirement applicability for R19 LB CA via switching

RAN4#116 agreed the following RRM requirement applicability:

  - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC:

  - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

  - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

## c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

### 6.3.2	Activated SDL SCell L3 measurement requirement

RAN4 #115 agreed to define the requirement for activated SDL SCell L3 measurement. For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern.

RAN4 #116 agreed followings:

- if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied.

- reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA.

- Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA.

- if the activated SCC measurement is MG-less in LB-CA, RAN4 to update Kp factor for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA.

- Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following:

  - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

    - For a window W of duration max(SMTC period,  xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion:

      - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and

      - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W.

      - No requirement when Navailable = 0.

### 6.3.3	Deactivated SDL SCell L3 measurement requirement

RAN4 #115 agreed to define the requirement for deactivated SDL SCell L3 measurement. The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement.

RAN4#116 agreed to define the interruption requirement as follows:

- For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level),

  - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19

  - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19

- Missed ACK/NACK rate

  - For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length.

### 6.3.4	SDL SCell activation/deactivation requirement

RAN4 #115 agreed that PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. The side condition for SDL SCell activation/deactivation requirement follows the legacy case and side conditions for SSB based inter-band FR1 SCell activation in existing spec.

The following principle design for SDL SCell activation/deactivation delay requirement was agreed in RAN4#115:

-	The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature.

-	R16 Direct SCell activation with HO is not considered for R19 LB CA.

-	R17 PUCCH SCell activation is not considered for R19 LB CA.

-	R18 SSB-less SCell activation for NES is not considered for R19 LB CA.

-	R17 Fast SCell activation is not considered for R19 LB CA.

-	Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

-	Known/unknown conditions are reused from existing SCell activation requirements

RAN4 #116 agreed follows:

- Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure:

  - For the SCell which is in activation procedure,

    - The RRC configured switching pattern for SDL SCell is applied.

    - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length

    - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length

      - Option 1: TLBCA =0ms

      - Option 2: TLBCA =1ms

      - Option 3: TLBCA =4ms

  - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options.

    - For MAC CE based SCell activation, according to the switching pattern,

      - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length.

      - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length.

      - Other options are not precluded.

    - For RRC based direct SCell activation, according to the switching pattern,

      - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length.

      - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length.

      - Other options are not precluded.

Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

  - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length).

    - Option 1: TLBCA =0ms

    - Option 2: TLBCA =1ms

    - Option 3: TLBCA =4ms

  - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

### 6.3.5	SDL SCell L1 requirement

If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

RAN4 will not define requirement for PL-RS switch delay for SDL SCell in LB CA via switching

For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

RAN4 #116 agreed following requirement for BFD/CBD requirements for SDL SCell:

- RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching

  - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where,

    - W window is max(TL1, xRP_max, switching pattern periodicity)

    - P = Ntotal/ Navailable

    - Ntotal is total number of BFD-RS or CBD-RS in W window.

    - Navailable is number of available BFD-RS or CBD-RS in W window.

RAN4 #116 agreed following requirement for TCI state switch delay requirement for SDL SCell:

- RAN4 to define the TCI state switch delay requirement for LB-CA via switching

- In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE.

RAN4 #116 agreed following requirement for L1-RSRP/SINR measurement requirements for SDL SCell:

- P factor in the existing L1-RSRP/SINR measurement requirements can be updated for LB-CA via switching, where,

  - W window is max(TL1, xRP_max, switching pattern periodicity)

  - P = Ntotal / Navailable

  - Ntotal is total number of L1-RSRP/SINR RS of SCell in W window.

### Navailable is number of available L1-RSRP/SINR RS overlapping with SCell duration according to the RRC configured switching pattern in W window.

### 6.3.6	SDL SCell performance requirement

RAN4#116 meeting agreed follows for SDL SCell performance requirement:

## Existing measurement accuracy requirements apply for LBCA operation through switching.

## 6.4	Discussion of impact on FDD PCell RRM

RAN4#115 meeting agreed that the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching.

RAN4#116 meeting agreed follows for RLM requirement:

- P factor in the existing RLM measurement requirements can be updated, where,

  - W window is max(TL1, xRP_max, switching pattern periodicity)

  - P = Ntotal/ Navailable

  - Ntotal is total number of RLM-RS in W window.

  - Navailable is number of available RLM-RS in W window.

- FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

RAN4#116 meeting agreed follows for others RRM requirement of FDD PCell in LB CA via switching:

- The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA.

<< end change 1 >>