3GPP TSG-RAN4 Meeting #116	R4-2512315

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| x |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Draft CR38.133 Introduction of measurement gap cancelation |  |  |  |  |  |  |  |  |  |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Measurement gap cancelation is part of the Rel-19 work item NR XR Phase 3. Delay requirements for cancelation of a particular measurement gap occasion are missing in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of requirements for measurement gap occasion cancelation used for XR. Clause 9.1.y - Specifies which gap types are subject to measurement gap skipping - Specifies applicability of the measurement gap occasion cancelation enhancement - Specifies requirements for the measurement gap occasion cancelation delay. This CR is drafted using RAN2 pseudo code approach to serve as one example for specification improvement as agreed in RAN4 #113 in TDoc R4-2420107. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing delay requirements and applicability for measurement gap occasion cancelation. The enhancement cannot be used for XR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2509959 |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### 9.1.13.3	Collision involving NCSGs

Collisions between occasions of concurrent measurement gap and NCSG or of two NCSGs may occur as specified in this clause if the two occasions are

-	two per-UE NCSGs, or

-	two per-FR NCSGs in the same FR, or

-	one per-UE NCSG and one per-UE measurement gap, or

-	one per-FR NCSG and one per-UE measurement gap, or

-	one per-UE NCSG and one per-FR measurement gap, or

-	one per-FR NCSG and one per-FR measurement gap in the same FR.

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collision rule applies.

When the first occasion is NCSG, the ending point of the first occasion is the end of VIL2 and/or when the second occasion is NCSG, the starting point of the second occasion is the start of VIL1.The requirements with concurrentMeasGapsNCSG-r18 apply provided that two gaps (at least one of the gaps is NCSG) colliding with each other are configured with different priorities.

### 9.1.y	Measurement gap occasion cancelation

#### 9.1.y.1	Introduction

This clause specifies the general requirements and applicability for UE supporting indication for cancelation of measurement gap occasions as specified in TS 38.213 [3].

#### 9.1.y.2	Applicable measurement gap configurations

Measurement gaps configured by one of the below configurations are applicable for measurement gap occasion cancelation:

-	GapConfig without suffix;

-	GapConfig-r17 with gapPriority-r17 and without ncsgInd-r17 or preConfigInd-r17 or gapAssociationPRS;

#### 9.1.y.3	Applicability

The measurement gap cancellation requirements apply under the following conditions.

1>	if an explicit indication is received via DCI according to TS 38.213 [3] to cancel a measurement gap occasion; and

1>	if the UE is configured with NR SA (with or without CA) in FR1 or FR2-1; and

1>	if the UE is not PC6 in FR2-1; and

1>	if the UE is not configured to do measurements for SRVCC in clause 9.4.6; and

1>	if the UE is not configured with any MUSIM measurement gap pattern via MUSIM-GapConfig [2]; and

1>	if the UE is not configured with positioning measurements requiring measurement gaps; and

1>	if the UE is configured with one of the measurement gap configurations listed in clause 9.1.y.2:

2>  if the UE is configured with per-UE measurement gap pattern(s), the measurement gap occasion cancelation applies to both FR1 and FR2-1, and the requirements with measurement gap occasion cancelation in clause 9.1.y.4 apply;

2> Otherwise, if the UE is configured with per-FR measurement gap pattern(s), the measurement gap occasion cancellation only applies to the per-FR measurement gap on the same frequency range as the one or more scheduled cells associated with the DCI format and has no impacts on UE measurement behaviour in the other FR, and the requirements with measurement gap occasion cancelation in clause 9.1.y.4 apply;

#### 9.1.y.4	Requirements for cancelling measurement gap occasions

For the measurement gap patterns configured via gapPriority-r17 and without any of ncsgInd-r17 or preConfigInd-r17 or gapAssociationPRS, the measurement gap cancelation applies to measurement gap occasions after applying collision rules as defined in clause 9.1.8.3.

UE behaviour during cancelled measurement gap occasions is specified in TS 38.213 [3].

The UE shall cancel the gap provided that the time offset between the DCI indication of cancelation of measurement gap and the start of the measurement gap occasion including MGTA is smaller than X in Table 9.1.y.4-1.

Table 9.1.y.4-1 – Time offset X between the indication of cancelation of measurement gap and the start of measurement gap occasions including MGTA

| Component 4 of FG 64-1 | Time offset X |
| --- | --- |
| 5 | 5 ms |
| 3 | 3 ms |

--- End of change 1 ---