# 3GPP TSG-RAN WG4 Meeting #116	R4-2512520

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.12.3

Source:	Apple, Qualcomm

Title:	Draft TP to TR on SCM study - 5.1 CDL approach

Release:	Rel-19

Document for:	Endorsement

# 1. Introduction

This draft TP captures the CDL approach for spatial channel modelling methodologies.

# 2. Draft TP

# 5	Spatial Channel Modelling Approaches

## 5.1	CDL Approach

The Clustered Delay Line (CDL) channel model is a widely used framework for simulating realistic wireless propagation environments. CDL models capture the spatial, temporal, and frequency-dependent characteristics of radio channels by representing multipath propagation through discrete clusters of scatterers, each contributing multiple delay paths. The details of the CDL based approach employed in the study are provided in this section with the relevant parameter configurations. The model employed in the study is applicable for 3.5 GHz frequency in urban macro scenario.

Figure 5.1.1-1: CDL Modeling Execution

### 5.1.1	Antenna Panel Configuration

The antenna panel is one of the primary components for CDL modeling methodology. The antenna panel configuration employed in this study are captured in simulation assumptions Table 6.1-2.

### 5.1.2	Antenna Array Virtualization

The antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one antenna port through a virtualizer (set of complex values).

The antenna panel (M,N,P,Ms,Ns) and AAV (Ms,Ns) configurations used in the study item are listed below, according to the terminology defined in Annex B.2 (Step 2).

Two different AAV configurations for base station have been considered in the study.

BS AAV Configurations:

- BS AAV(8,1) configuration:

    - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

    - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- BS AAV(1,1) configuration (single element subarray):

    - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

    - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

Only one UE AAV(1,1) configuration (single element subarray) has been considered in the study:

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4 RX

Figure 5.1.2-1: Example AAV(1,1) and AAV(8,1) configurations

### 5.1.3	Channel Models

The CDL-C model from 38.901 in Table 7.7.1-3 is taken as a starting point. The parameters for desired angle spread, RMS delay spread are derived based on urban macro (UMa) scenario for 3.5 GHz carrier. The randomness introduced by the channel generation based on 38.901 is alleviated by randomness reduction framework adopted from 38.827 [12].

#### 5.1.3.1	Parameters for Angle Scaling

The angles need to be scaled for the scenario to achieve the desired angular spread and direction. The parameters for scaling the angles of arrival and departure are provided here. The CDL tables provided in 5.1.3.3 include the delay scaling for RMS delay spread of 365ns and desired angle scaling. The scaled cluster (φn,scaled) and scaled ray (φn,m,scaled) angles and cluster specific angular spread are obtained as described below for AOD, AOD, ZOA, and ZOD.

φn,scaled=wrap(ASdesiredASmodel.wrap(φn,model-μφ,model)+μφ,model)                         (5.1.3.1-1)

c{ASA, ASD,ZSA,ZSD}_scaled= ASdesiredASmodel. c{ASA, ASD,ZSA,ZSD}_model                                  (5.1.3.1-2)

φn,m,scaled= φn,scaled+c{ASA, ASD,ZSA,ZSD}_scaledαm                               (5.1.3.1-3)

φn,model	is the tabulated CDL cluster angle from 38.901

ASmodel	is the rms angular spread of the tabulated CDL from 38.901 including the offset ray angles provided in Table 5.1.3.1-1

μφ,model	is the power weighted mean angle of the tabulated CDL from 38.901, calculated using the definition in Annex A.2 of 38.901[7]

cASA, ASD,ZSA,ZSD_model is the tabulated cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD from Table 7.7.1-3 of 38.901 [12].

ASdesired	is the desired rms angular spread provided in Table 5.1.3.1-1

φn,scaled	is the resulting scaled cluster angle.

φn,m,scaled	is the resulting scaled ray angle.

cASA, ASD,ZSA,ZSD_scaled is the scaled cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD in Table 5.1.3.3-1.

αm                  is the offset angle per ray from Table 7.5-3 of 38.901[7]

y = wrap(x) makes y to fall between -180 and +180 degrees so that y = (x+180) mod 360 - 180

μφ=arg{n=1Nm=1Mexp(jφn,m)Pn,m}

Pn,mφn,m

Note: The angle scaling procedure included in this section was used during the work in Rel-19 study item, and it is based on the companies’ agreed interpretation of the procedure, not captured in the version of TR 38.901 (v18) available during the derivation of the rCDL model. The angular scaling procedure for CDL models was since revised in TR 38.901 (v19), and the procedure in 38.901 shall be applicable to rCDL in future releases. The procedure here is captured as a reference for the procedure and rCDL table reproducibility only.

Table 5.1.3.1-1: Angle scaling parameters

| AS |  | ASmodel / ASdesired [deg] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ASD |  |  | ASA |  |  | ZSD |  | ZSA |  |
| Model |  | 37.4036 |  | 71.4535 | 4.0665 | 10.4182 |
| Desired |  | 25.7620 |  | 74.1138 | 4.8978 | 18.2050 |

#### 5.1.3.2	Parameters for Randomness Reduction

The randomness reduction framework employs fixed coupling pattern for ray angles and fixed initial phases for polarization matrices in the channel coefficient generation from 38.827 [12].

Within a cluster the AOD angles are coupled to AOD angles using the fixed coupling pattern as specified in Table 7.2-6 in 38.827 [2] and captured in Table 5.1.3.2-1. The same fixed coupling pattern is applied for all clusters n.

Table 5.1.3.2-1: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

The initial phase {Φn,mθθ,Φn,mθφ,Φn,mφθ,Φn,mφφ} for each ray m of each cluster n and for four different polarization combinations (θθ, θϕ, ϕθ, ϕϕ) are fixed as specified in Table 7.2-8 of 38.827 [12] and captured in Table 5.1.3.2-2.

Table 5.1.3.2-2: Fixed initial phases for 2x2 polarization matrices

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

#### 5.1.3.2	CDL Cluster Reduction

The CDL model is reduced to 12 clusters based on the procedure provided in Annex B.3.1. The channel model parameters in 5.1.3.3-1 is derived based on the angle scaling procedure and parameters in 5.1.3.1.1

#### 5.1.3.3	CDL Channel Models

5.1.3.3.1 rCDL-C1

The rCDL-C1 channel model parameters are derived based on 38.901 CDL-C, Uma scenario, 3.5 GHz frequency with the desired delay and angle spread applied and truncated to 12 significant clusters.

Table 5.1.3.3.1-1: Channel model parameters for rCDL-C1

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

### 5.1.4	Channel Power Normalization

To maintain the SNR definition used in RAN4 performance requirements, the channel power must be normalized. However, when directional antennas or antenna array virtualization are used, the channel could introduce a power gain. To account for this gain, a single normalization factor must be applied across all Rx chains. This normalization factor is calculated based on the long-term average PDP for the CDL channel, as well as the spatial response of the antenna and the gain achieved with antenna virtualization, i.e., the AAV gain including AE pattern is taken away as part of the channel normalization.