# 3GPP TSG-RAN WG4 Meeting #116	R4-2512542

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Way Forward on VSAT type for LEO only

Agenda Item:	7.7.1

Source: 	Huawei, Eutelsat Group, Thales, Airbus, Sharp, Iridium, SES, Viasat, Ericsson, Xiaomi, Aalyra, Vivo, Gilat, LGE, Mediatek, Sky Perfect JSAT, Fraunhofer HHI, Fraunhofer IIS, ST Engineering iDirect, Kratos Defense, FirstNet, ETRI, Bosch, BMW, Echostar

Document for:	Approval

# Introduction

NTN Mobile VSAT has been specified in Rel-18 version of the TS 38.101-5 in the following way:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |

One of the objectives of the WI on Introduction of Ku bands for NR NTN in RP-250799 was to specify RF requirements for mobile VSAT served by NGSO and GSO, based on existing mobile VSAT type 5 RF requirements.

The need for VSAT type 5 to communicate with both the NGSO (i.e., LEO) and GSO orbits (as per Ku WID objective) has triggered extensive discussion in RAN4, leading to identification of challenges related to proper requirements definition, as well as their specification. At the same time, multiple companies have expressed strong interest in pursuing specification of a Mobile VSAT communicating with NGSO (i.e., LEO) only.

It was also identified, that specification of a Mobile VSAT type for LEO only would be relatively easy, with related UE RF requirements already captured in a draft CRs to Rel-19 version of TS 38.101-5.

## Way forward

- RAN4 agreed to introduce a new Mobile VSAT type for Ku bands as follows:

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 6 | Mobile VSAT communicating with LEO only with electronic steering antenna. |

- RAN4 recommends to reflect the above agreement in the Ku WID revision during RAN#108 (September 2025).