3GPP TSG-RAN WG4 Meeting #116	R4-2512556

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.307 | CR |  | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.307: release independence aspects for the LTE-based 5G broadcast operation over geosynchronous satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Core |  |  |  |  |  |  |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In this CR to TS 36.307 we provide inputs on release independence aspects for the LTE-based 5G broadcast operation over geosynchronous satellite, as per agreement in the WF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of band 246 for the LTE-based 5G broadcast operation over geosynchronous satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of the LTE-based 5G broadcast operation over geosynchronous satellite would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 3A.1	Additional E-UTRA operating bands

Requirements for additional E-UTRA operating bands of TS 36.101 Rel-P [2] are introduced via this clause.

Table 3A.1-1: E-UTRA operating bands and UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 36.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands, band number <= 64, Power Class 3 | FDD, TDD | Rel-8 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number > 64, Power Class 3 | FDD, TDD | Rel-9 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, NS-value > 32 | FDD, TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Asymmetric operating bands, Power Class 3 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number <= 64, Power Class 1 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, Power Class 2 | FDD, TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, standalone downlink only | SDO | Rel-17 | Table B.2.1-1 (Clauses 4, 7, 8, and 9 only), Table B.4.1-1 (Clauses 5 and 7 only) |

For example, Band 19 was introduced in the Release 9 specifications. In order to implement a UE conforming to Release 8 but supporting Band 19, it is necessary for the UE to additionally conform to some parts of the Release 9 specifications (see corresponding Annexes of TS 36.307 Rel-9 which will point to the requirements in the Rel-9 of TS 36.101 [2] or TS 36.133 [3] to be fulfilled), such as the radio frequency and radio resource management requirements for the Band 19.

Requirements for additional E-UTRA operating bands of TS 36.102 Rel-P [6] are introduced via this clause.

Table 3A.1-2: E-UTRA operating bands

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 36.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands for LTE-based 5G broadcast operation over geosynchronous satellite | SDO | Rel-17 | Table B.2.1-1 (Clauses 4, 7, 8, and 9 only), Table F.1-3 |

------------------------------ Next modified section -------------------------

Annex F (normative):
Common requirements for UE operation over NTN

# F.1	Common UE RF requirements

The requirements and test cases listed in Table F.1-1 are specified in TS 36.102 Rel-P [6].

Table F.1-1: RF requirements for NB-IoT operation over NTN

| Clause | Description |
| --- | --- |
| 5.2B | Operating bands |
| 5.3B | Channel bandwidth |
| 5.4B (Note 1,2) | Channel arrangement |
| 6.1 | General transmitter characteristics |
| 6.2B | Transmit power |
| 6.3B | Output power dynamics |
| 6.4B | Transmit signal quality |
| 6.5B | Output RF spectrum emissions |
| 6.6B | Transmit intermodulation |
| 7.1 (Note 1) | General receiver characteristics |
| 7.2 | Diversity characteristics |
| 7.3B (Note 1) | Reference sensitivity power level |
| 7.4B | Maximum input level |
| 7.5B | Adjacent Channel Selectivity (ACS) |
| 7.6B | Blocking characteristics |
| 7.7B | Spurious response |
| 7.8B | Intermodulation characteristics |
| 7.9 | RX spurious emissions |
| NOTE 1: Rel-17 UEs are only subject to requirements with default Tx-Rx spacing NOTE 2: Rel-18 UEs supporting only standalone operation do not need to meet requirements specified for in-band operation with NR over NTN in clauses 5.4B.2.3 and 5.4B.3. In-band operation with NR is not supported in Rel-17. |  |

The requirements and test cases listed in Table F.1-2 are specified in TS 36.102 Rel-P [6].

Table F.1-2: RF requirements for eMTC operation over NTN

| Clause | Description |
| --- | --- |
| 5.2A | Operating bands |
| 5.3A | Channel bandwidth |
| 5.4A (Note) | Channel arrangement |
| 6.1 | General transmitter characteristics |
| 6.2A | Transmit power |
| 6.3A | Output power dynamics |
| 6.4A | Transmit signal quality |
| 6.5A | Output RF spectrum emissions |
| 6.6A | Transmit intermodulation |
| 7.1 (Note) | General receiver characteristics |
| 7.2 | Diversity characteristics |
| 7.3A (Note) | Reference sensitivity power level |
| 7.4A | Maximum input level |
| 7.5A | Adjacent Channel Selectivity (ACS) |
| 7.6A | Blocking characteristics |
| 7.7A | Spurious response |
| 7.8A | Intermodulation characteristics |
| 7.9 | RX spurious emissions |
| NOTE: Rel-17 UEs are only subject to requirements with default Tx-Rx spacing |  |

The requirements and test cases listed in Table F.1-3 are specified in TS 36.102 Rel-P [6].

Table F.1-3: RF requirements for LTE-based 5G Broadcast over Geosynchronous Satellite (BOG)

| Clause | Description |
| --- | --- |
| 5.2B | Operating bands |
| 5.3B | Channel bandwidth |
| 5.4B (Note) | Channel arrangement |
| 7.1 (Note) | General receiver characteristics |
| 7.2 | Diversity characteristics |
| 7.3B (Note) | Reference sensitivity power level |
| 7.4B | Maximum input level |
| 7.5B | Adjacent Channel Selectivity |
| 7.6B | Blocking characteristics |
| 7.7B | Spurious response |
| 7.8B | Intermodulation characteristics |
| 7.9 | RX spurious emissions |
| NOTE: Rel-17 UEs are only subject to requirements with default Tx-Rx spacing. |  |

# F.2	Common RRM requirements

The requirements and test cases listed in Table F.2-1 are specified in 36.133 Rel-P [3]. Note the requirements apply to serving cell measurements and GEO intra-frequency measurements when no satellite assistance information is provided to the UE.

T Table F.2-1: RRM requirements for NB-IoT operation over NTN

| Clause | Description |
| --- | --- |
| 4.6A1,2 | Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access |
| 6.5A2 | RRC Re-establishment for NB-IoT UEs for Satellite Access |
| 6.6A | Random Access for UE category NB1 for Satellite Access |
| 6.9A2 | RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access |
| 7.20A | UE transmit timing for NB-IoT for Satellite Access |
| 7.21A | UE timer accuracy for NB-IoT for Satellite Access |
| 7.22A | Timing Advance for NB-IoT for Satellite Access |
| 7.23A | Radio Link Monitoring for Category NB-IoT UE for Satellite Access |
| 8.14A1 | Measurements for UE category NB-IoT for Satellite Access |
| NOTE 1: If no satellite assistance information is provided for neighbor cells, the requirements in this clause for the serving cell measurement are also applicable. NOTE 2: If no satellite assistance information is provided for neighbor cells, the intra-frequency requirements in this clause are also applicable for GEO operations. |  |

The requirements and test cases listed in Table F.2-2 are specified in 36.133 Rel-P [3]. Note the requirements apply to serving cell measurements and GEO intra-frequency measurements when no satellite assistance information is provided to the UE.

Table F.2-2: RRM requirements for eMTC operation over NTN

| Clause | Description |
| --- | --- |
| 4.7A1,2 | Cell Selection and Re-selection Requirements for UE category M1 |
| 5.5A | E-UTRAN Handover for Cat-M1 UEs for Satellite Access |
| 6.2.3A | Random Access Requirements for Cat-M1 UEs for Satellite Access |
| 6.7A2 | RRC Re-establishment for Cat-M1 UEs for Satellite Access |
| 6.8A2 | RRC Connection Release with Redirection for UE Category M1 for Satellite Access |
| 7.19A | Radio Link Monitoring for UE Category M1 for Satellite Access |
| 7.24A | UE transmit timing for Category M1 for Satellite Access |
| 7.27A | UE timer accuracy for category M1 for Satellite Access |
| 7.28A | Timing Advance for Category M1 for Satellite Access |
| 8.13A1,2 | Measurements for UE Category M1 for Satellite Access |
| NOTE 1: If no satellite assistance information is provided for neighbor cells, the requirements in this clause for the serving cell measurement are also applicable. NOTE 2: If no satellite assistance information is provided for neighbor cells, the intra-frequency requirements in this clause are also applicable for GEO operations. |  |

# F.3	Common UE Demodulation requirements

The requirements and test cases listed in Table F.3-1 are specified in TS 36.102 Rel-P [6].

Table F.3-1: Demodulation requirements for NB-IoT operation over NTN

| Clause | Description |
| --- | --- |
| 8.3 | Demodulation performance requirements for UE category NB1 and NB2 |

The requirements and test cases listed in Table F.3-2 are specified in TS 36.102 Rel-P [6].

Table F.3-2: Demodulation requirements for eMTC operation over NTN

| Clause | Description |
| --- | --- |
| 8.2 | Demodulation performance requirements for UE category M1 |

------------------------------ End of modified section -------------------------