3GPP TSG-RAN4 Meeting #116	R4-2512575

# Bengaluru, India,  25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | draftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TR 37.941 on CLTA alternatives – wideband horn antenna |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of wideband horn antenna studies as an alternative to CLTA in the technical report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Wideband horn antenna has been studied in Rel-19 and this CR will capture the summary of findings during the WI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Findings from the Rel-19 work item are not included in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction of new clause 6.4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

### 6.4.4.4	CLTA alternatives - Wideband horn antenna

The use of wideband horn antenna (as an alternative to the OTA co-location testing with CLTA) has a number of potential advantages:

- Wideband horn antennas are easily available and cover wide frequency range.

- Testing with wideband horn antennas is expected to require smaller number of test antennas to cover the whole tested frequency range (e.g., in case of spurious emissions). This may be most beneficial in case where no off-the-shelf single column antennas are available for certain bands, requiring custom-made test antennas.

- Testing may be less time-consuming. For a standard horn antenna the antenna characteristics is provided in customer documentation (e.g. gain, radiation pattern, etc.), which may differ in case of custom-made CLTA products.

There are some basic differences with wideband horn antennas and CLTAs:

- Wideband horn antennas are intended to be used directed towards the DUT.

- Wideband horn antennas are different size than CLTA (considering that co-location testing with CLTA has certain DUT dimension-related restrictions).

- Wideband horn antennas are receiving different frequencies in different depths of the horn.

- Wideband horn antenna structure is semi-open from two sides and closed from two sides or closed from all four sides.

- Measurements were made with the horn antenna replacing the CLTA using the current CLTA arrangement. The horn antenna was placed next to the DUT keeping the 10cm distance and its boresight was facing the same direction as the DUT. Example measurement setup is shown in figure 6.4.4.4-1.

Figure 6.4.4.4-1. Co-located horn antenna example measurement setup.

Based on measurements with CLTA arrangements and different configurations, it was observed that:

- The size of the wideband horn antenna has a significant effect on the coupling loss results.

- There is no immediate correlation between the coupling loss in the near-field, increasing distance from 10cm (i.e., OTA co-location test setup specific arrangement) to 20cm or even to 50cm doesn’t guarantee higher coupling loss.

- CLTA test setup arrangement (as detailed in e.g., TS 38.141-2) as such is not optimal for wideband horn antennas due to the structure of the antenna and would need to be further studied.

- There is no straightforward one-to-one translation to use wideband horn antenna as CLTA with the current CLTA arrangement. This could be considered as a potential study area in future.

Single column passive antennas may not be readily available (for the purpose of CLTA test antenna) for all of the required frequency bands. They typically meet the CLTA requirements only at relatively narrow frequency ranges (e.g. 410MHz – 467.5MHz). Currently multiple custom-made antennas may be needed to cover OTA co-location testing in the required frequency range. This could make testing in higher and lower frequencies become unnecessarily complex and time-consuming. Therefore, a new or additional method for the OTA co-location testing may be needed to simplify testing.

<End of change