# 3GPP TSG-RAN WG4 Meeting #116	R4-2512578

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Way Forward on Tx spur co-ex

Agenda Item:	7.9.1

Source: 	Huawei

Document for:	Approval

# Way forward

The following is agreed towards simulation assumptions and their alignment.

- CDF %-tile

For calibration: Use CDF curve of coupling loss to align results. Aim to follow the stepwise approach for results alignment, using the following Excel spreadsheet as starting point.

For results derivation: Select just one value: TBD

- ISD

For calibration: select just one value (select 450m, or 500m): 500m

For results derivation: [Same value as for calibration.]

- Grid shift

For calibration: select just one value (50%, or 100%): 100%

For results derivation: TBD

- Antenna correlation value for the OoB antenna gain

For calibration: first 0, then 1.

For results derivation: (0, 0.5, or 1): TBD

- Evaluation scenarios

Grid: 19sites, 3 sectors, wrap around

For calibration:

- First step: WA for BS-BS (Urban macro in TR 38.922)

- Second step: MR for BS-UE (Dense urban in TR 38.922)

For results derivation: [Same value as for calibration.]

- Pathloss model

For calibration (decide which model to use): FFS as further inputs expected; aim to discuss offline after the meeting to align before RAN4#116bis.

For results derivation: [Same value as for calibration.]

- Other assumptions

For calibration: Based on TR 38.922, with interfering system at 2GHz and victim system at 6.425GHz.

- Timeline of this objective

Proceed with the assumption that the Core objective for Tx spur co-ex is extended by 1 quarter till December 2025.

- Simulation calibration in October meeting.

- Aim for conclusions and final CRs (if any) for November meeting.

- How and where to capture outcomes of this discussion

Companies agree to capture conclusions in a TR. Which TR to use as baseline is TBD