TSG-RAN Working Group 4 (Radio) meeting #116	R4-2512581

Bangalore, India, 25th to 29th August 2025

Source:	Ericsson

Title:	Way-Forward for RAN Task on specification improvements

Agenda item:	10.1

Document for:	Approval

# Introduction

At RAN4#116 the work related to the RAN task to improve BS RF specification structure for co-existence and co-location requirements progressed following issues captured in topic summary [1].

Based on the discussion during Tuesday evening session for thread [116][301] BSRF_Maintenance_TEI topic 5 agreements was captured in a way-forward to stimulate progress at coming meetings.

As background the approach to simplify the requirement structure is captured as background in Annex 5.

# Way-forward

Agreements:

- The specification simplification update applies to both conducted and OTA requirements.

- Follow option 2, not introduce Annex in MSR specification with complete frequency list.

- For the scenario where we have separate tables for BS classes, void old tables and introduce new common table for all the classes. Check reference for old tables, if needed update references.

- For out-of-band blocking adopt the same approach as used for NR for co-location requirements.

- Special exception related to specific bands needs to be captured as notes under the new table (e.g., IMT BS and FRMCS BS, IMT BS and broadcasting BS, V2X, Unlicensed bands, etc.).

- Adopt the approach exemplified in [2] and [3] as baseline for spurious emission coexistence, co-location spurious emission and co-location blocking requirements.

- At next meetings further check and if needed update notes below the requirement tables. E.g. update the NOTE1 for co-existence requirement as the following to solve the deleting note in note column of original table.

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.5.1.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the downlink operating band (see TS 38.104 [2], table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

# Work plan

To guid the work to complete the work the following workplan have been created:

- At RAN4#116 (This meeting) review baseline CRs in [2, 3] and if possible, technically endorse draft CRs for TS 37.104 and TS 38.141-1.

- At RAN4#116-bis meeting in October collect and endorse draft CRs according to work split.

- At RAN4#117 meeting in November approve formal CRs for the complete package.

- At RAN#110 in December report RAN task completed.

The work split from RAN4#114-bis [4] is captured below:

| Specification (TS) |  | Ericsson | Nokia | ZTE | CATT | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
| 38.104 | NR BS Core | X |  |  |  |  |
| 38.141-1 | NR BS Conf - Conducted |  | X |  |  |  |
| 38-141-2 | NR BS Conf - OTA |  |  |  |  | X |
| 38.106 | NR Repeater - Core |  |  |  | X |  |
| 38.115-1 | NR Repeater -Conf Cond. |  |  | X |  |  |
| 38.115-2 | NR Repeater -Conf OTA No impact |  |  |  |  |  |
| 38.174 | IAB Core |  |  |  | X |  |
| 38.176-1 | IAB Conf Conducted |  |  | X |  |  |
| 38.176-2 | IAB Conf OTA |  |  | X |  |  |
| 37.105 | AAS BS Core | X |  |  |  |  |
| 37.145-1 | AAS BS Conf Conducted |  |  |  | X |  |
| 37.145-2 | AAS BS Conf OTA |  |  |  |  | X |
| 37.104 | MSR BS Core |  | X |  |  |  |
| 37.141 | MSR BS Conf |  |  | X |  |  |
| 36.104 | E-UTRA BS Core | X |  |  |  |  |
| 36.141 | E-UTRA BS Conf |  | X |  |  |  |
| 36.106 | E-UTRA Repeater Core |  |  |  | X |  |
| 36.143 | E-UTRA Repeater Conf |  |  |  |  | X |

# References

[1]	R4-2511448, “Topic summary for [116][301] BSRF_Maintenance_TEI”, Moderator (Ericsson)

[2]	R4-2511663, “(TEI19) CR to 37.104 on framework simplification for co-location/co-existence requirements”, Nokia

[3]	R4-2511664, “(TEI19) CR to 38.141-1 on framework simplification for co-location/co-existence requirements”, Nokia

[4]	R4-2504741, “Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex”, Moderator (Ericsson)

[5]	R4-2508664, “Way forward for [115][340] BSRF_Simplify_CoLo_Coex”, Moderator (Ericsson)

# Annex

As background the approach is to consolidate requirement tables as exemplified below.

Transmitter co-existence spurious emission requirement in 37.104, clause 6.6.1.3.1:

| System type to co-exist with | Frequency range for co-existence requirement | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 MHz | -57 | 100kHz | NOTE 1, 3 |
|  | 824 - 849 MHz | -61 |  |  |
| GSM900 | 921 - 960 MHz | -57 |  |  |
|  | 876 - 915 MHz | -61 |  |  |
| DCS1800 | 1805 - 1880 MHz | -47 |  |  |
|  | 1710 - 1785 MHz | -61 |  |  |
| PCS1900 | 1930 - 1990 MHz | -47 |  |  |
|  | 1850 - 1910 MHz | -61 |  |  |
| Other system type | Frequency range of downlink operating band of the BS to co-exist with (NOTE 8, 9) | -52 | 1MHz | NOTE 1, 3 |
|  | Frequency range of uplink operating band of the BS to co-exist with (NOTE 9) | -49 |  | NOTE 1, 3, 5, 6, 7 |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in Table 6.6.1.3.1-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see Tables 4.5-1 and 4.5-2). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.1.3.1-1 assumes that two operating bands, where the frequency ranges in Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III, E-UTRA Band 3 or NR Band n3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 5:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band, where requirement applies 4 MHz above the Band n28 downlink operating band.

NOTE 6:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 or NR Band n12 UL operating band or E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band, where requirement applies 1 MHz below the Band 29 downlink operating band.

NOTE 7:	For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz.

NOTE 8:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104, table 5.5-1.

NOTE 9:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 37.104 annex X.

Transmitter co-location spurious emission requirement in TS 37.104, clause 6.6.1.4.1:

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93 | NR | N/A | N/A | -88 |  |
| UL frequency range of n53 | NR | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in Table 6.6.1.4.1-1 do not apply for the ΔfOBUE frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Tables 4.5-1 and 4.5-2). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7].

NOTE 2:	Table 6.6.1.4.1-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	Does not apply for co-location with V2X operation defined in TS 36.104, table 5.5-1.

NOTE 5:	UTRA, E-UTRA, NR as described in annex X in TS 37.104.

Co-location receiver blocking requirement in TS 37.104, clause 7.5.2:

Table 7.5.2-1: Blocking requirement for co-location with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS + x dB (Note 1) | +16 | +8 | -6 | CW carrier |
| NOTE 1: PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2. "x" is equal to 3 in case of GSM/EDGE wanted signal and equal to 6 in case of NR or UTRA or E-UTRA or NB-IoT wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD or NR TDD with E-UTRA FDD or NR FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805-1850MHz. NOTE 5: For a BS operating in band 11, 21, 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 6: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 7: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |