3GPP TSG-RAN WG4 Meeting #116	R4-2512644

Bangaluru, India, August 25th ‒ August 29th, 2025

Title: 	Way Forward for [116][331] NR_NTN_Ph3_demod

Agenda Item:	7.27.1

Source:	Ericsson

Document for:	Approval

# Topic #1: SAN demodulation requirements

## Sub-topic 1-1	PUSCH requirement with OCC length 4

Issue 1-1-1: Define PUSCH demodulation requirements with OCC length 4 or not

Agreement:

- Consider OCC length 2 only to define PUSCH demodulation requirements

## Sub-topic 1-2 Test setup for OCC length 2

Issue 1-2-1: OCC group configuration

Agreement:

- Consider one OCC group with aggregation factor 2 only. Initial RV configured as 0.

Issue 1-2-2: SCS/CBW

Agreement:

- FDD 15kHz/5MHz

- FDD 30kHz/10MHz

Issue 1-2-3: Frequency offset of two multiplexed UEs.

Way forward:

- Consider setting the same frequency offset for two UEs.

- Evaluate two frequency offsets: 0 Hz and 200 Hz. Down select to one value according to the simulation results.

- If there is no performance difference, select 200Hz.

Issue 1-2-4: Channel model

Way forward:

- Apply the same channel model to the multiplex UEs.

- Consider NTN-TDLC5-200. Interested companies can provide the simulation results with NTN-TDLA100-200.

- Down select one channel model according to the simulation results.

Issue 1-2-5: MCS

Agreement:

- MCS5 (QPSK, CR=99/1024) in MCS index Table 2 (TS 38.214 Table 6.1.4.1-2)

Issue 1-2-6: Test metric

Way forward:

- SNR at 70% of maximum throughput.

- Measure the PUSCH throughputs per UE.

  - UE1 uses Orthogonal sequence index 0

  - UE2 uses Orthogonal sequence index 1

- Set the same SNR for PUSCH from two UEs if there is no performance difference.

- FFS how to write the specification e.g., define PUSCH demodulation requirements for each UE.

Issue 1-2-7: Transmission/Reception signal power

Agreement:

- Assume the same receiving power for two UEs at gNB side

- Assume the same transmission power from two UEs.

Issue 1-2-8: Antenna configuration

Way forward:

- 1Tx – 1Rx

- 1Tx – 2Rx

- FFS whether to apply the existing SAN PUSCH demodulation requirements applicability between 1Rx and 2Rx requirements or not.

Issue 1-2-9: Other parameters including DMRS configuration

Agreement:

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | UE0 | UE1 |
|  |  | 0 | 1 |
|  | DM-RS sequence generation | Disable frequency and sequence hopping nSCID=0 NID0=0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |

Issue 1-2-10: Simulation alignment

Way forward:

In RAN4#116bis, interested companies are encouraged to provide simulation results of SAN demodulation for PUSCH with OCC length 2.

# Topic #2: UE demodulation requirements

## Sub-topic 2-1 RedCap demodulation for NTN

Issue 2-1-1: RedCap 1Tx-1Rx, 10MHz/15kHz, FDD/HD-FDD

Agreement:

Specify the following test cases:

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 |

Issue 2-1-2: FRC for HD-FDD RedCap demodulation

Way forward:

- Reuse “R.PDSCH.1-1.2 HD-FDD” in TS38.101-4 for MCS13/52PRB test.

- FFS whether to define a new HD-FDD reference channel for MCS4/52PRB tests.

  - Note: R.PDSCH.1-1.1 HD-FDD in TS 38.101-3 configures 18 DM-RS REs. MediaTek proposes to configure 12 DM-RS REs (1+1) instead of 18 REs (1+1+1).

## Sub-topic 2-2 eRedCap demodulation for NTN

Issue 2-2-1: eRedCap 1Rx and 2Rx

Agreement:

- Apply the same PDSCH demodulation test setup for eRedCap 1Rx and 2Rx UEs.

Issue 2-2-2: eRedCap 1Tx-1Rx/2Rx, 10MHz/15kHz, FDD/HD-FDD

Agreement:

- Specify the following test cases for the eRedCap 1Rx/2Rx UE without baseband bandwidth reduction capability:

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 |

Note 1: Test 1 is the same requirement as RedCap Test 1 in Issue 2-2-1.

Note 2: Test 3 is the same requirement as TS 38.101-5 Table 8.2.1.2.2.1.1-3 Test 1-1.

- Specify the following test cases for the eRedCap 1Rx/2Rx UE with baseband bandwidth reduction capability:

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 |

Issue 2-2-3: FRC for eRedCap demodulation

Way forward:

- Reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth

- FFS for other test cases.

Issue 2-2-5: Applicability rule between With/Without BB BW reduction requirements

Agreement:

- Follow Rel-18 eRedCap test applicability specified in TS 38.101-4.

Issue 2-2-6: Simulation alignment

Way forward:

In RAN4#116bis, interested companies are encouraged to provide simulation results of RedCap 1Rx UE and eRedCap 1Rx/2Rx demodulation according to Issue 2-1-1 and Issue 2-2-2.

Issue 2-3-1: Applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirements.

Way forward:

- Postpone the discussion depending on WI NR_IoT_NTN_req_test_enh discussion.