3GPP TSG-RAN WG4 Meeting #116	R4-2512655

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NTN Ku bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add OTA dynamic range, OTA in-band selectivity and blocking requirements for NTN Ku bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA dynamic range, OTA in-band selectivity and blocking requirements for NTN Ku bands are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4, 10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

10.4	OTA dynamic range

10.4.1	General

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received SAN channel bandwidth.

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

10.4.2	Minimum requirement for SAN type 1-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 10.4.2-1 for LEO SAN.

For NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A of TS 36.108 [17] with parameters specified in table 10.4.2-1a for LEO SAN.

The requirement is not applicable for SAN type 1-O operating above 10GHz.

Table 10.4.2-1: SAN LEO class dynamic range

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-NTN-A2-7 | -79.3- ΔOTAREFSENS | -90.4- ΔOTAREFSENS | AWGN |
| 5 | 15 | G-FR1-NTN-A2-1 | -76.4 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -77.1 - ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-NTN-A2-1 | -76.4 - ΔOTAREFSENS | -85.0 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -77.1 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -74.1- ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-NTN-A2-1 | -76.4- ΔOTAREFSENS | -83.2 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -77.1 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -74.1 - ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-NTN-A2-4 | -70.2 - ΔOTAREFSENS | -81.9 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-5 | -70.2 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-6 | -70.5 - ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |  |

Table 10.4.2-1a: LEO SAN dynamic range for NB-IoT operation in NTN NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | FRC A15-1 in Annex A.15 in TS 36.108 [17] | -89.4 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
| 10 |  |  | -85.0 - ΔOTAREFSENS |  |
| 15 |  |  | -83.2 - ΔOTAREFSENS |  |
| 20 |  |  | -81.9 - ΔOTAREFSENS |  |
| 5 | FRC A15-2 in Annex A.15 in TS 36.108 [17] | -95.3 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
| 10 |  |  | -85.0 - ΔOTAREFSENS |  |
| 15 |  |  | -83.2 - ΔOTAREFSENS |  |
| 20 |  |  | -81.9 - ΔOTAREFSENS |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |

<< Next change >>

10.5	OTA in-band selectivity and blocking

10.5.1	OTA adjacent channel selectivity

10.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

10.5.1.2	Minimum requirement for SAN type 1-O operating below 10 GHz

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1-NTN below 10 GHz, the OTA wanted signal and the interfering signal are specified in table 10.5.1.2-1 and table 10.5.1.2-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristic of the interfering signal is further specified in annex C.

For SAN supporting NB-IoT operation in NTN NR in-band, the wanted and the interfering signal at the SAN type 1-O RIB are specified in table 10.5.1.2-1 and the frequency offset between the wanted and interfering signal in table 10.5.1.2-2 for ACS. The reference measurement channel for the NB-IoT wanted signal is identified in clause 10.3.2 of TS 36.108 [17]. The characteristics of the interfering signal is further specified in annex C.

The OTA ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Table 10.5.1.2-1: OTA ACS requirement for SAN type 1-O operating below 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (NOTE 2) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3 | EISminSENS + 8 dB | SAN LEO class: -60 – ΔminSENS SAN GEO class: -57 – ΔminSENS |
| 5, 10, 15, 20 (NOTE 1) | EISminSENS + 6 dB |  |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth NOTE 2: EISminSENS depends on the SAN channel bandwidth |  |  |

Table 10.5.1.2-2: OTA ACS interferer frequency offset for SAN type 1-O operating below 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±1.5075 | 3 MHz CP-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 5 | ±2.5025 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |

10.5.1.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1-NTN above 10 GHz, the OTA wanted signal and the interfering signal are specified in table 10.5.1.2a-1 and table 10.5.1.2a-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristic of the interfering signal is further specified in annex C.

The OTA ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Table 10.5.1.2a-1: OTA ACS requirement for SAN type 1-O operating above 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (NOTE 2) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 10, 20, 25, 35, 50, 70, 100 (Note 1) | EISREFSENS + 6 dB | SAN LEO class: EISREFSENS_50M + 29.7 + ΔFR2_REFSENS SAN GEO class: EISREFSENS_50M + 23.7 + ΔFR2_REFSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth NOTE 2: EISREFSENS is given in clause 10.3.2 |  |  |

Table 10.5.1.2a-2: OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 10 | ±2.5075 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 35 | ±9.4625 |  |
| 50 | ±9.4625 |  |
| 70 | ±9.4675 |  |
| 100 | ±9.4675 |  |

10.5.1.3	Minimum requirement for SAN type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2-NTN, the OTA wanted and the interfering signal are specified in table 10.5.1.3-1 and table 10.5.1.3-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA ACS requirement is applicable outside the SAN RF Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 10.5.1.3-1: OTA ACS requirement for SAN type 2-O

| SAN operating band | SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- | --- |
| n510, n511, n512 | 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 1) | SAN LEO class: EISREFSENS_50M + 27.7+ ΔFR2_REFSENS SAN GEO class: EISREFSENS_50M + 21.7 + ΔFR2_REFSENS |
| n508, n509 | 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 1) | SAN LEO class: EISREFSENS_50M + 29.7 + ΔFR2_REFSENS SAN GEO class: EISREFSENS_50M + 23.7 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is given in clause 10.3.3 |  |  |  |

Table 10.5.1.3-2: OTA ACS interferer frequency offset for SAN type 2-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 | ±25.01 |  |
| 100 | ±25.03 | 50 MHz CP-OFDM NR |
| 200 | ±25.01 | signal,60 kHz SCS, 66 RBs |
| 400 | ±25.03 |  |

10.5.2	OTA in-band blocking

The requirement is not applicable in this version of the specification.

<< End change >>