TSG-RAN Working Group 4 (Radio) meeting #116	                                               R4-2512658

Bengaluru, IN, Aug 25th – 29th, 2025

Title:	LS on UE Capability for Rel.19 Ku band VSAT

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ku_bands

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Bo-Han Hsieh (Tank)

E-mail Address: pohanhsieh@cht.com.tw

Tomoki Yoshimura

E-mail Address: tomoki_yoshimura@sharp.co.jp

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Nil

1	Overall description

Based on the updated WID in RP-250799, RAN4 has started the work on defining two sets of bands for the NR NTN Ku bands, one for FR1 numerology and the other for FR2. The group has decided to extend both FR1-NTN and FR2-NTN frequency ranges to cover 10.7~14.5GHz frequency ranges in Rel.19 38.101-5 specification as Table 1 below. So far, two FR1-NTN bands (n248, n247) and two FR2-NTN bands (n509, n508) have been agreed to be included as the priority 1 Ku bands under the WID. A running CR capturing the aforementioned bands was endorsed during last RAN4 meeting in R4-2505737.

Table 1 Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

While working on the requirements for the Rel.19 NTN Ku band VSAT, RAN4 noticed that the two signalling parameters introduced in Rel.18, ntn-VSAT-AntennaType-r18 and ntn-VSAT-MobilityType-r18, are limited to FR2 only, as shown in Table 2 below.

Table 2 Rel.18 VSAT related capability signalling

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ntn-VSAT-AntennaType-r18 Indicates whether a VSAT UE uses electronic or mechanical steering antenna. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only |
| ntn-VSAT-MobilityType-r18 Indicates whether a VSAT UE is a mobile or fixed VSAT. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only |

Based on the discussion in RAN4#116 meeting, RAN4 suggest to extend the ntn-VSAT-AntennaType-r18 and ntn-VSAT-MobilityType-r18 to FR1 in Rel.19 for the NTN Ku bands, while the detail signaling design is up to RAN2.

2	Actions

To 3GPP TSG RAN2

ACTION:   RAN4 would like to ask RAN2 to take the information above into account and specify the necessary capability signaling.

3	Dates of next TSG RAN WG 4 meetings

3GPP RAN4 #116bis			October 13th – 17th, 2025		Prague, Czech Republic

3GPP RAN4 #117			November 17th – 21th, 2025		Dallas, US