3GPP TSG-RAN WG4 Meeting #116	R4-2512663

Bangaluru, India, 25th – 29th August 2025                                    (revision of R4-2511154)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.108 – for unwanted emission requirements for NTN Ku bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NTN Ku bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add OTA unwanted emission requirements for NTN Ku bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA unwanted emissions for SAN not updated correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change >>

## 9.7	OTA unwanted emissions

### 9.7.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [2]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the SAN channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emissions requirement for the SAN type 1-O is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and out-of-band emissions (OOBE). The unwanted emission requirements are applied per cell for all the configurations. Requirements for OTA unwanted emissions are captured as TRP requirements or directional requirements, as described per requirement.

There is in addition a requirement for occupied bandwidth.

### 9.7.2	OTA occupied bandwidth

#### 9.7.2.1	General

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [8].

The value of /2 shall be taken as 0.5%.

The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer's declared OTA coverage range at the RIB.

For NB-IoT operation in NTN NR in-band, the occupied bandwidth for each NR carrier with NB-IoT shall be less than the SAN channel bandwidth.

#### 9.7.2.2	Minimum requirement for SAN type 1-O and SAN type 2-O

The OTA occupied bandwidth for each carrier shall be less than the SAN channel bandwidth.

### 9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 9.7.3.1	General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

#### 9.7.3.2	Minimum requirement for SAN type 1-O operating below 10 GHz

The ACLR limit specified in tables 6.6.3.2-1 for SAN GEO class and 6.6.3.2-2 for SAN LEO class shall apply.

For a RIB operating in multi-carrier, the ACLR requirements in clause 6.6.3.2 shall apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in tables 6.6.3.2-1 and 6.6.3.2-2.

#### 9.7.3.2a	Minimum requirement for SAN type 1-O operating above 10 GHz

The ACLR limit specified in tables 9.7.3.2a-1 for SAN GEO class and 9.7.3.2a-2 for SAN LEO class shall apply.

For a RIB operating in multi-carrier, the ACLR requirements in clause 9.7.3.2a shall apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in tables 9.7.3.2a-1 and 9.7.3.2a-2.

Table 9.7.3.2a-1: SAN type 1-O ACLR limit for SAN GEO class

| SAN Operating Band | SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| n248, n247 | 10, 15, 20, 25, 35, 50, 70,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 16 |  |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |  |

Table 9.7.3.2a-2: SAN type 1-O ACLR limit for SAN LEO class

| SAN Operating Band | SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| n248, n247 | 10, 15, 20, 25, 35, 50, 70,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 16 |  |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |  |

#### 9.7.3.3	Minimum requirement for SAN type 2-O

The ACLR limit specified in tables 9.7.3.3-1 for SAN GEO class and 9.7.3.3-2 for SAN LEO class shall apply.

For a RIB operating in multi-carrier, the ACLR requirements in clause 9.7.3.3 shall apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in tables 9.7.3.3-1 and 9.7.3.3-2.

Table 9.7.3.3-1: SAN type 2-O ACLR limit for SAN GEO class

| SAN Operating Band | SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| n512, n511, n510 | 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 12 |  |
| n509, n508 | 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 16 |  |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |  |

Table 9.7.3.3-2: SAN type 2-O ACLR limit for SAN LEO class

| SAN Operating Band | SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| n512, n511, n510 | 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 12 |  |
| n509, n508 | 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 16 |  |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |  |  |

### 9.7.4	OTA out-of-band emissions

#### 9.7.4.1	General

The OTA limits for out-of-band emissions are specified as TRP per RIB unless otherwise stated.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

#### 9.7.4.2	Minimum requirement for SAN type 1-O

Out-of-band emissions in FR1-NTN are limited by OTA out-of-band emission limits. Unless otherwise stated, the out-of-band emission limits in FR1-NTN are defined from channel edge up to frequencies separated from the channel edge by 200% of the necessary bandwidth. The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in multi-carrier, the requirements apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in clause 6.6.4.1.

The OTA out-of-band emissions requirement for SAN type 1-O shall not exceed each applicable limit in clause 6.6.4.2.

#### 9.7.4.3	Minimum requirement for SAN type 2-O

Out-of-band emissions in FR2-NTN are limited by OTA out-of-band emission limits. Unless otherwise stated, the out-of-band emission limits in FR2-NTN are defined from channel edge up to frequencies separated from the channel edge by 200% of the necessary bandwidth. The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in multi-carrier, the requirements apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in clause 6.6.4.1.

The OTA out-of-band emissions requirement for SAN type 2-O shall not exceed each applicable limit in clause 6.6.4.2.

### 9.7.5	OTA transmitter spurious emissions

#### 9.7.5.1	General

Unless otherwise stated, all requirements are measured as mean power.

The OTA spurious emissions limits are specified as TRP per RIB unless otherwise stated.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

#### 9.7.5.2	Minimum requirement for SAN type 1-O

##### 9.7.5.2.1	General

The OTA transmitter spurious emission limits for FR1-NTN shall apply from 30 MHz to the 5th harmonic of the upper frequency edge of the DL operating band, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

##### 9.7.5.2.2	General OTA transmitter spurious emissions requirements

The basic limits of table 9.7.5.2.2-1 shall apply. The application of those limits shall be the same as for out-of-band emissionsin clause 6.6.4.

Table 9.7.5.2.2-1: General SAN transmitter spurious emission basic limits in FR1-NTN

| Spurious frequency range | Prated,t,TRP (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,t,TRP – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [2], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

The transmitter spurious emissions minimum requirements for SAN type 1-O are that the power summation emissions at the TAB connectors shall not exceed the basic limit in table 9.7.5.2.2-1.

##### 9.7.5.2.3	Protection of the SAN receiver

The co-location requirement is not applicable for SAN in this version of the specification.

##### 9.7.5.2.4	Additional spurious emissions requirements

The additional spurious emissions requirement is not applicable for SAN.

#### 9.7.5.3	Minimum requirement for SAN type 2-O

##### 9.7.5.3.1	General

The OTA transmitter spurious emission limits for FR2-NTN shall apply from 30 MHz to the 2nd harmonic of the upper frequency edge of the DL operating band, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

#### 9.7.5.3.2	General OTA transmitter spurious emissions requirements

The basic limits of table 9.7.5.3.2-1 shall apply. The application of those limits shall be the same as for operating band unwanted emissions in clause 9.7.4.3.

Table 9.7.5.3.2-1: General radiated SAN transmitter spurious emission limits in FR2-NTN

| Spurious frequency range | Prated,t,TRP (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 2nd harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,t,TRP – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [2], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

##### 9.7.5.3.3	Protection of the SAN receiver

The co-location requirement is not applicable for SAN in this version of the specification.

##### 9.7.5.3.4	Additional spurious emissions requirements

The additional spurious emissions requirement is not applicable for SAN.

## 9.8	OTA transmitter intermodulation

The requirement is not applicable in this version of the specification.

## << End change >>