3GPP TSG-RAN WG4 Meeting #116	R4-2512822

# Bangalore, India,  25th August – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0040 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the L-band NTN HPUE power back-off simulations to TR 38.863 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Oppo, Qualcomm, LGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the NTN high-power UE feature will enable power classes higher than 23dBm for the NTN bands. However, since some of the NTN bands have additional regulatiry emission requirements, additional analysis is required to check required power back-off values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Annex is added to capture power back-off simulations results based on the input from the following contributions: R4-2511400 (Qualcomm) R4-2510521 (Oppo) R4-2510776 (LGE) R4-2509536 (Apple) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.863 will not have technical analysis regarding power back-off needed to meet existing regulatory emission requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex E:
Power back-off simulation results for the L-band

## E.1	PC3 power class

### E.1.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

#### E.1.1.1	NS_02N

##### E.1.1.1.1	Company A (OPPO/R4-2510521)

##### E.1.1.1.1.1	Simulation assumptions

Since filter rejection at the frequencies 1559-1610MHz is not a constant 10dB attenuation, but rather has a slope, it is hard to add the filter rejection without the slope factor. Thus, to evaluate the rejection effect of the filter, required power back-off is simulated under the following assumptions:

-	no filter rejection is assumed;

-	PC3 CP-OFDM QPSK using fixed bias PA and APT PA;

-	5, 10, 15 and 20MHz channels residing at the lowest edge of the uplink operating band n255

##### E.2.1.1.1.2	Simulation results

For PC3 CP-OFDM QPSK using fixed bias PA:

-	Power back-off 0dB is needed for 5MHz, 10MHz and 15MHz without additional filter rejection.

-	Power back-off 3dB is needed for 20MHz without additional filter rejection.

For PC3 CP-OFDM QPSK using APT PA:

-	Power back-off 0dB is needed for 5MHz and 10MHz without additional filter rejection.

-	Power back-off 2dB for 15MHz and 10.5dB for 20MHz without additional filter rejection.

Based on the simulation results, no matter which PA model is used, the filter attenuation is needed for NS_02N for PC3 to be able to operate within existing MPR margins.

## E.2	PC2 power class

### E.2.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

#### E.2.1.1	NS_02N

##### E.2.1.1.1	Company A (Qualcomm/R4-2511400)

###### E.2.1.1.1.1	Simulation assumptions

###### E.2.1.1.1.2	Simulation results

If the 10dB filter rejection is assumed for the frequency range 1559-1610MHz, then a small A-MPR is needed to meet emission requirements. As presented in Table below A-MPR is needed only for the 20MHz channel.

Table E.2.1.1.1.2-1: Proposed A-MPR regions and values.

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 20MHz | 1636.5 ≤ Fc ≤ 1650.5 | ≤ 0.54 | < 1.08 | A1 |
|  |  |  | > 14.4 | A1 |

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 2.5 |
| DFT-s-OFDM 16 QAM | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 4 |

##### E.2.1.1.2	Company B (OPPO/R4-2510521)

###### E.2.1.1.2.1	Simulation assumptions

Simulation assumptions are as follows:

-	PC2 power class

-	10, 15 and 20MHz channels residing at the lowest edge of the uplink operating band n255

-	CP-OFDM and DFT-s-OFDM

-	both cases, with and without 10dB filter attenuation are considered

###### E.2.1.1.2.2	Simulation results

To check the effect of filter rejection, power back-off results are presented for the 10, 15 and 20MHz channels with PC2 CP-OFDM QPSK using APT PA w/o filter rejection and with a constant 10 dBfilter attenuation at the frequencies 1559-1610 MHz. In the following figures, A-MPR and "A-MPR – MPR" simulation results are provided separately, i.e. "A-MPR – MPR" figres show the difference between required power back-off and existing MPR margings.

Figure E.2.1.1.2.2-1: Power back-off change for 10MHz based on a constant 10dB filter rejection using APT PA

From Figure E.2.1.1.2.2-1 for the 10MHz channel it can be seen that:

- For PC2 without filter isolation, the AMPR region starts at LCRB>16RB and the largest AMPR value is around 4.5dB and even larger.

- For PC2 with 10dB filter isolation, we can see that required power back-off is smaller than the existing MPR margin, i.e. there is no need to define A-MPR for 10MHz channel bandwidth.

Figure E.2.1.1.2.2-2: Power back-off change for 15MHz based on a constant 10dB filter rejection using APT PA

From Figure E.2.1.1.2.2-2 for the 15MHz channel it can be seen that:

- The A-MPR regions with the 10dB filter isolation becomes smaller than the A-MPR regions without filter isolation, but the overall required power back-off is still larger than MPR.

- For PC2 with the 10dB filter isolation, the AMPR region starts building at LCRB>30RB, whereupon the largest power back-off is larger than 3.5 dB.

Figure E.2.1.1.2.2-3: Power back-off change for 20MHz based on a constant 10dB filter rejection using APT PA

From Figure E.2.1.1.2.2-3 for the 20MHz channel it can be seen that:

- The A-MPR region with the 10dB filter isolation becomes smaller than the AMPR region without filter isolation, but the required power back-off is larger than MPR.

- For PC2 with the 10dB filter isolation, the AMPR region starts at LCRB>34RB, and the required power back-off is larger than 6dB.

In addition to the above results, Figure E.2.1.1.2.2-4 and E.2.1.1.2.2-5 below presents additional results for the CP-OFDM and DFT-s-OFDM for the 15MHz channel with different modulations.

Figure E.2.1.1.2.2-4: Power back-off for 15MHz CP-OFDM QPSK, 16QAM and 64QAM.

Figure E.2.1.1.2.2-5: Power back-off for 15MHz DFT-s-OFDM PI/2 BPSK, QPSK, 16QAM and 64QAM.

From Figure E.2.1.1.2.2-4 and Figure E.2.1.1.2.2-5 it can be seen that:

- The A-MPR regions and the A-MPR values are almost the same for different modulations for both of CP-OFDM and DFT-s-OFDM.

Considering presented results and the 10dB filter rejection, PC2 A-MPR may look as presented in the table below.

Table E.2.1.1.2.2-1: Proposed A-MPR regions and values

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 15MHz | 1634 < Fc ≤ 1649 |  | >10.8 | A1 |
| 20MHz | 1636.5 < Fc ≤ 1650.5 |  | >10.8 | A2 |
|  |  | <0.54 | <1.8 |  |

| Modulation |  | A1 | A2 |
| --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 3 |
|  | QPSK/16QAM/64QAM | ≤ 3.5 | ≤ 6 |
| CP-OFDM | QPSK/16QAM/64QAM | ≤ 4 | ≤ 6.5 |

##### E.2.1.1.3	Company C (LGE/R4-2510776)

###### E.2.1.1.3.1	Simulation assumptions

Simulation assumptions are as follows:

-	PC2 power class

-	5, 10, 15 and 20MHz channels in the frequency range of 1629-1654MHz

-	DFT-s-OFDM QPSK

-	both cases, with and without 10dB filter attenuation are considered

###### E.2.1.1.3.2	Simulation results

Table below presents required power back-off values for two cases with and without 10dB filter rejection. According to the simulation results, there is only one case without the 10dB filter at 20 MHz bandwidth and center frequency of 1636.5MHz (RBstart: 0, LCRB: 51) requires A-MPR driven by the NS_02N limit. In all other cases, power back-off is determined by factors other than NS_02N, such as ACLR or SEM. Furthermore, when the 10 dB filtering is applied, none of the scenarios require A-MPR driven by NS_02N.

Table E.2.1.1.3.2-1: Summary of A-MPR simulations for DFT-s-OFDM QPSK.

| Channel bandwidth | Frequency range | (RB_start, LCRB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5MHz | 1629 < Fc ≤ 1654 | (0,11) | (0,1) |  |  |
| PC2 W/O 10dB |  | 0 | 1.47 |  |  |
| PC2 W 10dB |  | 0 | 1.47 |  |  |
| 10MHz | 1631.5 < Fc ≤ 1641.5 | (0,24) | (0,1) |  |  |
| PC2 W/O 10dB |  | 0 | 2.09 |  |  |
| PC2 W 10dB |  | 0 | 2.09 |  |  |
| 10MHz | 1641.5 < Fc ≤ 1655.5 | (0,24) | (0,1) |  |  |
| PC2 W/O 10dB |  | 0 | 2.03 |  |  |
| PC2 W 10dB |  | 0 | 2.03 |  |  |
| 15MHz | 1634 < Fc ≤ 1649 | (0,38) | (0,1) |  |  |
| PC2 W/O 10dB |  | 0.31 | 4.03 |  |  |
| PC2 W 10dB |  | 0.31 | 4.03 |  |  |
| 15MHz | 1649 < Fc ≤ 1653 | (0,38) | (0,1) |  |  |
| PC2 W/O 10dB |  | 0.34 | 4.16 |  |  |
| PC2 W 10dB |  | 0.34 | 4.16 |  |  |
| 20MHz | 1636.5 < Fc ≤ 1640.5 | (0,19) | (0,51) | (0,1) | (50,1) |
| PC2 W/O 10dB |  | 0 | 1.19 | 3.69 | 3.28 |
| PC2 W 10dB |  | 0 | 0.38 | 3.69 | 3.28 |
| 20MHz | 1640.5 < Fc ≤ 1650.5 | (0,38) | (0,1) | (0,1) | (50,1) |
| PC2 W/O 10dB |  | 0 | 0.38 | 3.72 | 3.09 |
| PC2 W 10dB |  | 0 | 0.38 | 3.72 | 3.09 |

Additionally, further simulations are conducted as shown below to determine the required A-MPR region for the 20MHz channel bandwidth. Accordin to these results A-MPR would be needed for scenarios from (0,51) to (0,45).

Table E.2.1.1.3.2-2:	Further results for the 20MHz channel at 1636.5 < Fc ≤ 1640.5.

| Channel | (RB start, LCRB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20MHz | (0,51) | (0,49) | (0,47) | (0,45) | (0,43) | (0,41) | (0,39) | (0,37) |
| DFT-s-OFDM QPSK | 1.19 | 0.97 | 0.72 | 0.41 | 0.34 | 0.38 | 0.41 | 0.41 |

Based on the presented results potential A-MPR definitions may look as follows.

Table E.2.1.1.3.2-3: Proposed A-MPR regions and values

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 20MHz | 1636.5 < Fc ≤ 1640.5 |  | >16.2 | A1 |

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 1.19 |
| DFT-s-OFDM QPSK | 1.19 |
| DFT-s-OFDM 16 QAM | 1.41 |
| DFT-s-OFDM 64 QAM | 1.44 |
| CP-OFDM QPSK | 1.91 |
| CP-OFDM 16 QAM | 1.94 |
| CP-OFDM 64 QAM | 1.91 |

##### E.2.1.1.4	Company D (Apple/R4-2509536)

###### E.2.1.1.4.1	Simulation assumptions

Simulation assumptions are as follows:

-	Tx power: PC2 (+26dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: Requirements from NS_02N

-	10dB filter rejection is assumed

For the sake of brevity, simulations are considered for the 5, 10, 15 and 20MHz channel bandwidths, whereupon the corresponding channel is always located at the lower edge of the n255 band:

-	5MHz: Fc=1629MHz

-	10MHz: Fc=1631.5MHz

-	15MHz: Fc=1634MHz

-	20MHz: Fc=1636.5MHz

###### E.2.1.1.4.2	Simulation results

According to the preliminary simulation results, no additional power back-off is needed for the 5 and 10MHz channel bandwidths. Thus, the corresponding figures for these channel bandwidths will not be presented.

As for the 15 and 20MHz channels, some power back-off is needed as can be seen from figures below.  Nevertheless, the 15MHz channel power back-off is quite marginal and is within the existing MPR margins. As for the 20MHz channel, it naturally experiences higher power back-off, which in some cases reaches existing MPR margins, especially for the full channel and single RB edge allocations.

Figure E.2.1.1.4.2-1: Power back-off results for 15MHz at Fc=1634MHz.

Figure E.2.1.1.4.2-2: Power back-off results for 20MHz at Fc=1636.5MHz.

Based on the presented results it is not likely that 15MHz channel needs A-MPR. As for the 20MHz channel, small A-MPR can be considered for large allocations.

## E.3	PC1.5 power class

### E.3.1:	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

#### E.3.1.1	NS_02N

##### E.3.1.1.1	Company B (OPPO/ R4-2510521)

###### E.3.1.1.1.1	Simulation assumptions

The following simulation assumptions are used:

-	PC1.5 power class with 2Tx

-	10, 15 and 20MHz channels residing at the lowest edge of the uplink operating band n255

-	CP-OFDM and DFT-s-OFDM

-	without 10dB filter attenuation

###### E.3.1.1.1.2	Simulation results

Figure E.3.1.1.1.2-1 and Figure E.3.1.1.1.2-2 show the AMPR and AMPR-MPR simulation results for CP-OFDM and DFT-s-OFDM with QPSK for 15MHz and 20MHz sepatately.

Figure E.3.1.1.1.2-1: Power back-off change for 15MHz without filter rejection using APT PA

Figure E.3.1.1.1.2-2: Power back-off change for 20MHz without filter rejection using APT PA

From Figure E.3.1.1.1.2-1 for the 15MHz channel it can be seen that:

- For PC1.5 without filter isolation, the AMPR region is at LCRB>15RB, and the largest AMPR is almost 7.5 dB.

From Figure E.3.1.1.1.2-2 for the 20MHz channel we can find that:

- For PC1.5 without filter isolation, the largest AMPR is almost 9.5 dB for the AMPR region at LCRB>15RBs. And the AMPR region of LCRB<15RBs and RBstart<3RBs.

In additional, due to time limitation, we didn’t do the simulation with 10dB filter isolation totally, we just simulated the AMPR for the point of (RBstart=0, LCRB=52) with 10dB filter isolation, the AMPR with 10dB filter isolation is smaller 1.5dB than the AMPR value without filter isolation.

If the 10dB filter rejection is assumed for the frequency range 1559-1610MHz, PC1.5 A-MPR presents in the table below:

Table E.3.1.1.1.2-1: Proposed A-MPR regions and values.

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 20MHz | 1636.5 ≤ Fc ≤ 1650.5 | ≤ 0.54 | < 1.08 | A1 |
|  |  |  | > 14.4 | A1 |
|  |  |  | > 10.8, ≤ 14.4 | A2 |
| 15MHz | 1634 ≤ Fc ≤ 1645 |  | > 10.8 | A2 |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 8 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 8 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 8 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 8 | ≤ 4 |
| CP-OFDM QPSK | ≤ 8 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 8 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 8 | ≤ 6 |

## E.4	PC1 power class

### E.4.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

#### E.4.1.1	NS_02N

##### E.4.1.1.1	Company A (Qualcomm/R4-2511400)

###### E.4.1.1.1.1	Simulation assumptions

###### E.4.1.1.1.2	Simulation results

If the 10dB filter rejection is assumed for the frequency range 1559-1610MHz, then a small A-MPR is needed to meet emission requirements. As presented in Table below A-MPR is needed only for the 20MHz channel.

Table E.4.1.1.1.2-1: Proposed A-MPR regions and values.

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 20MHz | 1636.5 ≤ Fc ≤ 1650.5 | ≤ 0.54 | < 1.08 | A1 |
|  |  |  | > 14.4 | A1 |
|  |  |  | > 10.8, ≤ 14.4 | A2 |
| 15MHz | 1634 ≤ Fc ≤ 1645 |  | > 10.8 | A2 |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 6 | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 4 |