3GPP TSG-RAN4 Meeting # 116	R4-2512854

Bangalore, IN. 25th – 29th, Aug. 2025

# Source:	ZTE Corporation, Sanechips

# Title:	TP for PRD 01 to correct MSD test point for single UL band with one CC for two-band DL CA

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

This TP for PRD 02[1] is to correct the MSD test point for single UL band with one UL CC  for two-band inter-band DL CA, the reasons are explained in the following:

1. To add the band group definition table

Actually the table 1 and 2 are copied from TR38.719-02-01, in which the band group is defined in the Annex A table. However, the Annex A for PRD 02 is change history part. Without correcting the existing Annex A part, a new table (i.e. table 2a.1) is introduced for the band group definition to align with the note1 descriptions in table 2.

2. UL harmonic/harmonic mixing interference falling into NR-U bands

Currently, the principles and table format for UL harmonic/harmonic mixing interference falling into victim DL NR normal band and victim DL NR-U band are different, and the description in the texts shall only for victim DL NR normal band. In other words, the descriptions in the texts are not applied for the victim DL NR-U bands, a.k.a band n46, band n96 or band n102.

Actually, there were no new principles for UL harmonic/harmonic mixing interference falling into victim DL NR-U bands.

3. UL RB allocation

The Lcrb value for harmonic/harmonic mixing MSD test point have already been implemented in the specification by following the rules (15kHz/30kHz) below:

-          For the UL1/DLx test point, Lcrb=25RB / 12RB,

-          For the UL2/DLx test point, Lcrb=12RB / 6RB,

-          For the UL3/DLx test point, Lcrb=8RB / 4RB,

-          For the UL4/DLx test point, Lcrb=6RB / 3RB,

-          For the UL5/DLx test point, Lcrb=5RB / 2RB.

Additionally, LCRB = 25RB / 12RB for UL1/DLx harmonic mixing MSD for SCS 15kHz / SCS 30kHz respectively.

The above LCRB is assumed that the UL RB allocations are centered on the UL channel carrier frequency, and scaling Lcrb with the UL harmonic order is implemented. It is used to replace the RBstart method which does not seem to serve any particular goal. Therefore, the RBstart texts are not applied any more.

4.  UL/DL harmonic order and collision type

The descriptions are duplicated and a bit conflict with the notes description in table 1. Since the principle and description for the notes in table1 are the newest, so we propose to remove the old descriptions in the texts.

5. “>ACLR2” test points for cross band isolation

Less than [40]dB is used in the texts. It should be 45dB to keep consistence with Note 1 in table 2.

- Reference

[1] Permanent Reference Document (PRD);Rules, guidelines and ways of working for introduction of band combinations in NR and LTE, v0.3.0

- Text Proposal

----- Start of TP -----

### 5.5.2	Inter-band CA

#### 5.5.2.1	Two-band Inter-band Downlink CA

#### 5.5.2.1.1	One Uplink Band

#### 5.5.2.1.1.1	One Uplink CC

#### Guideline on co-existence studies for harmonic interference and cross-band isolation:

Table 1 summarizes the frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 1: UL/DL harmonics/harmonic mixing analysis.

| UL/DL harmonics |  | nX | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow |  |  |  |  |  |  |
| nY | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| nX | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 2 summarizes the frequency ranges where cross band isolation may occur for CA_nX-nY.

Table 2 Cross-band isolation analysis

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis |  |  |  |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table 2a.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

The band group definition for same or adjacent band-group criterion is defined in table 2a.1.

Table 2a.1: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

#### Guidelines on handling new “greatest/smallest” CBW due to BCS4/5 [6] for MSD due to hamonic and cross-band isolation interference:

Table 3 and Table 4 indicate cases in which the MSD test point must be re-visited, and in cases where the MSD test point may be revisited on a case-by-case basis for:

- NR-CA combinations when a BCS4/5 request introduces a new greatest CBW or a new smallest CBW,

- EN-DC combinations when a new greatest CBW or a new smallest CBW is introduced in one of the constituent frequency band,

- Legacy BCS4/5 NR-CA combinations when a new greatest CBW or a new smallest CBW is introduced in one of the constituent frequency bands.

Table 3: Handling of MSD test points due to cross-band isolation interference

|  | New greatest CBW | New smallest CBW |
| --- | --- | --- |
| UL band | MUST be revisited | May be revisited on case-by-case |
| DL band | May be revisited on case-by-case | MUST be revisited |

Table 4: Handling of MSD due to harmonic interference (uplink and/or Rx harmonic mixing)

|  | New greatest CBW | New smallest CBW |
| --- | --- | --- |
| UL band | May be revisited on case-by-case | May be revisited on case-by-case |
| DL band | May be revisited on case-by-case | MUST be revisited |

It is recommended that proponents provide justification for cases where the MSD test point may need to be revisited.

#### Handling of legacy UEs that may not support the new greatest/smallest CBW introduced by BCS4/5:

For MSD test points due to harmonic interference and cross-band isolation interference:

- To ensure that the conformance testing of legacy UEs is not impacted, the so-called “first MSD test point” remains unchanged, i.e., it remains the baseline mandatory MSD test point.

- When the MSD test point must be or may be revisited on case-by-case basis, the new MSD test point becomes the second optional MSD test point in the current Release where BCS4/5 is introduced,

- If the second optional test point is already specified, the revisited MSD test point replaces/overwrites the legacy agreed second optional test point.

#### Guidelines for MSD Test Points due to direct-hit harmonic interference:

#### These guidelines apply to both MSD due to UL harmonic interference and harmonic mixing interference except the victim NR DL band is either band n46, band n96 or band n102.

Guidelines for the DL band configuration:

- For MSD due to direct-hit harmonic interference, a maximum of two test points can be specified:

- the first test point captures the smallest DL channel bandwidth (CBW) MSD,

- the second test point is optional and may capture the greatest DL channel bandwidth (CBW) MSD. For this second test point, other DL configurations are not precluded.

- The “smallest” and the “greatest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

Guidelines for the UL band configuration:

UL RB allocation:

- The parameter “LCRB” is specified as follows for SCS 15kHz / SCS 30kHz respectively,

- For the UL2/DLx test point, LCRB = 12RB / 6RB,

- For the UL3/DLx test point, LCRB = 8RB / 4RB,

- For the UL4/DLx test point, LCRB = 6RB / 4RB,

- For the UL5/DLx test point, LCRB = 5RB / 2RB.

- For the harmonic mixing UL1/DLx test point, LCRB = 25RB / 12RB,

- where x=1 for MSD due to UL harmonics, and where the valid “x” for MSD due to harmonic mixing is defined in Table 1.

UL band CBW:

- The UL band is configured with the smallest UL CBW and the smallest SCS that can accommodate the specified UL LCRB,

- The “smallest” and the “greatest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

#### Guidelines for MSD Test Points due to near-miss uplink harmonic interference.

- Near-miss MSD test points are specified only for UL2/DL1 harmonic interference and only when the band combination does not meet the direct-hit criteria.

- For near-miss MSD test points:

  - Only one MSD requirement is specified for near-miss,

  - The MSD is specified with:

    - UL CBW = 5MHz,

    - UL LCRB = 25RB (fully allocated),

    - The Note 6 of Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1 is modified as follows:

NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWCℎannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

Guidelines for MSD Test Points due to cross-band isolation.

Guidelines for the DL band configuration:

- The affected DL band carrier centre-frequency shall be configured closest to the UL band.

- A maximum of two test points can be specified:

- the first test point captures the smallest DL channel bandwidth (CBW) MSD,

- the second test point is optional and may capture the greatest DL channel bandwidth (CBW) MSD. For this second test point, other DL configurations are not precluded to account for say, regional spectrum allocations specificities, or proponent concerns on specific CBW of interest etc...

Guidelines for the UL band configuration:

- UL band CBW:

- The UL band is configured with the highest supported CBW. This ensures that the UL band smallest IMD order has a maximum reach towards the DL affected band.

- UL band carrier frequency and SCS:

- The UL band carrier centre frequency is configured closest to the affected DL band.

- The UL band SCS is specified for the lowest SCS that can be supported for the specified UL CBW.

- UL RB allocation:

- The parameter “LCRB” is that specified in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the corresponding UL band CBW.

- The parameter “RBSTART” is specified to ensure the UL RBs are positioned closest to the DL affected band.

- For SUL, the requirements are specified with the LCRB of the NR band counterpart as defined in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the specified SUL band CBW. A SUL-NR counterpart look-up is provided in Table 5.

Table 5: Handling SUL-NR counterpart lookup table

| SUL band | NR UL Band counterpart | FUL_low – FUL_high (MHz) |
| --- | --- | --- |
| n80 | n3 | 1710 – 1785 |
| n81 | n8 | 880 – 915 |
| n82 | n20 | 832 – 862 |
| n83 | n28 | 703 – 748 |
| n84 | n1 | 1920 – 1980 |
| n86 | n66 | 1710 – 1780 |
| n89 | n5 | 824 – 849 |
| n95 | n34 | 2010 – 2025 |
| n97 | n40 | 2300 – 2400 |
| n98 | n39 | 1880 – 1920 |
| n99 | n24 | 1626.5 – 1660.5 |

Guidelines on specifying the “source of interference”:

- The interference source can be either “ACLR1”, “ACLR2”, or greater than ACLR2 (“>ACLR2”).

- For “ACLR1”, or “ACLR2” test points:

  - MSD may be neglected if the order of the IMD product of the modulated RB and its image that overlaps the affected DL band is greater than [9],

  - For RB/Image IMD order less or equal to [7], the MSD needs to be analysed taking into consideration RF-FE complexity (quadplexer, triplexer, etc...),

- For “>ACLR2” test points, aka “flat-band noise cases” / large separation distances:

  - MSD may need to be evaluated if the Tx noise rejection in the DL affected band is less than 45dB. The underlying assumption is that the flat Tx noise PSD is -130dBm/Hz and DL affected CBW is 5MHz.

----- End of TP -----