3GPP TSG-RAN WG4 Meeting #116	R4-2512871

Bengaluru, India, 25th August – 29th August 2025

Source:	Ericsson, Rogers

Title:	TP for TR 37.719-21-11 adding DC_12A_n7A-n77A

Agenda item:			6.2.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_12A_n7A-n77A as defined in WID [1].

# ---Start of changes---

## 7.x	DC_12_n7-n77

### 7.x.1	Configurations for DC

Table 7.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration |
| --- | --- |
| DC_12A_n7A-n77A | DC_12A_n7A DC_12A_n77A |

### 7.x.2	Co-existence analysis for DC

Table 7.x.2-1: Band 12 and Band n7 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2500 | 2570 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1784 | 1871 | 3199 | 3286 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1068 | 1172 | 4284 | 4441 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3898 | 4002 | 5699 | 5856 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 473 | 352 | 6784 | 7011 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3568 | 3742 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4597 | 4718 | 8199 | 8426 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6398 | 6572 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9284 | 9581 | 226 | 364 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6068 | 6312 | 2852 | 3043 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10699 | 10996 | 5296 | 5434 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8898 | 9142 | 7097 | 7288 |

Table 7.x.2-2: Band 12 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1868 | 2802 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5168 | 7002 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7998 | 9832 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12484 | 16101 | 1404 | 436 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8468 | 11202 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

Based on Table 7.x.2-1 and Table 7.x.2-2 there are 2nd and 3rd IMD products than may fall into band n7 and there are 3rd IMD products that might fall into band n77.

### 7.x.3	∆TIB and ∆RIB values

Values are derived from DC_7-12_n77.

Table 7.x.3-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_12_n7-n77 | 0.5 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 7.x.3-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_12_n7-n77 | 0.5 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

### 7.x.4	Analysis of MSD requirements

Based on Table 7.x.2-1 and Table 7.x.2-2 there are 2nd and 3rd IMD products than may fall into band n7. IMD2 MSD value are reused from DC_12A_n7A-n78A. IMD3 MSD value are reused from DC_5A_n41A-n77A.

Based on Table 7.x.2-1 and Table 7.x.2-2 there are 3rd IMD products that might fall into band n77. IMD3 value are reused from DC_7A_n12A-n77A.

Table 7.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n7A-n77A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n7 | N/A | 5 | N/A | 2662 | 29.6 | IMD2 |
|  | n77 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n7 | N/A | 5 | N/A | 2670 | 13.2 | IMD3 |
|  | n77 | 4086 | 10 | 50 | 4086 | N/A | N/A |
|  | 12 | 702 | 5 | 25 | 732 | N/A | N/A |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3909 | 16.0 | IMD3 |
|  | 12 | 673 | 5 | 25 | 732 | N/A | N/A |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3664 | 10.3 | IMD4 |

---End of changes---

# Reference

[1] 	 RP-250966. Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia