3 Tx 3GPP TSG-RAN-WG4 Meeting #116 	R4-2512881

# Bangalore, India, 25th – 29th August, 2025

# Title:	TP for TR38.750 for adding 3Tx HPUE for DL CA_n25-n66-n77

# Source:	T-Mobile USA

# Agenda Item:	6.8.3

# Document for:	Approval

# Introduction

This contribution provides a text proposal for TR38.750 adding the following HPUE NRCA combinations with 3Tx PC2 and PC1.5 for UL CA_n25A-n66A.

- CA_n25A-n66A-n77A

This revision is for Objective 3 HPUE on FDD only for 38.750

# Reference

[1] 3GPP TS38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19)”: V19.0.0

# Text Proposal

## 6.X	CA_n25-n66-n77

### 6.X.1	Configurations

Table 6.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | n257 n667 n777,9 CA_n25A-n66A7,13 CA_n25A-n77A7 CA_n66A-n77A7 |  |  |  |  | n25 |  |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  |  |  |  |  | n66 |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  |  |  | n77 |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  |  | n25 |  |  | n25 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  |  | n66 |  |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |  |
|  |  |  |  |  | n77 |  |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 6.X.2	Maximum output power

<Editor’s note: In table 6.X.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.X.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration |  | Power class 2 cases for CA_nX-nY |  | CA power class |  | Carrier X power class |  | Carrier Y power class |  | Status |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25-n66 |  | Case a |  | 26dBm |  | 23dBm |  | 23dBm |  | Completed |  |
|  |  | Case b |  | 26dBm |  | 23dBm |  | 26dBm |  | New |  |
|  |  | Case c |  | 26dBm |  | 26dBm |  | 23dBm |  | New |  |
|  |  | Case d |  | 26dBm |  | 26dBm |  | 26dBm |  |  |  |

### 6.X.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.X.3.1	Power Class 2 and Power Class 1.5 MSD

There ae several IMDs for the three different uplink configurations. The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD in Table 6.X.3-1.

Table 6.X.3-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration |  | Victim Band |  | IMD |  | PC3 MSD (dB) |  | PC2 MSD (dB) |  | PC1.5 MSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25-n66 | n77 |  | IMD2 |  | 29.4 |  | 35.4 |  |  |  |
|  |  | IMD4 | 8.9 | 19.9 |  |

### 6.X.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.