| 3GPP TS 38.194 V0.0.2 (2025-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for styles	8

4.1	Heading styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <C>: <Informative annex title for a Technical Report>	15

Annex <D> (informative): Bibliography	16

Annex <E> (informative): Index	17

Annex <X> (informative): Change history	18

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document establishes the minimum RF characteristics and minimum performance requirements of Ambient IoT Base Station (BS) and Carrier-Wave (CW) node.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 38.291: " Ambient IoT Physical layer"

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

antenna connector: connector at the conducted interface of the BS type 1-C

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to a BS type 1-C antenna connector

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth.

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [2] used for the formulation of unwanted emission requirements for FR1

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS transmission bandwidth: set of resource blocks located within the BS channel bandwidth which may be used for transmitting by the BS

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

maximum carrier output power: mean power level measured per carrier at the indicated interface, during the transmitter ON period in a specified reference condition

maximum total output power: mean power level measured within the operating band at the indicated interface, during the transmitter ON period in a specified reference condition

measurement bandwidth: RF bandwidth in which an emission level is specified

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in tables 5.2-1 and 5.2-2.

rated carrier output power: mean power level associated with a particular carrier the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

transmission bandwidth: RF Bandwidth of an instantaneous transmission from a UE or BS, measured in resource block units

transmitter OFF period: time period during which the BS transmitter is not allowed to transmit

transmitter ON period: time period during which the BS transmitter is transmitting data and/or reference symbols

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

BWChannel	BS channel bandwidth

BWConfig	Transmission bandwidth, where BWConfig = NRB x SCS x 12

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔfBE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔFGlobal	Global frequency raster granularity

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔFRaster	Channel raster granularity

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

Ffilter	Filter centre frequency

f_offset	Separation between the channel edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	A-IoT Absolute Radio Frequency Channel Number (AIoT-ARFCN)

NREF-Offs	Offset used for calculating NREF

Pmax,c,AC	Maximum carrier output power measured per antenna connector

Prated,c,AC	The rated carrier output power per antenna connector

Prated,t,AC	The rated total output power declared at the antenna connector

PREFSENS	Conducted Reference Sensitivity power level

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

2SB	Double sideband

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AWGN	Additive White Gaussian Noise

A-IoT	Ambient IoT

A-IoT RAN	Ambient IoT Radio Access Network

BPSK	Binary phase-shift keying

BS	Base Station

BW	Bandwidth

CW	Carrier-wave

CW2D	Carrier-wave, or carrier-wave node, to device

D2R	Device to reader

FR	Frequency Range

FRC	Fixed Reference Channel

OOK	On-off keying

R2D	Reader to device

REFSENS	Reference Sensitivity

RF	Radio frequency

SCS	Sub-Carrier Spacing

SFO	Sampling-frequency offset

UEM	Unwanted Emissions Mask

4	General

4.1	Relationship between minimum requirements and test requirements

Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification TS 38.195.

The minimum requirements given in this specification make no allowance for measurement uncertainty. The test specifications TS 38.195 define test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in recommendation ITU-R M.1545.

4.2	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

5	Operating bands and channel arrangement

5.1	General

The channel arrangements presented in this clause are based on the operating bands and BS channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and BS channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which A-IoT can operate according to the present version of the specification are identified as described in table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |

5.2	Operating bands

A-IoT is designed to operate in the operating bands defined in table 5.2-1

Table 5.2-1: A-IoT operating bands in FR1

| A-IoT operating band | Uplink (D2R&CW) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (R2D) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| n8 | 880 – 915 | 925 – 960 | FDD |

5.3	BS channel bandwidth

5.3.1	General

5.3.1	R2D Channel bandwidth

5.3.1.1	General

The R2D channel bandwidth supports a single reader RF carrier in R2D link at the reader.

The relationship between the R2D channel bandwidth, the guardband and the transmission bandwidth is shown in figure 5.3.1.1-1.

Figure 5.3.1.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one reader channel

5.3.1.2	R2D Transmission bandwidth

The transmission bandwidth NRB for each reader channel bandwidth and subcarrier spacing is specified in table 5.3.1.2-1.

Table 5.3.1.2-1: R2D Transmission bandwidth configuration NRB for FR1

| R2D Channel Bandwidth | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
| SCS (kHz) | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	All BS Tx and device Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.1.2-1.

5.3.1.3	Minimum guardband and R2D transmission bandwidth configuration

The minimum guardband for each reader channel bandwidth and SCS is specified in table 5.3.3-1.

Table 5.3.1.3-1: Minimum guardband (kHz) (FR1)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

The number of RBs configured in any reader channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.1.3-1: reader PRB utilization

5.3.1.4	RB alignment

For each reader channel bandwidth, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

5.3.1.5	R2D channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1. The transmission bandwidth configuration in table 5.3.2-1 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for the Tx path.

Table 5.3.5-1: BS channel bandwidths and SCS per operating band

| A-IoT Band | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n8 | 15 | 200 | 400 | 600 | 800 |

5.3.2	D2R Channel bandwidth

5.3.2.1	General

The D2R channel bandwidth supports a single NR RF carrier in the uplink at the BS. From a BS perspective, different device channel bandwidths may be supported within the same spectrum for transmitting to and backscattering from devices connected to the BS.

5.3.2.2	Minimum guardband

The minimum guardband for each D2R channel bandwidth at BS side is specified as 10% D2R channel bandwidth at BS side.

5.3.2.3	D2R channel bandwidth per operating band

The requirements in this specification only apply to the operating band n8 shown in in table 5.3.2.3-1 for BS.

Table 5.3.2.3-1: BS D2R channel bandwidth

| BS D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Norminal D2R transmission Bandwidth without SFO (kHz) | Norminal Small frequency shift without SFO(kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 19 | 28 | 46 | 83 | 156 | 303 | 596 | 1183 |  |
| 30 |  | 37 | 55 | 92 | 165 | 312 | 605 | 1192 |  |
| 60 |  |  | 74 | 110 | 184 | 330 | 624 | 1210 |  |
| 120 |  |  |  | 147 | 220 | 367 | 660 | 1247 |  |
| 240 |  |  |  |  | 294 | 440 | 734 | 1320 |  |
| 480 |  |  |  |  |  | 587 | 880 | 1467 |  |
| 960 |  |  |  |  |  |  | 1174 | 1760 |  |
| 2880 |  |  |  |  |  |  |  |  | 3520 |

5.4	Channel arrangement

5.4.1	Channel raster

5.4.1.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels and other elements. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.1.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.1.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The mapping between the channel raster and corresponding resource element is given in clause 5.4.1.2. The applicable entries for each operating band are defined in clause 5.4.1.3.

5.4.1.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.1.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the BS.

Table 5.4.1.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

k,  and NRB are as defined in TS 38.211 [9].

5.4.1.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each A-IOT operating band are given through the applicable NR-ARFCN in table 5.4.1.3-1, using the channel raster to resource element mapping in clause 5.4.1.2.

Channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.1.3-1 is given as <2>.

Table 5.4.1.3-1: Applicable NR-ARFCN per operating band for enhanced channel raster

| A-IoT operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n8 | 10 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

5.4.2	Channel raster

5.4.1.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels and other elements. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.1.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.1.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The mapping between the channel raster and corresponding resource element is given in clause 5.4.1.2. The applicable entries for each operating band are defined in clause 5.4.1.3.

5.4.1.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.1.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the BS.

Table 5.4.1.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

k,  and NRB are as defined in TS 38.211 [9].

5.4.1.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each A-IOT operating band are given through the applicable NR-ARFCN in table 5.4.1.3-1, using the channel raster to resource element mapping in clause 5.4.1.2.

Channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.1.3-1 is given as <2>.

Table 5.4.1.3-1: Applicable NR-ARFCN per operating band for enhanced channel raster

| A-IoT operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n8 | 10 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

6	A-IoT BS transmitter characteristics

6.1	General

Unless otherwise stated, the conducted transmitter characteristics are specified at the antenna connector for BS type 1-C. A-IoT BS transmitter characteristics refer to that for BS type 1-C in clause 6 in TS38.104.

6.2	Base station output power

### 6.2.1	General

The A-IoT BS conducted output power requirement is at antenna connector for A-IoT BS type 1-C.

The rated carrier output power of the A-IoT BS type 1-C shall be less than or equal to 38dBm.

6.3	Transmit ON/OFF power

6.3.1	Transmitter OFF power

6.3.1.1 General

Transmitter OFF power for A-IoT BS is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6.3.1.2 Minimum requirement for A-IoT BS type 1-C

For A-IoT BS type 1-C, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

6.3.2	Transmitter transient period

6.3.2.1 General

The transmitter transient period for A-IoT BS is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.4.2.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period

For A-IoT BS type 1-C, this requirement shall be applied at the antenna connector supporting transmission in the operating band.

6.3.2.2 Minimum requirement for A-IoT BS type 1-C

For A-IoT BS type 1-C, the transmitter transient period shall be shorter than the values listed in the minimum requirement table 6.4.2.2-1.

Table 6.3.2.2-1: Minimum requirement for the transmitter transient period for A-IoT BS type 1-C

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

6.4	Transmitted signal quality

6.4.1	Frequency error

#### 6.4.1.1	General

The requirements in clause 6.4.1 apply to the transmitter ON period.

Frequency error is the measure of the difference between the actual BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

#### 6.4.1.2	Minimum requirement for BS type 1-C

For BS type 1-C, the modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 6.4.1.2-1 observed over 1 ms.

Table 6.4.1.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Medium Range BS | ±0.1 ppm |

6.4.2	Modulation quality

Based on TS38.291, R2D signal includes SIP (Start indicator part), CAP (Clock acquisition part), PRDCH, the R2D postamble and padding if needed.

An is measured peak high level for the nth chip, in units of V/m or A/m

Bn is measured peak low level for the nth chip, in units of V/m or A/m

Anavg is the measured average high level for the nth chip during1/2 duration above 90%An, in units of V/m or A/m

Bnavg is the measured average low level for the nth chip during 1/2 duration below 10%An, in units of V/m or A/m

Modulation depth:

Modulation depth is defined with equation below and modulation depth for OOK chip 0/1 shall meet the requirements in Table 6.4.2-1.

Modulation depth =(Anavg-Bnavg)/Anavg

The envelope of electric filed strength for OOK chip 0 RF pulse shall comply the timing mask in Figure 6.4.2-1 and meet the requirements in Table 6.4.2-1. The envelope of electric filed strength for OOK Bit 0 shall decrease monotonically from 90% to less than 10 % of initial value Einitial during t1. The envelope of electric filed strength for OOK Bit 0 shall increase monotonically from 10% to less than 90 % of its initial value Einitial during t3. The initial value Einitial is defined as the field strength difference between Anavg and Bnavg.

RF Envelop Rise Time:

The Tr,10-90 measures the rise of the OOK bit 0 pulse and starts when envelop rises to 10% level of the initial value Einitial and ends when the envelop rises to 90% of the initial value Einitial.

RF Envelop Fall Time:

The Tf,10-90 measures the fall time of the OOK chip 0 pulse and starts when envelop falls to 90% level of the initial value Einitial and ends when the envelop falls to 10% of the initial value Einitial.

Tf,10-90 starts when the envelop drops to the 90% level of the initial value Einitial and ends when envelop rise to 10% level of the initial value Einitial.

Ripple:

Ripple_high (%) = ((An − Anavg) / (Anag-Bnavg)) × 100%

Ripple_low (%) = ((Bn − Bnavg) / (Anavg-Bnavg)) × 100%

In case of an overshoot or undershoot the field shall remain within +/- Ripple_high % of Einitial for OOK chip 1 and +/-Ripple_low % of Einitial for OOK chip 0.

Pulsewidth

The PW measures the time between envelop falling edge at 50% of the initial value Einitial and envelop rinsing edge at 50% of the initial value Einitial.

Table 6.4.2-1: A-IoT BS RF envelope parameters

| R2D Chip duration：Tc | Parameter | Symbol | Value | Units |
| --- | --- | --- | --- | --- |
| Tc=103M*15(us) M∈ {2,6,12,24} | Modulation Depth | (A–B)/A | 80 | % |
|  | RF Envelope Ripple | Ripple_high Ripple_low | <=±15 | % |
|  | RF Envelop Rise Time | Tr,10-90 | <=0.66Tc | µs |
|  | RF Envelop Fall Time | Tf,10-90 | <=0.66Tc | µs |
|  | RF Pulsewidth | PW | <=1.3 Tc | µs |

Figure 6.4.2-1: Timing mask for OOK chip 0 pulse

6.5	Unwanted emissions

6.5.1	General

Unwanted emissions consist of out-of-band emissions and spurious emissions according to ITU definitions [2]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the BS R2D channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The out-of-band emissions requirement for the BS transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE).

The maximum offset of the operating band unwanted emissions mask from the operating band edge is 10MHz. The Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges 10MHz above and 10MHz below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

6.5.2	Occupied bandwidth

6.5.2.1	General

The occupied bandwidth requirement shall apply during transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the occupied bandwidth.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

6.5.2.2	Minimum requirement for BS type 1-C

The occupied bandwidth for each NR carrier shall be less than the BS R2D channel bandwidth.

6.5.3	Adjacent Channel Leakage Power Ratio

6.5.3.1	General

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The requirements shall apply outside the Base Station RF Bandwidth or Radio Bandwidth whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer’s specification.

The requirement shall apply during the transmitter ON period.

6.5.3.2	Limits and Basic limits

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired spectrum, the ACLR shall be higher than the value specified in table 6.5.3.2-1 in band n8.

Table 6.5.3.2-1: Base station ACLR limit

| BS R2D channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted (kHz) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200 | 300 | A-IoT of same BW | Square (180 kHz) | 40dB |
|  | 500 | A-IoT of same BW | Square (180 kHz) | 45dB |
| 400 | 500 | A-IoT of same BW | Square (360 kHz) | 40dB |
|  | 900 | A-IoT of same BW | Square (360 kHz) | 45dB |
| 600 | 700 | A-IoT of same BW | Square (540 kHz) | 40dB |
|  | 1300 | A-IoT of same BW | Square (540 kHz) | 45dB |
| 800 | 900 | A-IoT of same BW | Square (720 kHz) | 40dB |
|  | 1700 | A-IoT of same BW | Square (720 kHz) | 45dB |

6.5.3.3	Minimum requirement for BS type 1-C

The ACLR  limits in table 6.5.3.2-1  shall apply for each antenna connector.

6.5.4	Operating band unwanted emissions

6.5.4.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) limits in FR1 are defined from 10MHz below the lowest frequency of each supported downlink operating band up to 10MHz above the highest frequency of each supported downlink operating band. The values of 10MHz are defined in table 6.5.1-1 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification.

Basic limits are specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For Medium Range BS, the requirements in clause 6.5.4.2.3 shall apply.

6.5.4.2	Basic limits

6.5.4.2.3	Basic limits for Medium Range BS

For A-IoT medium range BS (maximum output power 31 < Prated,c  38 dBm), emissions shall not exceed the maximum levels specified in Table 6.5.4.2.3-1.

Table 6.5.4.2.3-1: A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm

| BS R2D channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 200kHz | 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
|  | 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
|  | 0.15 MHz f < 0.6 MHz | 0.165MHz f_offset < 0.615MHz |  | 30 kHz |
|  | 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
|  | 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Prated,c - 52 dB | 1 MHz |
|  | 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Prated,c - 52 dB, -15dBm) | 1 MHz |
|  | 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | Prated,c - 56 dB | 1 MHz |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -40dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 51dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c - 56dB | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 40dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 53dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | Prated,c - 58dB | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 40dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 54dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | Prated,c - 59dB | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

For A-IoT (maximum output power Prated,c  31 dBm), emissions shall not exceed the maximum levels specified in Tables 6.5.4.2.3-2.

Table 6.5.4.2.3-2: A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm

| BS R2D channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 200kHz | 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
|  | 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
|  | 0.15 MHz f < 0.6 MHz | 0.165MHz f_offset < 0.615MHz |  | 30 kHz |
|  | 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
|  | 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -21 dBm | 1 MHz |
|  | 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | -25 dBm | 1 MHz |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | -9dBm - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | -20dBm- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -25dBm | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | -9dBm - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | -22dBm - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | -27dBm | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | -9dBm - 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -23dBm - 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | -28dBm | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100kHz |

6.5.4.3	Minimum requirements for BS type 1-C

The operating band unwanted emissions for BS type 1-C for each antenna connector shall be below the applicable basic limits defined in clause 6.5.4.2.

6.5.5	Transmitter spurious emissions

6.5.5.1	General

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10MHz below the lowest frequency of each supported downlink operating band, up to 10MHz above the highest frequency of each supported downlink operating band.

Unless otherwise stated, all requirements are measured as mean power (RMS).

6.5.5.2	Basic limits

6.5.5.2.1	General transmitter spurious emissions requirements

The basic limits of either table 6.5.5.2.1-1 (Category A limits) or table 6.5.5. 2.1-2 (Category B limits) shall apply.

Table 6.5.5.2.1-1: General BS transmitter spurious emission limits in FR1, Category A

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -13 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz 12.75 GHz |  | 1 MHz |

Table 6.5.5.2.1-2: General BS transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 12.75 GHz | -30 dBm | 1 MHz |

6.5.5.2.3	Additional spurious emissions requirements

These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.) as listed below.

The spurious emission basic limits are provided in table 6.5.5.2.3 -1 for a BS where requirements for co-existence with the system listed in the first column apply.

Table 6.5.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for A-IoT to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
|  | 1805 – 1880 MHz | -47 dBm | 100 kHz | This requirement does not apply to BS operating in band n3. |
| DCS1800 | 1710 – 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 1930 – 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to BS operating in band n2, n25 or band n70. |
| PCS1900 | 1850 – 1910 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n2 or n25 since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 869 – 894 MHz | -57 dBm | 100 kHz | This requirement does not apply to BS operating in band n5 or n26. |
| GSM850 or CDMA850 | 824 – 849 MHz | -61 dBm | 100 kHz | This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band I or | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65 |
| E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band II or | 1930 – 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2 or n70. |
| E-UTRA Band 2 or NR Band n2 | 1850 – 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band III or | 1805 – 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
| E-UTRA Band 3 or NR Band n3 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band IV or | 2110 – 2155 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n66 |
| E-UTRA Band 4 | 1710 – 1755 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band V or | 869 – 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n5 or n26. |
| E-UTRA Band 5 or NR Band n5 | 824 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band VI, XIX or | 860 – 890 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n18. |
| E-UTRA Band 6, 18, 19 or NR Band n18 | 815 – 830 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n18, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 830 – 845 MHz | -49 dBm | 1 MHz |  |
| UTRA FDD Band VII or | 2620 – 2690 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n7. |
| E-UTRA Band 7 or NR Band n7 | 2500 – 2570 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n7, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band IX or | 1844.9 – 1879.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
| E-UTRA Band 9 | 1749.9 – 1784.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band X or | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n66 |
| E-UTRA Band 10 | 1710 – 1770 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band XI or XXI or | 1475.9 – 1510.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. |
| E-UTRA Band 11 or 21 | 1427.9 – 1447.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n51, n74, n75, n76, n91, n92, n93, or n109. |
|  | 1447.9 – 1462.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. |
| UTRA FDD Band XII or | 729 – 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85. |
| E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.5.5.2.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |
| UTRA FDD Band XIII or | 746 – 756 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n13. |
| E-UTRA Band 13 or NR Band n13 | 777 – 787 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n13, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band XIV or | 758 – 768 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n14. |
| E-UTRA Band 14 or NR band n14 | 788 – 798 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n14, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 734 – 746 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 17 | 704 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |
| UTRA FDD Band XX or | 791 – 821 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n20 or n28. |
| E-UTRA Band 20 or NR Band n20 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band XXII | 3510 – 3590 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n48, n77 or n78. |
| or E-UTRA Band 22 | 3410 – 3490 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n77 or n78. |
|  | 1525 – 1559 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n24. |
| E-UTRA Band 24 or NR Band n24 | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n24, since it is already covered by the requirement in clause 6.5.5.2.2. |
| UTRA FDD Band XXV or | 1930 – 1995 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70. |
| E-UTRA Band 25 or NR band n25 | 1850 – 1915 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n25 since it is already covered by the requirement in clause 6.5.5.2.2. For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.5.5.2.2. |
| UTRA FDD Band XXVI or | 859 – 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n5 or n26. |
| E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n26 since it is already covered by the requirement in clause 6.5.5.2.2. For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.5.5.2.2. |
|  | 852 – 869 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5. |
| E-UTRA Band 27 | 807 – 824 MHz | -49 dBm | 1 MHz | This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 5). |
| E-UTRA Band 28 or | 758 – 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, n67 or n28. |
| NR Band n28 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.5.5.2.2. For BS operating in band n67, it applies for 703 MHz to 736 MHz. |
| E-UTRA Band 29 or NR Band n29 | 717 – 728 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n29 or n85 |
| E-UTRA Band 30 or | 2350 – 2360 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n30 |
| NR Band n30 | 2305 – 2315 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n30, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 31 or NR Band n31 | 462.5 – 467.5 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n31 or n72. |
|  | 452.5 – 457.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n31, since it is already covered by the requirement in clause 6.5.5.2.2. This requirement does not apply to BS operating in band n72. |
| UTRA FDD band XXXII or E-UTRA band 32 | 1452 – 1496 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109 |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -52 dBm | 1 MHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 – 2025 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n34. |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -52 dBm | 1 MHz |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n2 or n25. |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -52 dBm | 1 MHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n39. |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n30 or n40. |
| E-UTRA Band 41 or NR Band n41, n90 | 2496 – 2690 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n41, n53 or [n90]. |
| E-UTRA Band 42 | 3400 – 3600 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. |
| E-UTRA Band 43 | 3600 – 3800 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. |
| E-UTRA Band 44 | 703 – 803 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n28. |
| E-UTRA Band 45 | 1447 – 1467 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n46, n96 or n102. |
| E-UTRA Band 47 | 5855 – 5925 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93, n94 or n109. |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93, n94 or n109. |
| E-UTRA Band 53 or NR Band n53 | 2483.5 - 2495 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n41, n53 or n90. |
| E-UTRA Band 54 or NR Band n54 | 1670 – 1675 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n54 |
| E-UTRA Band 65 or | 2110 – 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65. |
| NR Band n65 | 1920 – 2010 MHz | -49 dBm | 1 MHz | For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.5.5.2.2. This requirement does not apply to BS operating in band n65, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 66 or | 2110 – 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n66. |
| NR Band n66 | 1710 – 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 67 or NR Band n67 | 738 – 758 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n28 or n67. |
| E-UTRA Band 68 | 753 -783 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n28. |
|  | 698-728 MHz | -49 dBm | 1 MHz | For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.5.5.2.2. |
| E-UTRA Band 69 | 2570 – 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n38. |
| E-UTRA Band 70 or | 1995 – 2020 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, n25 or n70 |
| NR Band n70 | 1695 – 1710 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n70, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 71 or | 617 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105 |
| NR Band n71 | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 72 or NR Band n72 | 461 – 466 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n31 or n72. |
|  | 451 – 456 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n72, since it is already covered by the requirement in clause 6.5.5.2.2. This requirement does not apply to BS operating in band n31. |
| E-UTRA Band 74 | 1475 – 1518 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n50, n74, n75, n92, n94 or n109. |
| or NR Band n74 | 1427 – 1470 MHz | -49 dBm | 1MHz | This requirement does not apply to BS operating in band n50, n51, n74, n75, n76, n91, n92, n93 or n94 or n109. |
| E-UTRA Band 75 or NR Band n75 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94 or n109. |
| E-UTRA Band 76 or NR Band n76 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93, n94 or n109. |
| NR Band n77 | 3.3 – 4.2 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 |
| NR Band n78 | 3.3 – 3.8 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n79 |
| NR Band n80 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.5.5.2.2. |
| NR Band n82 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.5.5.2.2. |
| NR Band n83 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.5.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. |
| NR Band n84 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 85 or NR Band n85 | 728 – 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |
|  | 698 – 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.5.5.2.2. |
| NR Band n86 | 1710 – 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.5.5.2.2. |
| NR Band n89 | 824 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76 or n109 |
| NR Band n91 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76 or n109. |
| NR Band n92 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76 or n109. |
|  | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76 or n109. |
| NR Band n95 | 2010 – 2025 MHz | -52 dBm | 1 MHz |  |
| NR Band n96 | 5925 – 7125 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n46, n96, n102 or n104. |
| NR Band n97 | 2300 – 2400MHz | -52 dBm | 1 MHz |  |
| NR Band n98 | 1880 – 1920MHz | -52 dBm | 1 MHz |  |
| NR Band n99 | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n24, since it is already covered by the requirement in clause 6.5.5.2.2. |
|  | 874.4 – 880 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n100, since it is already covered by the requirement in clause 6.5.5.2.2. |
| NR band n101 | 1900 – 1910 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n101. |
| NR Band n102 | 5925 – 6425 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n46, n96, n102 or n104. |
| E-UTRA Band 103 | 757 – 758 MHz | -52 dBm | 1 MHz |  |
|  | 787 – 788 MHz | -49 dBm | 1 MHz |  |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n96, n102 or n104 |
| NR Band n105 | 612 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n71 or n105 |
|  | 663 – 703 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in n105, since it is already covered by the requirement in clause 6.5.5.2.2. |
| E-UTRA Band 106 or NR Band n106 | 935 - 940 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n106 |
|  | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5 or n26. This requirement does not apply to BS operating in n106, since it is already covered by the requirement in clause 6.5.5.2.2. |
| NR Band n109 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93, n94 or n109 |
|  | 703 –733MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.5.5.2.2. |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.5.5.2.3 -1 do not apply for the 10MHz frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.5.5.2.3 -1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

Table 6.5.5.2.3-3: Void

6.5.5.2.4	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The basic limits are in table 6.5.5.2.4-1 for a BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class.

Table 6.5.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Type of co-located BS | Frequency range for | Basic limits | Measurement | Note |
| --- | --- | --- | --- | --- |
|  | co-location requirement | MR BS | bandwidth |  |
| DCS1800 | 1710 – 1785 MHz | -91 dBm | 100 kHz |  |
| PCS1900 | 1850 – 1910 MHz | -91 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 – 849 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 – 1910 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 – 1785 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 – 1755 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 – 849 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 – 845 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 – 2570 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 – 1784.9 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 – 1770 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 –1447.9 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n50, n75, n91, n92, n93 or n94 |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13 | 777 – 787 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 788 – 798 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 – 716 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 18 or NR Band n18 | 815 – 830 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 – 862 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n50, n75, n92 or n94 |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n48, n77 or n78 |
| E-UTRA Band 24 or NR Band n24 | 1626.5 – 1660.5 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1850 – 1915 MHz | -91 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 – 824 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 30 or NR Band n30 | 2305 – 2315 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 31 or NR Band n31 | 452.5 – 457.5 MHz | -91 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 – 1920 MHz | -91 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 – 2025 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -91 dBm | 100 kHz |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n2 or band n25 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -91 dBm | 100 kHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n30 or n40. |
| E-UTRA Band 41 or NR Band n41, n90 | 2496 – 2690 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n41, n53 or [n90] |
| E-UTRA Band 42 | 3400 – 3600 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n48, n77 or n78 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n48, n77 or n78 |
| E-UTRA Band 44 | 703 – 803 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n28 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n46, n96 or n102 |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n48, n77 or n78 |
| E-UTRA Band 50 or NR Band n50 | 1432 – 1517 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n51, n74, n75, n91, n92, n93 or n94 |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | 100 kHz | This is not applicable to BS operating in Band n50, n74, n75, n76, n91, n92, n93 or n94 |
| E-UTRA Band 53 or NR Band n53 | 2483.5 – 2495 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n41, n53 or n90 |
| E-UTRA Band 54 or NR Band n54 | 1670 – 1675 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n54 |
| E-UTRA Band 65 or NR Band n65 | 1920 – 2010 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 66 or NR Band n66 | 1710 – 1780 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 70 or NR Band n70 | 1695 – 1710 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 72 or NR Band n72 | 451 – 456 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 74 or NR Band n74 | 1427 – 1470 MHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n50, n51, n91, n92, n93 or n94 |
| NR Band n77 | 3.3 – 4.2 GHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n48, n77 or n78 |
| NR Band n78 | 3.3 – 3.8 GHz | -91 dBm | 100 kHz | This is not applicable to BS operating in Band n48, n77 or n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -91 dBm | 100 kHz |  |
| NR Band n80 | 1710 – 1785 MHz | -91 dBm | 100 kHz |  |
| NR Band n82 | 832 – 862 MHz | -91 dBm | 100 kHz |  |
| NR Band n83 | 703 – 748 MHz | -91 dBm | 100 kHz |  |
| NR Band n84 | 1920 – 1980 MHz | -91 dBm | 100 kHz |  |
| E-UTRA Band 85 or NR Band 85 | 698 – 716 MHz | -91 dBm | 100 kHz |  |
| NR Band n86 | 1710 – 1780 MHz | -91 dBm | 100 kHz |  |
| NR Band n89 | 824 – 849 MHz | -91 dBm | 100 kHz |  |
| NR Band n91 | 832 – 862 MHz | N/A | 100 kHz |  |
| NR Band n92 | 832 – 862 MHz | -91 dBm | 100 kHz |  |
| NR Band n95 | 2010 – 2025 MHz | -91 dBm | 100 kHz |  |
| NR Band n96 | 5925 – 7125 MHz | -90 dBm | 100 kHz | This is not applicable to BS operating in Band n46, n96, n102 or n104 |
| NR Band n97 | 2300 – 2400MHz | -91 dBm | 100 kHz |  |
| NR Band n98 | 1880 – 1920MHz | -91 dBm | 100 kHz |  |
| NR Band n99 | 1626.5 – 1660.5 MHz | -91 dBm | 100 kHz |  |
| NR Band n100 | 874.4 – 880 MHz | NA | 100 kHz |  |
| NR Band n101 | 1900 – 1910 MHz | NA | 100 kHz |  |
| NR Band n102 | 5925 – 6425 MHz | -90 dBm | 100 kHz | This is not applicable to BS operating in Band n46, n96, n102 or n104 |
| E-UTRA Band 103 | 787 – 788 MHz | -91 dBm | 100 kHz |  |
| NR Band n104 | 6425 – 7125 MHz | -90 dBm | 100 kHz | This requirement does not apply to BS operating in Band n96, n102 or n104. |
| NR Band n105 | 663 – 703 MHz | -91 dBm | 100 kHz |  |
| NR Band n109 | 703 – 733 MHz | -91 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.5.5.2.4-1 do not apply for the frequency range extending 10MHz immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4].

NOTE 2:	Table 6.5.5.2.4-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

6.5.5.3	Minimum requirements for BS type 1-C

The Tx spurious emissions for BS type 1-C for each antenna connector shall not exceed the basic limits specified in clause 6.5.5.2.

7	A-IoT BS receiver characteristics

7.1	General

Conducted receiver characteristics are specified at the antenna connector for BS type 1-C , with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 7:

-	Requirements apply during the BS receive period.

-	Reference requirements defined for the conducted receiver characteristics do not assume HARQ retransmissions.

NOTE 1:	In normal operating condition, A-IoT BS is configured as HD-FDD operation.

7.2	Reference sensitivity level

7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for BS type 1-C at which a BLER requirement shall be met for a specified reference measurement channel.

7.2.2	Minimum requirements for BS type 1-C

The BLER shall be less than or equal to 10% of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 for A-IoT Medium range BS.

Table 7.2.2-1: A-IoT Medium range BS reference sensitivity levels

| BS D2R channel bandwidth (KHz) | DSB (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | A-FR1-A1-1 | -95.2 |
|  |  | A-FR1-A1-2 | -92.2 |
| 3520 | 2880 | A-FR1-A1-3 | -72.4 |
|  |  | A-FR1-A1-4 | -69.4 |
| NOTE: Reference sensitivity power level is defined based on the CW power at the BS antenna connector as -38dBm without the cancellation of CW phase noise considered. |  |  |  |

7.4	In-band selectivity and blocking

7.4.1	Adjacent Channel Selectivity

#### 7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 7.4.1.2	Minimum requirement for BS type 1-C

The MDR performance shall be [1%] of the reference measurement channel.

The wanted and the interfering signal coupled to the BS type 1-C antenna connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS D2R channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.4.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table X |  |  |  |  |

- Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | ±100 | 3 MHz DFT-s-OFDM NR signal |

7.4.2	In-band blocking

#### 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.4.2.2	Minimum requirement for BS type 1-C

The MDR performance shall be [1%] of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3 for general blocking and narrowband blocking requirements. The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

The in-band blocking requirement shall apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. The ΔfOOB for BS type 1-C is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.4.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low ≤ 200 MHz | 20 |
| BS type 1-C | 200 MHz < FUL,high – FUL,low ≤ 900 MHz | 60 |

- Table 7.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + x dB | -38 | ±7.5 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + x dB | -38 | ±4760 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables [7.2.2-1] NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

7.5	Out-of-band blocking

### 7.5.1	General

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

### 7.5.2	Minimum requirement for BS type 1-C

the MDR performance shall be [1%] of the reference measurement channel, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in table 7.5.2-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

- Table 7.5.2-1: Out-of-band blocking performance requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -15 | CW carrier |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in Table 7.2.2-1. |  |  |

7.6	Receiver spurious emissions

### 7.6.1	General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the antenna connector (for BS type 1-C). The requirements apply to all BS with separate RX and TX antenna connectors.

NOTE:	In this case for FDD operation the test is performed when both TX and RX are ON, with the TX antenna connectors terminated.

For antenna connectors supporting both RX and TX in FDD, the RX spurious emissions requirements are superseded by the TX spurious emissions requirements, as specified in clause 6.6.5.

### 7.6.2	Basic limits

The receiver spurious emissions basic limits are provided in table 7.6.2-1.

- Table 7.6.2-1: General BS receiver spurious emissions limits

| Spurious frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the UL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. For multi-band connectors, the exclusion applies for all supported operating bands. NOTE 5: Applies for Band for which the upper frequency edge of the UL operating band is greater than 5.2 GHz. |  |  |  |

### 7.6.3	Minimum requirement for BS type 1-C

The RX spurious emissions requirements for BS type 1-C are that for each antenna connector, the power of emissions shall not exceed basic limits specified in table 7.6.2-1.

7.7	Receiver intermodulation

7.7.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

### 7.7.2	Minimum requirement for BS type 1-C

The MDR performance shall be [1%] of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS type 1-C antenna connector, with the conditions specified in tables 7.7.2-1 and 7.7.2-2 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the NR interfering signal in table 7.7.2-2 or 7.7.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.7.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB* | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5MHz E-UTRA signal, 1 RB** |
| 3520 | PREFSENS + 6dB* | -53 | ±270 | CW |
|  |  | -53 | ±780 | 3.0 MHz E-UTRA signal, 1 RB** |
| Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.1-5c. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

8	A-IoT CW transmitter characteristics

8.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the CW node with a single or multiple transmit antenna(s). The CW waveform for D2R backscattering is a single-tone unmodulated sinusoid.

NOTE: CW transmission and A-IoT BS Downlink (DL) data transmission are non-concurrent. For FDD bands, the CW is transmitted in Uplink (UL) operating band.

8.2	CW Output power

### 8.2.1	General

Output power of the CW node is the mean power of the single-tone signal delivered to a load with resistance equal to the nominal load impedance of the transmitter.

Rated total output power (Prated) of the CW node is the mean power of the single-tone signal that the manufacturer has declared to be available at the antenna connector during the transmitter ON period. The rated output power, Prated,, of the CW node shall be than or equal to +33 dBm.

Maximum output power (Pmax) of the base station is the mean power level of the single-tone signal measured at the antenna connector during the transmitter ON period in a specified reference condition.

### 8.2.1	Minimum requirement

In normal conditions, the CW maximum output power, Pmax, shall remain within +2 dB and -2 dB of the rated output power, Prated, declared by the manufacturer.

In extreme conditions, the base station maximum output power, Pmax, shall remain within +2.5 dB and -2.5 dB of the rated output power, Prated, declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

8.3	Frequency error

### 8.3.1	General

The requirements in clause 8.3 apply to the transmitter ON period.

Frequency error is the measure of the difference between the actual carrier wave transmit frequency and the assigned frequency. This requirement shall be applied at the antenna connector supporting transmission in the operating band.

### 8.3.2	Minimum requirement

The frequency of carrier wave measurements shall be accurate to within ±0.1 ppm observed over a period of 1 ms compared to the carrier frequency declared by the manufacturer.

## 8.4	Transmit OFF power

### 8.4.1	General

Transmit OFF power is defined as the mean power when the transmitter is OFF.

### 8.4.1	Minimum requirement

The transmit OFF power shall not exceed -50 dBm/MHz

8.5	Unwanted emission

### 8.5.1	General

Unwanted emissions consist of phase noise, operating band unwanted emissions and spurious emissions.

Unless otherwise stated, all requirements are measured as mean power (RMS).

### 8.5.2	Phase noise

#### 8.5.2.1	General

The phase noise is the unwanted emissions outside the centre frequency of carrier wave resulting from random fluctuations in the phase of signal in the transmitter but excluding spurious emissions. The phase noise of the CW applies to ±7.5 kHz and ±120 kHz frequency offset (Δf) from centre frequency of carrier wave.

For the CW equipment declared capable of performing phase noise cancellation, the requirement in 8.5.2.2 is not applied.

#### 8.5.2.2	Minimum requirement

The phase noise power of CW transmitter shall not exceed the levels specified in Table 8.5.2.2-1

Table 8.5.2.2-1: CW phase noise emission limit

| Δf (kHz) | Phase noise emission limit (dBc/Hz) |
| --- | --- |
| ± 7.5 | -97 |
| ± 120 | -102 |

### 8.5.3	Operating band unwanted emissions

#### 8.5.3.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) limits in FR1 are defined from 10 MHz below the lowest frequency of each supported uplink operating band up to 10 MHz above the highest frequency of each supported uplink operating band.

Emissions shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the assigned transmission frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

#### 8.5.3.2	Minimum requirement

The operating band unwanted emissions shall not exceed the maximum levels specified in the Table 8.5.2.2-1. The spectrum emission limit between each Δf is linearly interpolated.

Table 8.5.3.2-1: CW node Operating band unwanted emissions

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -18 | 30 kHz |
| 250 | -20 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~10000 | -23 | 1 MHz |

### 8.5.4	Transmitter spurious emissions

#### 8.5.4.1	General

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10MHz below the lowest frequency of each supported uplink operating band, up to 10 MHz above the highest frequency of each supported uplink operating band.

#### 8.5.4.2	Minimum requirement

The power of any spurious emission shall not exceed the limits in Table 8.5.3.2-1.

Table 8.5.4.2-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |

Annex A (informative):
D2R channel bandwidth

The following describes the equation to derive BS D2R channel bandwidth.

For BS D2R CBW:

D2R CBW for BS (kHz)

= ceiling ((2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9)

=ceiling ( (2+2R)/Tb × (1+∣SFO∣)/0.9)

=ceiling ( (1+R)/ (Tc ×R) × (1+∣SFO∣)/0.9)

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band).

Annex B (normative):
Reference measurement channels

B.1	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking, (BPSK, OOK)

| Reference channel | A-FR1-A1-1 | A-FR1-A1-2 | A-FR1-A1-3 | A-FR1-A1-4 |
| --- | --- | --- | --- | --- |
| DSB (kHz) | 15 | 15 | 2880 | 2880 |
| Payload size (bits) | 96 | 96 | 96 | 96 |
| CRC (bits) | 16 | 16 | 16 | 16 |
| Preamble length | 31 | 31 | 31 | 31 |
| Midamble length | 31 | 31 | 31 | 31 |
| Midamble configuration I | 48 | 48 | 48 | 48 |
| FEC | 1/3 | 1/3 | 1/3 | 1/3 |
| Line code | Manchester | Manchester | Manchester | Manchester |
| Modulation | BPSK | OOK | BPSK | OOK |
| Waveform (CW) | unmodulated single tone | Unmodulated single tone | unmodulated single tone | unmodulated single tone |
| Sampling frequency（SFO） | between 0.01 and 0.1 | Between 0.01 and 0.1 | between 0.01 and 0.1 | Between 0.01 and 0.1 |
| Total symbols | 398 | 398 | 398 | 398 |

Annex I (Informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | RAN4#114bis | R4-2505228 |  |  |  | 3GPP_TS 38.194 skeleton | 0.0.1 |
| 2025-08 | RAN4#116 | R4-2511749 |  |  |  | Agreed TP in RAN4#116: R4-2511722 TP for TR 38.194 5.3 BS channel bandwidth and 5.4 Channel arrangement R4-2511723 TP for TR 38.191 section 3 Definitions, symbols and abbreviations R4-2511725 draft TP for TS 38.194 to introduce base station output power and transmit ON/OFF power R4-2511727 TP for TR 38.194 6.5 Unwanted emissions and 6.6 Transmitter intermodulation R4-2511728 draft TP to TS 38.194 on Transmitted signal quality R4-2511731 TP to TS38.194: REFSENS requirement for A-IoT BS and FRC R4-2511732 TP to TS38.194 : ACS , Inband blocking, OOB and Spurious R4-2511742 TP to TS 38.194 on CW frequency error and unwanted emssion R4-2511743 TP to 38.194 on general and CW output power | 0.0.2 |

Annex <D> (informative):
Bibliography

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <E> (informative):
Index

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Index is optional. If it exists, it shall immediately precede the Changes history annex.

Generate the index using MS Word's index field feature.

Annex <F> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |