3GPP RAN WG4 Meeting #116-bis																R4-2513030

Prague, CZ, 13th – 17th October 2025

Agenda item:	8.12

Source:	Apple Inc

Title:	Views on 6G OTA Testability

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

In RAN#108, the Rel-20 SID was approved that aimed at enabling the 6G radio access technology for the next-generation radio access system capable of addressing the full spectrum of future use cases, operational constraints, and deployment realities [1].

Using the same reference in [2] the main RF specific outcomes towards 6G will be useful to recollect considering the focus of this discussion is to trigger some ideas on the 6G OTA testability study in RAN WG4 -

- Spectrum efficiency and new bands: Emphasis on better usage of existing FR1/FR2 bands while studying the feasibility of new spectrum ranges (mainly the upper mid-band)

- Flexible numerology and frame structure: Interest in exploring leaner, scalable radio structures that can adapt to diverse services and applications and avoid some of the apparent complexity from 5G.

- Integrated Sensing and Communication (ISAC): Recognized as key 6G feature, introduce RF design that supports high-resolution sensing along with communications

- Ubiquitous coverage (TN+NTN): Ensure harmonized operation across terrestrial and non-terrestrial networks (Satellite, HAPS etc.).

- Interference Management and co-existence: A critical expected enhancement given denser deployments and multi-band operation, needs to be addressed early in RF studies.

One of the SID objectives was to define and introduce the testability methodology framework into the 3GPP specifications.

This paper shares a number of views on the OTA testability of potential RF requirements for devices capable of the 6G radio access technology specification.

# 2	Discussion

2.1 New Spectrum frequency ranges of interest of 6G OTA Study

The 6G RAT SID[1] captures several aspects in its objectives that are relevant to the testability observations. The frequency range for the 6G technology is addressed in this WI are:

- Full range of FR-1 (0.4 GHz to 7GHz)

- New spectrum frequency range between FR1 and FR2: 7GHz to 24 GHz

- FR2-1: 24.25 to 52.6 GHz

- Additionally, as illustrated in Figure-1 captured in TR 38.820, the new spectrum range (7-24 GHz) was studied based on sub-ranges[3]. A similar approach can be introduced to define the test methodology.

Figure-1- 6G New Frequency Range between FR1 and FR2 (7 GHz to 24GHz)

- Observation 1: 6G frequency range overlaps with existing 5G NR frequency range FR-1 and FR-2.

Proposal 1: RAN4 to reuse as much as possible the existing baselines of the NR frequency ranges FR-1 and FR-2 for RF requirements definition and testability of 6G frequency ranges.

Proposal 2: Identify the specific frequencies of interest (lower part of 7-24 GHz range seems to have obvious interest) to begin 6G OTA study for SISO/MIMO.

Proposal 3: To further discuss the testing methodology for the 6G new frequency spectrum range (~7GHz to 24 GHz), with a primary focus on Standalone (SA) mode over Non-Standalone (NSA) mode.

- General Approach for OTA Testing Methodology

OTA measurement is expected to be the baseline testing methodology for 6G frequency ranges.

Conductive testing remains applicable for 6G frequency range overlapping with 5G NR FR-1.

OTA Test system methodologies shall be considered depending on the frequency range.

Anechoic or reverberation test chamber for 6G frequency range overlapping with SISO FR-1.

Reuse of respective Multi Probe Anechoic Chambers for 6G overlapping bands with FR1 and FR2 MIMO.

Compact Antenna Test Range (CATR) for 6G frequency range overlapping with FR-2.

New spectrum can utilize either/[both] Anechoic chamber or/[and] CATR for frequency range 7GHz to 24 GHz.

Proposal 4: Include within the scope of the 6G OTA study the feasibility of extending current baseline test methods used for FR1 (SISO/MIMO) OTA performance testing to the new spectrum ranges frequencies (lower part of the 7-24 GHz range) after identifying the specific frequencies that are of initial interest.

Some background and history of how the 5G OTA work progressed will be helpful to revisit here as it provides a good template to build upon for 6G OTA tasks and timelines.

- Work Items Begin (Release 15): The formal work item for New Radio Access Technology began in March 2017, focusing on the Release 15 specifications.

- Work item initiated on Release 19, introducing MIMO OTA with Dynamic Channel Models for 6G overlapping bands with FR1.

- Test Methods Study (TR 38.810): The study item for 5G NR test methods (FR2 OTA, RRM, and Demodulation), which led to TR 38.810, was started in RAN4 in April 2017. This study was crucial for defining the OTA testing methodology for millimeter-wave (mm-Wave) devices.

- RF Requirements Normative Work: The normative work on the RF requirements for NR, including the specification that would become TS 38.521-1, began later in 2017 as part of the Release 15 work item, following the groundwork laid by the study items.

  - TS 38.521-1 Planning: Initial planning for the normative test specification TS 38.521-1 began around November 2017.

2.2 Harmonized TN/NTN framework

Harmonized TN and NTN design has been listed as a key consideration in [2] vis-a-vis 6G RAN and there were multiple associated contributions in the 3GPP 6G workshop that suggested ideas on how this can be potentially achieved in 6G. A summary of some RF and physical layer considerations that need to be factored in such a potential harmonized TN and NTN framework, and which were listed across the contributions leading to [1] are -

- Numerology and waveform harmonization - To minimize UE complexity and enable reuse of TN hardware, there is possibility of choosing numerologies or waveforms that can work reasonably well in both TN and NTN environments. This includes methods to compensate for time-frequency distortions due to Doppler or Delay spread.

- Hardware constraints: Harmonization implies that UEs might need RF components, antenna designs, frequency references etc. that allow operation in both terrestrial conditions (lower delay, relatively less mobility) and NTN conditions (higher delay, relatively higher mobility and maybe wider dynamic range).

- Waveform choices and candidate designs: The shared waveforms have to be evaluated to see performance under delay/doppler and to assess which ones maintain acceptable performance while being hardware efficient.

- Co-existence and interference management: Shared bands between TN and NTN can lead to potential challenges around interference, scheduling, beamforming, guard bands etc. The broader study must focus on interference probability and performance trade-offs.

While the above aspects will mostly be considered during the study of the NTN topic itself, what will be interesting in the OTA domain is to consider the harmonization of the test framework as well. Till date, the NTN OTA testing aspects are still ongoing development vis-a-vis test methods and metrics. Which in turn means that the existing reference OTA test methods used for smartphones would also be a reference for NTN devices. This can be further extended to study whether all aspects of a harmonized OTA system can be used for testing potential 6G harmonized TN/NTN devices especially since there might be several overlapping bands used for both types of testing.

Proposal 5: Study a harmonized approach (test method/test system) to testing TN/NTN devices in 6G OTA including but not limited to test method and test metric definition, UE positioning guidelines, MU assessment etc.

2.3 AI/ML testability use cases

RAN4 has been working on some initial use cases for 5G AI/ML during the Release 19 work item focused on beam prediction accuracy, CSI reporting accuracy, positioning accuracy, and AI/ML based mobility in Rel-20 with a subset of these requirements making it to the normative portion. The use cases are primarily FR2 focused and therefore a parallel OTA study has also occurred in Rel-19.  In parallel, RAN1 has begun study on AI/ML work earlier this year with a work plan finalized in [4] that has significant AI/ML scope. Some early way forwards have been summarized in [5].

The Rel-19 work has also focused on finalizing the test methodology aspects for verifying the core requirements to be finalized for AI/ML performance and therefore a Rel-20 study on AI/ML OTA testing can start with the baseline established in Rel-19.

Proposal 6: Use the AI/ML test methodology finalized in Rel-19 as a baseline for Rel-20 6G OTA study and further evaluate study of AI/ML testability use cases across RF/RRM/Demod areas.

# 3	Conclusions

In this discussion paper, we have presented our initial views on 6G OTA testability.

Observation 1: 6G frequency range overlaps with existing 5G NR frequency range FR-1 and FR-2.

Proposal 1: RAN4 to reuse as much as possible the existing baselines of the NR frequency ranges FR-1 and FR-2 for RF requirements definition and testability of 6G frequency ranges.

Proposal 2: Identify the specific frequencies of interest (lower part of 7-24 GHz range seems to have obvious interest) to begin 6G OTA study for SISO/MIMO.

Proposal 3: To further discuss the testing methodology for the 6G new frequency spectrum range (~7GHz to 24 GHz), with a primary focus on Standalone (SA) mode.

Proposal 4: Include within the scope of the 6G OTA study the feasibility of extending current baseline test methods used for FR1 (SISO/MIMO) OTA performance testing to the upper mid band frequencies (lower part of the 7-24 GHz range) after identifying the specific frequencies that are of initial interest.

Proposal 5: Study a harmonized approach (test method/test system) to testing TN/NTN devices in 6G OTA including but not limited to test method and test metric definition, UE positioning guidelines, MU assessment etc.

Proposal 6: Use the AI/ML test methodology finalized in Rel-19 as a baseline for Rel-20 6G OTA study and further evaluate study of AI/ML testability use cases across RF/RRM/Demod areas.

# 4	References

- RP-251881, "New SID: Study on 6G Radio", NTT DOCOMO, June 9-13, 2025

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, Aug 25th – Aug 29th, 2025

- TR-38.820, "NR; 7 - 24 GHz frequency range (Release 16)"

- RAN1 workplan for Rel-20 Study of 6GR, RAN1#122, Bengaluru, India, Aug 25th-29th, 2025

- R1-2506455, Moderator summary #1 on AI/ML for 6GR, RAN1#122