3GPP TSG-RAN4 Meeting  #116bis	R4-2513032

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.10

Source:	Apple

Title:	On spectrum sharing for 6GR

Document for:	Discussion

# 1	Introduction

At RAN#108, 6G WG SI was approved [1], with the following objectives related to spectrum sharing for 6GR:

(2) Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] …… (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

From the above objectives, and following RAN4 chair’s guidance in [2], RAN4 spectrum sharing discussion includes the following scope:

In this contribution, we share our views on spectrum sharing for 6GR.

# 2	Discussion

## 2.1 6GR spectrum landscape and x-RAT spectrum sharing

To ensure a successful 6G commercialization, service providers must have sufficient spectrum for addressing both capacity demand and coverage demand in their network. The spectrum for 6GR deployment includes,

- New spectrum to be identified in WRC-27.

- Frequency bands defined in 5G specification while no deployment is considered till now. e.g. Band n104.

- Frequency bands already deployed for 5G. E.g. many FR1 (and possibly FR2) bands such as frequency ranging from 3GHz to 5GHz (n41, n77, n78, n79 etc.).

- Possible frequency bands still in LTE operation mainly in the frequency below 3GHz.

Unfortunately, new frequency bands to be identified in WRC-27 is expected to be very limited and all in high frequency range (e.g. around 7GHz) which are not optimal for IMT deployment. To ensure operators’ 6GR deployment and smooth transition for legacy spectrum, it is pivotal to deploy 6GR in the existing frequency bands which are mostly occupied by 5G and 4G. Therefore, spectrum sharing between 6GR and legacy systems is a necessity. The basis for this is a highly dynamic MRSS scheme, allowing the scheduler to, on a dynamic basis, assign resources to 5G or 6G users following the short-term traffic variations for both RATs and providing smooth migration.

Figure 2.1-1 dynamic sharing between 6GR and 5G

While RAN1 is responsible to enable MRSS from frame structure, numerology, signal alignment design perspective, RAN4 could start to discuss MRSS preference regarding target deployment scenario for different bands.

- Thanks to the ultra-lean design, 6G - 5G MRSS demand is clear and efficient for FR1 bands migration. Such MRSS is an enhancement of LTE-NR DSS where DL spectral efficiency (SE) of the NR carrier is sacrificed a lot. The NR-6GR MRSS, by RM patterns around the few NR always-on signals such as SSBs or SIB1, the spectral efficiency penalty of NR carrier can be pretty low. (see an example calculation in Table 2.1-1). In this regard, 6G-5G MRSS for FR1 bands should be prioritized. Also the calculation indicates that the overhead is larger for smaller channel bandwidth than that for larger channel bandwidth. It would make sense to focus large channel bandwidth (e.g.>5Hz) for MRSS.

Table 2.1-1MRSS SSB+SIB1 overhead for FR1: SSB (20RBs) periodicity =20ms, SIB1(48RB) periodicity=40ms

- The demand on spectrum sharing between 6GR and 4G in below 3GHz bands where there may be LTE networks is unclear. Unless it is a must migration path from 4G direct to 6G, we prefer not to support 6G-4G MRSS due to large overhead of addressing 4G always-on signal and the commercial unsuccessfulness of 5G-4G DSS.

- The demand on spectrum sharing between 6GR and 5G is unclear for FR2 bands due to limited 5G deployment in millimetre-wave band. we prefer not to consider spectrum sharing in FR2 or treat it with low priority.

Proposal 1: RAN4 should prioritize 6G-5G MRSS in FR1.

Proposal 2: Do not consider MRSS between 6G&5G for small channel bandwidths, e.g., 3MHz (or even 5MHz)

Proposal 3: Prefer not to support MRSS in FR2 or treat it with low priority.

## 2.3 RAN4 implication for MRSS feasibility

Depending on how MRSS signal is designed as well as the migration scenarios to be addressed there could be different implication for RAN4 feasibility study. E.g. The SSB design is fundamental to initial access, cell detection, and synchronization in 5G NR. There could be different discussion direction on SSB design in RAN1, including using sharing/not sharing signal channel between 5G and 6G, sharing scheme (FDM or TDM) etc. RAN4 assessment on raster design is critical to ensure efficient MRSS operation in terms of fast cell search and achieving UE power saving goal. Also, RAN4 may need to assess the impact to RRM measurement delay performance and the chance to reduce measurement burden. So, timely coordination on these aspects between RAN1/4 working groups is helpful for a robust MRSS design.

Another potential aspect is co-existence between 6GR and legacy RATs to be shared with.  RAN4 introduced UL 7.5kHz shifting solution (shown in figure 2.3-1) in 5G-4G DSS due to different waveform is used for LTE and NR mode respectively. For 6G-5G MRSS, such mechanism is not needed. While it is still needed to address the co-existence issue for potential 6G-4G MRSS.

Table 2.3-1 UL 7.5kHz shifting for DSS

Proposal 4: 7.5kHz shifting is not needed for 6G-5G MRSS.

Proposal 5: Timely coordination between RAN1/4 on MRSS signal design/raster design/measurement shall be ensured for efficient and robust MRSS operation.

At last we would like to emphasize that reflecting upon the NR DSS feature, which was introduced in Rel-16, we observe that no network deployments of DSS have appeared in practice.  At least two possible explanations can be identified: 1) the availability of EN-DC as an evolution path from LTE to NR and 2) the impact of DSS on the performance of both the legacy (LTE) and new (NR) carriers.  Attempting to apply this lesson to 6G MRSS, we observe that 5G-6G dual connectivity is not considered a high-level objective for the 6G design.  Thus, the second lesson remains of interest.  6G-5G MRSS design should take this practical consideration into account.

# 3	Conclusions

In this contribution, we discussed 6G spectrum sharing and would like to make the following proposals.

Proposal 1: RAN4 should prioritize 6G-5G MRSS in FR1.

Proposal 2: Do not consider MRSS between 6G&5G for small channel bandwidths, e.g., 3MHz (or even 5MHz)

Proposal 3: Prefer not to support MRSS in FR2 or treat it with low priority.

Proposal 4: 7.5kHz shifting is not needed for 6G-5G MRSS.

Proposal 5: Timely coordination between RAN1/4 on MRSS signal design/raster design/measurement shall be ensured for efficient and robust MRSS operation.

# 4	References

RP‑251881, "New SID: Study on 6G Radio," NTT DOCOMO (Moderator)

R4-2511652, "Plan on RAN4 6G discussion and meeting arrangement in Rel-20," RAN4 Chair (Apple)