# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513034

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	8.7

Source:	Apple

Title:	On 6G RAN4 RRM scope

Document for:	Discussion

# Introduction

In RAN #109 meeting, the revised SID of study on 6G Radio was agreed [1], and the RAN4 RRM related scopes were summarized as follows:

(5) 6G RRM core and performance requirements d) RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles Others aspects that have RRM impact: (2) Physical Layer structure for 6GR h) Initial access [RAN1, RAN2, RAN4] i) 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] (4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

In RAN4 #116 meeting, RAN4 chairman has also shared the 6G work plan [2] for RAN4, and following RRM scopes are highlighted:

In this contribution, we discuss the scope of RRM for 6GR and propose the direction for future specification.

# Measurement gap for 6GR RRM

The measurement gap (MG) has been specified in both 4G and 5G standard as an essential functionality for UE to measure target cell with a specific disconnection from current serving cell. Besides MG, there is other concept similar as MG, i.e., scheduling restriction in 5G RRM. Regarding the baseline MG concept in 5G system, there are two type of MGs (per-FR and per-UE MG),

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE does not support independent measurement gap patterns for different frequency ranges, the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers. If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports independent measurement gap patterns for different frequency ranges, the network must provide either per-FR measurement gap patterns for frequency range where UE requires per-FR measurement gap for concurrent monitoring of all frequency layers of each frequency range independently, or a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers of all frequency ranges.

In current 5G design, there are 25 legacy patterns in TS38.133 table 9.1.2-1 with complicated applicability/prioritization rules. All of them are semi-static configuration from network as specified in TS38.331 and the inflexible configuration/applicability will result into high overhead (15% throughput loss for 40ms MGRP with 6ms MGL). Moreover, different feature has different specific MG pattern: e.g., positioning MG, MUSIM MG, i.e., no unified solution of MG can be used for per-UE basis.

The key problems of current MG design are summarized below,

- NR has both MG and scheduling restriction for serving link disconnections, and they are applied independently

The summary for baseline scheduling restriction in 5G is as follows,

- Scheduling restriction of MG-less L3 measurement - To apply for FR1 due to mixed numerology btw. SSB and PDCCH/PDSCH - To apply for FR2 due to mixed numerology and Rx beam sweeping btw. SSB and PDCCH/PDSCH - Restriction window: - Async scenario: SMTC window - Sync scenario: {symbols for SSBs to measure, and/or symbols for RSSI to measure} +/- 1 symbols - Scheduling restriction of MG-less L1 measurement (RLM/BFD/CBD/L1-RSRP) - To apply for FR1 due to mixed numerology btw. SSB and PDCCH/PDSCH - To apply for FR2 due to mixed numerology and Rx beam sweeping btw. SSB/CSI-RS and PDCCH/PDSCH - Restriction window: - symbols for SSB/CSI-RS to measure - Scheduling restriction periodicity follows RS periodicity of L3 or L1 (if periodic measurement is configured)

Similar as MG configuration, the scheduling restriction design also follows semi-static configuration, i.e., according to the target reference signal periodicity, and it has high overhead (e.g., in async case, 25% throughput loss for 5ms SMTC length with 20ms SMTC periodicity). However, currently, MG and scheduling restriction are handled independently following the 3GPP rule in 5G (like in figure 1), i.e.,

- MG follows MG pattern,

- Restriction applies outside MG, and need to differentiate L3 and L1 based restriction with resource sharing factor

Figure 1. summary for MG, scheduling restriction and interruption

Besides the above design, the legacy measurement resource coordination for RRM is also very complicated and inflexible, including,

- Gap based L3 and L1(LTM) measurement vs. Gap-less based L3 and L1 measurement

  - Fully overlapped: share the MG resource based on share factor

  - Partially overlapped: Gap-less based L3 and L1 measurement are performed outside MG

- Gap-less L3 measurement vs. L1 measurement

  - Fully overlapped: resource sharing is L3:L1 = 2:1

  - Partially overlapped: L1 is performed outside SMTC

For mobility and serving link management purposes, network can configure UE with L3 measurement objects with/without MG (SSB based or CSI-RS based L3 measurement), L1 measurements with/without MG (e.g., RLM, BFD, L1-RSRP, LTM) and corresponding MG, and then UE may need to check the following steps to decide the resource allocation ratio among these RRM tasks.

- Step 1: Check the L3 measurement without MG are fully overlapped with MG or not. MG pattern may or may not colliding with the L3 measurement without MG(e.g., intra-frequency measurement).

- Step 2a: if ‘No’ in step 1, all RRM measurement without MG will be performed outside MG occasion, and this includes partial overlap and fully non-overlap case, e.g., SMTC periodicity is 20ms and MGRP is 40ms, it means UE can perform at least one RRM measurement outside the MG occasion. However, such RRM measurement without MG may still cause scheduling restriction.

- Step 2b: If ‘Yes’ in step 1, all the RRM measurement with MG will share the MG resource. The MG sharing means, UE may use e.g., a round robin manner (up to UE implementation) to perform the RRM measurement on MG occasions following the MGRP.

- Step 3a: following step 2a, UE needs to further check if all L1 measurement occasions are fully overlapped with SMTC or not. Especially in FR2, due to the Rx beam difference, it was assumed that L1 measurement cannot be performed in parallel with L3 measurement. If ‘Yes’, then L3 and L1 measurement outside MG follows 2:1 resource sharing; otherwise, if ‘No’, L1 measurement is performed outside SMTC and MG occasions. Such L1 measurement outside MG can also have its own scheduling restriction.

- Step 3b: following step 2b, all other L1 measurement is performed outside MG and SMTC. Such L1 measurement outside MG can also have its own scheduling restriction.

The complete workflow for RRM resource coordination can be illustrated in the following diagram. In current 5G design, the total disconnection duration will count both MG and scheduling restriction, e.g., >27.5% throughput loss when MGL=6ms, and SMTC window=5ms in a SMTC periodicity of 40ms.

Figure 2. workflow for 5G RRM resource coordination considering MG

Based on the above analysis, we observed that the independent design of MG and scheduling restriction has resulted into the complicated UE implementation and higher overhead (throughput loss).

Observation 1: the independent design for MG and scheduling restriction is complicated and overhead consuming.

- The MG patterns design was over-designed and applicability of MG patterns is complicated.

In current 5G design, there are 25 legacy patterns in TS38.133 table 9.1.2-1 with complicated applicability/prioritization rules, as in following tables.

In practice, the actual deployed MG patterns in 5G are much less than the specified patterns in the above table, which causes unnecessary UE implementation complexity and cost. In order to simplify specification and UE/network implementation, 6G shall consider reducing the MG patterns and shall only focus on the most typical MG patterns for real use cases.

Observation 2: the actual deployed MG patterns in 5G are much less than the specified patterns in RRM spec.

In addition, some special MG patterns were introduced for specific features, e.g., pattern 24/25 in table 9.1.2-1 for positioning and 29 patterns in Table 9.1.10-1 for MUSIM Gap Pattern Configurations. In other words, the diverse MG patterns were specified for different features and the applicability of MG is super complicated.

Observation 3: the diverse MG patterns were specified for different features and the applicability of MG is super complicated.

- MG patterns were requested/configured for certain specific use cases only

In 5G RRM, MG patterns are requested/configured for certain specific use cases only, e.g., MUSIM measurement and positioning measurement. MG patterns #24 and #25 can be requested only when the UE is configured with any of RSTD, UE Rx-Tx, PRS-RSRP or PRS-RSRPP measurements requiring such gaps and can only be used during the corresponding positioning measurement period. As specified in the TS38.133, a UE shall request a priority for all requested periodic MUSIM gaps when the UE requests MUSIM gaps via MUSIM-GapConfig-r17. The UE shall request different priority level for each periodic MUSIM gaps. The network may assign priority to each periodic MUSIM gaps. The allocated priorities may differ from the priorities requested by the UE. The UE MUSIM requirements apply if the configured MUSIM gap priorities retain the same relative priorities among MUSIM gaps as requested by the UE.

However, in our view, such MG requesting mechanism could be extended to a general mechanism for RRM, i.e., UE can request MGRP and MGL configuration based on UE capability and mobility status. For instance, UE with short RF tuning/retuning time can request a short MGL as well as MG TA, where in 5G MGL is technically equivalent to “2*RF-tuning + measurement duration”, as shown in the following figure. Additionally, UE in low mobility or has more searcher resources may request longer MGRP. Less RRM measurement occasions are needed if AI/ML model can help to predict the target measurement object instead of real measurement, and then network can also configure sparser MG on time domain without any mobility performance degradation.

Figure 3. MGL example

Another drawback of legacy design in 5G is: the network is unaware that UE is really performing inter-frequency measurement or not during the MG. That means it will always have 15% throughput loss for 40ms MGRP with 6ms MGL after MG configuration, since if L3 measurement with MG is configured with MeasGapConfig, the scheduling/control channel will be skipped during the MGL regardless of UE is really performing inter-frequency measurement or not. And it will result into complicated and lengthy gap configuration/reconfiguration procedure.

Observation 4: MG requesting mechanism has benefits to let network understand the UE status, e.g., whether configured MG is really used and which MGRP/MGL is needed. However, such MG requesting mechanism was not considered for general RRM measurement in 5G.

- The granularity of MG sharing was coarse

In 5G, the MG sharing was designed to coordinate MG resource allocation between intra-frequency MOs and inter-frequency/inter-RAT MOs; and in previous RAN4 discussion, some operators prefer to further improve the MG sharing mechanism to prioritize some specific RATs, e.g. LTE in EN-DC mode. But the singling bit string is quite limited in Rel-15/16, e.g. 2 bits, to indicate the MG sharing ratio, and therefore it’s far from enough to adequately cover all the prioritization preference from network operators.

Table 1. MG sharing in 5G

For instance, LTE also plays key role in EN-DC operation; so depending on different demands from different network operators, it would be much beneficial if the MG sharing mechanism can allow network to prioritize any RAT measurement as it wants. In Rel-18, LTM ICBM L1 measurement with MG was introduced, but there was no flexible L3 and L1 MG sharing mechanism either.

Observation 5: in 5G, MG sharing can only be configured for RRM resource sharing between {Intra-frequency with MG} vs. {Inter-frequency and Inter-RAT with MG}, and the sharing factor cannot differentiate inter-RAT vs. inter-frequency MOs, or L3 measurement and L1 measurement.

- Only one CC measurement was assumed in each MG occasion

Regarding the measurement inside UE MG, in 5G only one frequency layer is required in each single MG, and CSSF inside MG was specified in section 9.1.5.2 of TS38.133. It means the total measurement delay would be decided by the gap-based measurement object number and the MGRP, e.g., Tmeasure=sample_num*MGRP*MO_num.

However, if UE can support CA combination for multiple frequency layers, there might be more possibilities to enhance the measurement behavior, e.g., UE can perform multiple frequency layers measurements in a single MG and only the UE searcher limitation will limit the frequency layers in a single MG, e.g., if UE supports 2 searchers for the measurements inside MG, it’s possible that Tmeasure=sample_num*MGRP*MO_numsearcher_num. It’s very straightforward to have half measurement delay for all MG based MOs and mobility performance can be enhanced accordingly.

Observation 6: in 5G, only one CC measurement was assumed in each MG occasion regardless of UE’s band combination capability and searcher number. However, in many cases, UE can simultaneously perform multiple frequency layers measurements in a single MG, as long as UE supports such CA combination of the target frequency layers and 2 searchers.

Based on the above analysis, we also think MG design could be a RAN4 led direction in this 6G SI, since MG design is not strongly coupled with the RS sequence design from other WGs and all the MG enhancement WI were initiated from RAN4 and then led by RAN4.

- Rel-16 inter-frequency measurement without MG. RP-201883. RAN4 led - Rel-17 NR and MR-DC measurement gap enhancements. RP-220772. RAN4 led - Pre-configured MG pattern(s) per configured BWP (fast MG configuration) - Multiple concurrent and independent MG patterns - Network Controlled Small Gap (NCSG) - Rel-18 Further enhancements on NR and MR-DC measurement gaps and measurements without gaps. RP-232440. RAN4 led - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - RRM requirements for measurements without gaps

Thus, we have the following proposals for 6G RRM SI scope:

Proposal 1: MG design is discussed in Rel-20 6G SI from RAN4 RRM perspective.

Proposal 2: for MG design scope, following aspects can be studied:

- To study reducing MG patterns from 5G(e.g., only focus on most typical MG use cases)

- To study unified MG concept in 6G

  - Unified MG for NR MG and NR scheduling restriction

  - Unified MG sharing among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement)

  - Unified MG for different feature related measurements, e.g., RRM measurement, MUSIM related measurement, positioning measurement, and etc

- To study UE-request-based MG configuration and applicability

  - UE request of MG for multiple measurement purposes

  - MG activation/deactivation due to measurement demands

- To study multiple CC measurement in single MG occasion

# Virtual RRM UE group for 6GR

In current market, many UE vendors including Apple take the interoperable ecosystem approach, aiming to build interconnected experiences, e.g., iPhone, iPad, Apple watch, and Vision Pro. In such ecosystem, usually the trusted device feature allows users to authenticate and secure their interactions across devices seamlessly. Question is now: How much can we leverage the industry trend of the cross-device interoperability in wireless communication especially from RRM perspective?

Figure 4. virtual RRM UE group

As of now, RRM function is operated individually on these interoperable devices regardless of proximity status, which will probably cause the unnecessary overhead and power consumption. In our view, why RRM could leverage such ecosystem is:

- No coordination among devices is required in terms of timing, data sharing, hardware sharing (Antenna sharing)

- RRM overhead can cost more than 25% time/frequency resources, e.g., 5ms SMTC window in 20ms periodicity

- RRM is a significant contributor to power consumption in the Idle/Inactive mode of a UE, where it remains for a large portion of time, especially in low-activity scenarios.

From our perspective, the benefits by using virtual RRM UE group is also very apparent to both UE and network, e.g.,

- Throughput improvement to both UE and network: e.g., Offload/share RRM measurement with MG or scheduling restriction from high-traffic-load device to other low-traffic-load device.

- UE power saving: e.g., offload RRM measurement from power-hungry device to other devices

- Signaling overhead reduction by using single unified RRM measurement configuration/reporting

However, to decouple from the other WG discussion, RAN4 can prioritize the case without side-link, and there is no Tx/Rx timing sharing, no data sharing, and no coherent transmission/reception among the devices in the same virtual RRM UE group. How devices coordinate with each other in the virtual group can be left to UE proprietary implementation, and it shall not have any spec impact.

Thus, the scope of virtual RRM UE group can include: study the virtual RRM group concept for Only RRM functionality only; and study the schemes for measurement load balance among devices in virtual RRM group.

Proposal 3: virtual RRM UE group is discussed in Rel-20 6G SI from RAN4 RRM perspective.

Proposal 4: for virtual RRM UE group, following aspects can be studied:

- To study the feasibility of Virtual RRM group concept

  - Only RRM functionality (e.g., L3 RSRP/RSRQ measurement) is considered

  - No Tx/Rx timing sharing, no data sharing, no coherent transmission/reception is considered

  - Prioritize the case without side-link

- To study the feasibility and schemes of Measurement load balance among devices in virtual RRM group

# RRM enhancement for mobility

Mobility is one of the most critical procedures in wireless communication. In 5G design, mobility is supported for UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED modes. Here we would like to focus on connected mode. Mobility in RRC_CONNECTED mode includes handover, RRC re-establishment, RRC redirection upon RRC release, L1/L2 triggered inter cell mobility (LTM) and etc.

Most typically, mobility in RRC_CONNECTED mode is triggered by UE RRM measurement. Measurements to be performed by a UE for connected mode mobility are classified in at least four measurement types:

-	Intra-frequency NR measurements;

-	Inter-frequency NR measurements;

-	Inter-RAT measurements for E-UTRA;

-	Inter-RAT measurements for UTRA.

For each measurement type one or several measurement objects can be defined (a measurement object defines e.g. the carrier frequency to be monitored). For each measurement object, one or several reporting configurations can be defined (a reporting configuration defines the reporting criteria). Three reporting criteria are used: event triggered reporting, periodic reporting and event triggered periodic reporting.

In typical handover procedure, network first configures measurement objects to UE so that UE can perform RRM measurement accordingly. Once UE realizes neighbor cell becomes better than serving cell in terms of signal quality such as RSRP and/or RSRQ, UE sends measurement report to network. Upon receiving UE measurement report, network would trigger handover procedure. Detailed steps can be found in the following figure.

Figure 5. Intra-AMF/UPF Handover

RRM measurement is periodic after configuration. UE updates its measurement results following a certain measurement cycle from hundreds of milliseconds to several seconds, rather than every slot. The measurement cycle is determined by measurement type, number of carriers configured for measurement and UE’s own measurement scheme. Therefore, there is latency between the time when channel condition can meet event triggered threshold and the time when UE realizes the condition is met. Usually, network would configure threshold with value larger than 0 (e.g. RSRP from neighbor cell is 3dB higher than that of serving cell) and Time-To-Trigger (TTT) window to avoid unnecessary ping-pong handover. That means during this latency, UE would suffer from low SINR, which results in low spectrum efficiency.

Observation 7: long measurement latency results in poor handover performance and user experience.

Figure 6. baseline handover procedure in 5G

During handover procedure, data is interrupted after UE receives RRC for handover command (upon handover execution). Data is resumed after handover is completed. Total interruption time, according to TS38.133, is from dozens of milliseconds to several seconds, depending on configuration and scenarios.

Actually, 3GPP has introduced several solutions to enhance RRC_CONNECTED mode handover procedures in 4G and 5G:

- Conditional handover (CHO): to increase HO successful rate and throughput

- RACH-less handover: to reduce interruption time

- Make-before-break handover: to reduce interruption time

- Dual active protocol stack (DAPS) handover: target at 0ms interruption

- L1/L2 triggered inter-cell mobility (LTM): to reduce interruption and increase throughput

Note that none of these enhancements has been widely used in commercial deployments due to limited gain and complexity at both UE and network sides. However, it doesn’t mean nothing needs to be enhanced. e.g. handover failure and throughput degradation during handover can still be observed in the field. Some new 6G business expect higher handover performance.

Observation 8: there is still room for interruption reduction during handover procedure.

Proposal 5: for study on RRM enhancement for mobility, following two directions can be studied:

- Facilitate preparation phase: to reduce latency between the time when channel condition can meet event triggered threshold and the time when UE realizes the condition is met.

- Reduce interruption during HO execution without significantly increase complexity at both UE and network sides.

# Other RRM aspects for 6GR

- UE state based RRM

In 5G, measurement and reporting configuration are often semi-static and network driven, regardless of UE is stationary or in mobility. Thresholds to trigger neighbor cell measurement are severely dependent on the user mobility state. If UE is stationary, it may not have high demand of neighbor cell measurement.

This is not only about UE power saving, but also about measurement efficiency. In R16 UE power saving, some RRM relaxation was introduced to slow down RRM measurement, but mainly for idle mode. However, even for connected mode UE, we still see some room for further optimization. For instance, if UE is stationary, we believe it is feasible to relax both L1 and L3 measurement in connected mode. Besides, since UE is stationary, we don’t expect sharp change in measurement and CSI feedback. Therefore, reporting reduction can also be considered. Also, low handover rate is expected for stationary UE. To avoid ping-pong handover, a different threshold for neighbor cell measurement triggering when UE is in stationary mode can be considered.

Proposal 6: study UE state based RRM, e.g.

- Investigate measurement reduction for stationary UE, including both L3 and L1 measurement

- Investigate reporting reduction for stationary UE, including both RRM and CSI reporting

- Investigate threshold for neighbor cell measurement triggering when UE is in stationary mode

- RRM in mTRP

mTRP has been supported in NR. Note that so far only intra-frequency is supported for mTRP according to RAN1 design. However, UE can still be configured with CA. Thus, mTRP can also be considered as inter-frequency to serving cells on other carriers. Therefore, from deployment point of view, it is feasible to extend mTRP to inter-frequency in 6G.

We are thinking about what RAN4 needs to do if such scenario is to be supported.

Figure 7. mTRP deployment example in 6G

In our understanding, TRPs can be categorized into anchor TRP (A-TRP) and compacity TRP (C-TRP, wherein A-TRP is mainly for cell level mobility maintenance while C-TRP is mainly for data communication (e.g. with larger BW). From RRM perspective, we need RS from both TRPs (called A-TRP and C-TRP. For instance, we can rely on CD-SSB from A-TRP while NCD-SSB or even CSI-RS from C-TRP. They can be used for RRM measurement, C-TRP selection, addition/release, activation/deactivation, RLM/BFD/CBD and so on. Some of the existing design in NR could be reused, but some others may not since different TRPs are on different carriers. Further study is expected.

Proposal 7: study RRM impact from mTRP on different carriers, if supported.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Observation 1: the independent design for MG and scheduling restriction is complicated and overhead consuming.

Observation 2: the actual deployed MG patterns in 5G are much less than the specified patterns in RRM spec.

Observation 3: the diverse MG patterns were specified for different features and the applicability of MG is super complicated.

Observation 4: MG requesting mechanism has benefits to let network understand the UE status, e.g., whether configured MG is really used and which MGRP/MGL is needed. However, such MG requesting mechanism was not considered for general RRM measurement in 5G.

Observation 5: in 5G, MG sharing can only be configured for RRM resource sharing between {Intra-frequency with MG} vs. {Inter-frequency and Inter-RAT with MG}, and the sharing factor cannot differentiate inter-RAT vs. inter-frequency MOs, or L3 measurement and L1 measurement.

Observation 6: in 5G, only one CC measurement was assumed in each MG occasion regardless of UE’s band combination capability and searcher number. However, in many cases, UE can simultaneously perform multiple frequency layers measurements in a single MG, as long as UE supports such CA combination of the target frequency layers and 2 searchers.

Proposal 1: MG design is discussed in Rel-20 6G SI from RAN4 RRM perspective.

Proposal 2: for MG design scope, following aspects can be studied:

- To study reducing MG patterns from 5G(e.g., only focus on most typical MG use cases)

- To study unified MG concept in 6G

  - Unified MG for NR MG and NR scheduling restriction

  - Unified MG sharing among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement)

  - Unified MG for different feature related measurements, e.g., RRM measurement, MUSIM related measurement, positioning measurement, and etc

- To study UE-request-based MG configuration and applicability

  - UE request of MG for multiple measurement purposes

  - MG activation/deactivation due to measurement demands

- To study multiple CC measurement in single MG occasion

Proposal 3: virtual RRM UE group is discussed in Rel-20 6G SI from RAN4 RRM perspective.

Proposal 4: for virtual RRM UE group, following aspects can be studied:

- To study the feasibility of Virtual RRM group concept

  - Only RRM functionality (e.g., L3 RSRP/RSRQ measurement) is considered

  - No Tx/Rx timing sharing, no data sharing, no coherent transmission/reception is considered

  - Prioritize the case without side-link

- To study the feasibility and schemes of Measurement load balance among devices in virtual RRM group

Observation 7: long measurement latency results in poor handover performance and user experience.

Observation 8: there is still room for interruption reduction during handover procedure.

Proposal 5: for study on RRM enhancement for mobility, following two directions can be studied:

- Facilitate preparation phase: to reduce latency between the time when channel condition can meet event triggered threshold and the time when UE realizes the condition is met.

- Reduce interruption during HO execution without significantly increase complexity at both UE and network sides.

Proposal 6: study UE state based RRM, e.g.

- Investigate measurement reduction for stationary UE, including both L3 and L1 measurement

- Investigate reporting reduction for stationary UE, including both RRM and CSI reporting

- Investigate threshold for neighbor cell measurement triggering when UE is in stationary mode

Proposal 7: study RRM impact from mTRP on different carriers, if supported.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, RANP #109

[2] R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, RAN4 Chair (Apple), RAN4 #116