3GPP TSG-RAN WG4 Meeting # 116bis 	    R4-2513060

Prague, CZ, 13 – 17 October 2025

Agenda Item:	8.9

Source: 	Ericsson

Title:	RAN4 study on 6G AI/ML

Document for:	Approval

# Introduction

Study item description for 6GR has identified objectives for RAN4 that needs to be achieved by RAN#116 meeting [1]. According to the approved timeline for 6GR study, this is the first meeting for 6G discussions in RAN4. To meet the objectives identified for RAN4 for 6GR study item, RAN4 chair in RAN4#116 meeting shared the planned topics for until RAN4 meetings in Q1 2026 [2]. According to the plan shared by chair, detailed discussion on AI/ML use cases, CDL model derivation for demodulation requirements, and the scope of AI/ML study in RAN4 are discussed in this contribution.

# Discussion

## Use cases

### AI/ML for UL post-distortion compensation

Problem description

The power amplifier (PA) is typically the most power-hungry component in a wireless transceiver. Therefore, any improvement in PA energy efficiency directly enhances the overall efficiency of radio transceivers. To increase coverage and improve energy efficiency in radio networks, it is desirable to operate PAs in a regime that maximizes output power and efficiency. However, this often requires operating in a more non-linear region. Such operation, while energy-efficient, introduces non-linear distortions that degrade link quality and make reliable signal reception at the receiver more difficult, as shown in Figure 1.

Figure 1: Power amplifier input-output characteristics

The trade-off between power efficiency and throughput remains one of the key implementation challenges in radio transmitters. Digital Pre-Distortion (DPD) is a widely used technique that applies an inverse distortion through a pre-distorter before the signal is fed into the PA, aiming to achieve an overall linear RF transmitter behavior. Implementing DPD typically requires good estimation of the PA’s nonlinear characteristics, obtained through a feedback loop from the PA output, along with signal processing. For wideband signals at higher frequency bands, the required signal processing becomes even more complex and power intensive. Moreover, in uplink (UL) communication, user equipment (UE) or mobile devices often have limited computational resources and power consumption, making the implementation of advanced DPD potentially challenging. As a result, power back-off is commonly applied to mitigate PA nonlinearity by shifting the operating point into a more linear region. However, this approach reduces UE output power and degrades the UL coverage.

Observation 1: PA should operate in a linear region to ensure reliable signal reception. For PA to be linear, UE needs to back-off power which affects UL coverage and energy efficiency.

Observation 2: Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off.

Proposal 1: RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion.

Apart from the PA, several UE RF front-end components can introduce non-linearities that degrade UL performance, including the oscillators, IQ modulator/demodulator, mixers, and duplexers etc. Additionally Crest Factor Reduction (CFR), if exists, applied to limit PAPR, can introduce in-band distortions. These components can give rise to additional impairments such as phase noise and IQ imbalance.

Proposed technical solution

To address the issue described in the previous section, Digital Post-Distortion (DPoD) provides a solution for compensating UE front-end nonlinearities. DPoD techniques can be applied at the BS receiver, where the receiver reconstructs the transmitted signal by modeling and compensating the nonlinear distortion introduced by the UE’s non-linear components, as shown in Figure 2.  However, with AI-based methods, this knowledge can be implicitly captured by the AI/ML model, enabling a flexible and adaptive compensation without relying on an explicit nonlinearity model. Further, BS has more computational resources and energy availability to train and process such AI model.

Figure 2: UL Post-Distortion Concept

Observation 3: AI-DPoD at the BS receiver offers solution to compensate UE RF non-linearity to reduce UE’s MPR for higher order and thus improve UL high data rate availability.

Further, the usefulness of DPoD depends on what limits the UE’s PA linearity. If the primary limitation comes from in-channel distortion caused by PA non-linearities, then DPoD could help mitigate the issue by compensating for the distortion at the receiver side. However, DPoD cannot correct the unwanted emissions, such as out-of-band radiation and spurious emissions that impact neighboring channels.

Observation 4: The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions.

A simple solution would be to adjust the EVM requirements for the UE on in-band distortion in the presence of DPoD, thereby reducing the needed UE MPR. However, RAN4 should evaluate not only the limit to which EVM can be adjusted, but also whether there are dependencies affecting this limit, such as resource block (RB) allocation or operating frequency, and whether the extent to which EVM can be adjusted varies depending on the UE implementation. Additionally, if there are additional UE requirements related to the UE transmit signal, while ensuring that the UE remains compliant with Adjacent Channel Leakage Ratio (ACLR) and Spectrum Emission Mask (SEM) specifications. Further, RAN4 should avoid adding additional margins in the BS test to incorporate non-linearities or impairments in the UE. This would ensure strict RAN4 performance requirements while keeping the baseline of creating minimum requirements that can be passed by both UE and BS.

Proposal 2: RAN4 could consider adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit.

As mentioned earlier, the UL PA is not the only source of non-linearity in the UE transmitter. Other RF impairments, such as oscillator phase noise, carrier leakage and IQ imbalance, can also have a significant impact on performance. The wide variety of UE RF front-end  in the market makes it challenging to standardize specific models or to define RAN4 performance requirements and test parameters accordingly. A more viable approach is to employ AI/ML techniques to train a generalized model based on RF impairment characteristics collected from multiple UEs. This generalized model can then be studied at the BS receiver baseband to effectively compensate for overall UE RF front-end nonlinearities and thereby improve UL signal quality. Further, the complexity and implementation of this generalized model need to be evaluated which can be acceptable to the companies. Also, the parameters that define this model and data needed to train this model needs further discussion.

Proposal 3: RAN4 should study AI based UL-Post distortion compensation at BS receiver and understand the feasibility of creating RAN4 core and performance requirements.

Furthermore, the proposed uplink post-distortion solution should be strictly network controlled. In other words, the UE may only adjust its EVM requirement and apply MPR reduction when explicitly signaled by the network. In the absence of such signaling, the UE must continue to comply with the existing RF requirements.

Proposal 4: UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

Performance benefits

In the following, preliminary evaluation results of AI/ML-based uplink PA DPoD are presented as compared to the baseline in an uplink DFT-s-OFDM scenario. Table 1 and Table 2 in the appendix present the parameters used for training and evaluation.

Figure 3 shows the receiver architecture with AI/ML de-mapper to handle UE RF non-linearities.

Figure 3: Schematic of AI based UL Post-Distortion

Figure 4. Uplink throughput vs. SNR for i) an ideal PA (“no ML, no PA”), and a PA with 5 dB back-off ii) when using an AI/ML-empowered receiver that compensates for PA non-linearities (“ML, 5 dB backoff”), and iii) a non-AI/ML receiver not attempting to perform.

In Figure 4, the “ML, 5 dB backoff” curve refers to a AI/ML receiver compensating for the distortions introduced by a UE PA (see [5]) operating with a 5 dB back-off. The PA operation still allows the UE to comply with ACLR requirements but is assumed not to be required to satisfy the current error vector magnitude (EVM) requirements. In contrast, "no ML, 5 dB backoff" represents a traditional (non-ML) receiver operating with a 5 dB back-off (without DPoD capability), and "no ML, no PA" corresponds to an ideal case with no distortion. The simulation includes link adaptation targeting a 10% BLER, reflecting realistic adaptive modulation and coding behavior. Notably, the key performance gains from the AI/ML DPoD are observed in the high SNR region (above 10 dB), where 256-QAM is typically used. This highlights the AI/ML de-mapper’s effectiveness in handling high-order modulation under non-ideal transmitter conditions, while no noticeable gain is seen for lower-order modulation schemes (16QAM and below) at lower SNRs. In our initial evaluation, we have only considered UE’s PA non-linearity and no other RF components in the UE front-end.

Observation 5: AI/ML-based uplink PA digital post-distortion (DPoD) could in certain conditions achieve performance similar to that of ideal PA without power backoff, resulting in a potentially higher UE output power and/or more power efficient operation for higher order modulations.

Benefit of the proposed solution

UE MPR reduction and EVM adjustment

With the AI-based UL post-distortion (AI-DPoD) feature, UEs can compensate for PA non-linearities more effectively, improving distortion tolerance. As a result, the required UE MPR (Maximum Power Reduction) can be reduced, allowing UEs to transmit at higher effective power without violating linearity requirements. This has potentially two key benefits:

- Increased UL coverage: Higher transmit power extends the uplink range for higher order modulations.

- Improved UE energy efficiency: Lower MPR reduces the back-off from PA saturation, enabling the UE to operate more efficiently, possibly reducing power consumption.

Proposal 5: RAN4 to evaluate the potential MPR reduction enabled by AI-DPoD and its impact on UL coverage and UE transmit energy efficiency.

Observation 6: In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits.

Current UL EVM requirements are specified assuming that the UE transmitter operates within the linear region of the PA, where distortions are minimal. With AI-based UL post-distortion (AI-DPoD) applied at the base station, much of the PA non-linearity can be compensated in the network. This could enable the adjustment of UE EVM requirements but would require changes in EVM measurement procedure and equalizer by inclusion of AI-DPoD receiver.

Proposal 6: RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station.

Observation 7: The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance.

Potential study areas for this feature

There are several open issues to be addressed, in order to study and eventually define RAN4 core and performance requirements.

UE transmitter model

For UL post-distortion at the BS receiver to function effectively, accurate modeling of the UE front-end transmitter is essential, with emphasis on the PA model and other non-linear RF elements such as oscillators, mixers etc. The UE RF front-end model can be trained using AI/ML algorithms; however, since numerous UEs with diverse front-end architectures exist in the market, this leads to a very large dataset of RF models that must be learned by the AI/ML algorithm. Furthermore, RF characteristics vary across the frequency spectrum, and the non-linear behavior is further influenced by additional real-world impairments such as phase noise, local oscillator leakage, IQ imbalance, and variations in EVM across subcarriers. Inter-symbol interference introduced by the PA itself may also affect its nonlinear response. In addition, PA nonlinearity is sensitive to temperature, operating frequency, and load mismatch conditions. Envelope tracking (ET) adds further complexity because the PA supply voltage is continuously adjusted in real time. This means the PA’s behavior is not fixed but changes with the signal envelope and power management circuit response, making accurate modeling even more challenging.

Therefore, developing a robust and accurate UE model is critical for the effectiveness of AI/ML-based UE front-end modeling.​

Observation 8: Accurate modeling of UEs PA is necessary for this feature. However, transmitter characteristics have many variations due to other RF impairments, frequency range etc. Therefore, UE RF front-end needs to be evaluated by RAN4, so that reliable and representative data from the RF front-end can be obtained to effectively train the AI/ML model.

Proposal 7: RAN4 needs to study feasibility on how to train AI/ML algorithms with variety or subsets of UE transmitter models.

AI/ML model training

There can be two approaches to train AI/ML algorithms with different UE RF front-end models: offline and online.

In offline training, UE transmitter characteristics are collected through measurements or simulations and used to build generalized models in advance. This allows the use of large datasets and complex AI/ML architectures without adding real-time processing burden on the UE, but it may not capture device-specific variations or adapt to changes in transmitter behavior over time due to factors such as temperature, frequency band shifts, or device aging etc. Further, offline trained models may need to be updated or fine-tuned during certain intervals to account for new transmitter models being implemented by the UEs.

Online training, on the other hand, enables adaptive learning at the BS receiver, thereby capturing dynamic variations in transmitter characteristics and improving accuracy for individual UEs. However, this approach introduces higher computational complexity, potential latency issues at the BS, and scalability concerns for the UE in multi-user scenarios. Another issue is the training time; if it takes too long to train the AI/ML model, the data may already be delivered from the UE to the BS.

Observation 9: To characterize multiple UE transmitters, either a fine-tuned offline model or less precise and adaptive online approach can be feasible, considering the UE power consumption needed for model training.

Observation 10: Offline trained AI/ML models may need to be updated in a certain time period to include new UE transmitter models being implemented by UEs.

Proposal 8: RAN4 may study which AI/ML algorithms are feasible for modeling UE transmitter characteristics, and which type of model training would be feasible.

Data Collection and Feedback:
To effectively train AI models, data should be collected when UEs operate in the compressed region, i.e., the non-linear operating range of the transmitter, where post-compensation techniques are most beneficial. Training is not feasible if UEs remain in the linear region, since little or no distortion is present. For data collection, it is important for RAN4 to evaluate suitable approaches. A key question is whether simulation-based data, generated using realistic UE Tx front-end models and diverse PA characteristics, would be sufficient, or whether data gathered directly from multiple UEs would provide a more reliable basis.

- Proposal 9: RAN4 to study data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region. Both simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs) could be evaluated. Other RF impairments should be considered and included.

UE RF Front-End Variation:

- For UL post-distortion studies, it is important to recognize the UE dependency of RF front-end characteristics and DPD implementations. A UE’s performance is shaped by multiple non-linear components such as the PA, oscillator, and IQ modulator, and in practice, different UEs on the market exhibit varying front-end designs and impairment levels. Also, CFR, when applied to limit PAPR, can also introduce in-band distortion. This variability should be handled by the AI/ML model that is both representative and practical. RAN4 should therefore account for this diversity when developing study assumptions and evaluating compensation techniques.

- Proposal 10: RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable AI/ML model for UL post-distortion studies.

- Proposal 11: The AI/ML model used for DPoD should be trained effectively to handle the UE front-end variation.

- This can be handled by introducing new EVM measurement procedure with updated equalizer taking to account the DPoD receiver.

BS Demodulation Impact

To define base station performance requirements for this feature, RAN4 should consider several aspects such as:

Frequency range: The study should cover both FR1 and FR2. Higher PA non-linearity is generally expected at higher frequencies, but this depends on PA type, device design and operating conditions. At higher frequencies additional impairment sources become important (oscillator/phase-noise, PA nonlinearities interacting with phase noise and DAC quantization/noise).

Waveform: As agreed in RAN4#116, CP-OFDM and DFT-s-OFDM are supported for UL transmission. For  initial evaluation, both waveforms can be studied.

Channel Model: As the AI-DPoD feature aims to reduce MPR and allow UEs to transmit at higher power, it is important to evaluate which channel models are most suitable for the selected test metrics.

Modulation: Based on initial evaluation, key performance gains from AI/ML DPoD are observed in the high-SNR region, where higher-order modulations such as 256-QAM are typically applied. Therefore, it is feasible to evaluate the feature with higher modulation orders,

Reference AI/ML model selection: To support consistent evaluation of BS performance, it is necessary to define a reference AI-model with reasonable complexity level. This reference model can serve as a common framework for agreement on key parameters, given that different companies may implement AI/ML solutions differently.

Proposal 12: RAN4 to evaluate UE PA non-linearity and other related impairments across both FR1 and FR2 for AI based UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

- Proposal 13: RAN4 to study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature.

Proposal 14: RAN4 to evaluate channel models for link-level simulation with AI-DPoD feature, considering their impact on test metrics under higher UE transmit power.

Proposal 15: RAN4 to evaluate AI/ML DPoD performance with higher modulation orders focusing on high-SNR conditions.

Proposal 16: RAN4 to evaluate on how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations.

Observation 11: As part of the study, RAN4 should decide the appropriate complexity level of the reference AI/ML model.

### AI/ML receiver for DM-RS reference signals overhead reduction

Demodulation reference signal (DM-RS) overhead is substantial in practical NR PxSCH transmissions when the number of active DM-RS ports grows. This overhead hampers the achievable spectral efficiency of the NR air interface, which the 6GR interface aims at enhancing. Advanced receivers have the capability of reducing the number of orthogonal time-frequency DM-RS resources required for PxSCH processing for 6GR operation with respect to NR.

Observation 12: RAN1 proposes to study the use case of AI/ML receivers for PxSCH reception with substantially lower DMRS overhead than in NR.

### AI/ML receiver for CSI-RS reference signals overhead reduction

As the number of antenna ports increases, Channel State Information (CSI) acquisition for codebook-based DL transmission becomes more challenging. One problem is the large CSI-RS overhead. For example, in Rel-19 NR, up to 128 CSI-RS ports are supported, which can be used for a network with 128 active antenna ports. It is expected that the number of antenna ports can go even higher in 6G. Therefore, a potential solution is to reduce the CSI-RS overhead in port domain (or spatial domain), with the help of a UE-sided AI/ML model, where the UE-sided AI/ML model can estimate the channel for all antenna ports based on measurement of a subset of antenna ports.

Observation 13: RAN1 proposes to study the AI/ML use case of CSI-RS overhead reduction with UE-side model.

## CDL model derivation for demodulation requirements

The discussions on AI/ML for RAN started from Rel-18. The topic mainly focusses on how to apply AI models for RAN relevant key features and how to standardize these models. In agreed 6G SID, both non-AI and AI features would be introduced. In general, channel model should be common for both non-AI-based and AI-based receivers, but the difference is the AI models might be more trained by data with realistic channels which would be more randomly variable in time, frequency and spatial domains. Simplified channel model used for non-AI-based receiver might not be suitable for AI-based receiver requirement definition and tests.

Observation 14: Channel models used for non-AI-based receiver might not be suitable for AI-based receiver due to AI model would be trained by data experienced real and more complicated channels.

The AI models should be trained and tested by tremendous data, but it is tricky to have such kind of data. Possible sources could be field measurement, lab tests or system level simulations etc. No matter which kind of data would be used, the channel model used for requirement definition should get aligned with the channel properties of training data, otherwise it could have misleading performance.

In most RAN1 evaluations on AI models, CDL models are used in system level simulations. As for link level simulations for performance evaluation, there is no consensus on using CDL models or TDL models since there is no standardized CDL models used for demodulation for now. But the problem of TDL model is lack of realistic spatial domain fading information which leads to some quite different signal properties at receiver side. In that case, CDL models seem better for AI model testing than TDL models. However, the performance impact of CDL models on AI/ML model-based receiver also needs further study.

Observation 15: Using TDL models might get misleading performance results from AI models trained by different types of data including field data, lab data and system level simulation (with CDL models) data.

Proposal 17: RAN4 to study the performance impact by using CDL models for AI/ML based receiver.

If CDL models are considered for AI/ML receiver demodulation requirements, following aspects can be considered for feasibility validation.

- Scenario dependency

- Different conditions coverage

- Richness of spatial domain

- Testability

A certain AI feature could be developed for one or several target scenarios, such as CSI prediction for dense urban or indoor office or for both, when considering wide model generalization. In that case, the model would be trained to cover those target scenarios. From the CDL channel model perspective, different scenarios would lead to quite different model parameters. To test the AI feature, multiple CDL models derived from different scenario SCM statistical distribution would be needed.

Observation 16: Different scenarios might be covered by an AI feature and multiple CDL models would be needed to cover these scenarios.

If we focus on one scenario, the channel condition would also be time variable with UE antenna orientation or UE movement etc. The AI model should work well in all these conditions, so the corresponding tests might need to cover these conditions. Thus, the CDL models might also be time, frequency and spatial variable to show the channel changing within a reasonable range. For example, the CSI prediction, the time changing model would be very important to verify the AI model performance. In previous releases using TDL models, the general way is to only consider the worst case for demodulation, such as the highest Doppler shift and delay spread. But for AI based receiver, it is hard to define the worst case, such as handheld UE antenna orientation changing, for AI models, and the coverage might be more critical here. In that case, arrival/departure angels, UE velocity vector etc., could be variable within a reasonable range during the tests.

Another relevant aspect is on receiver capabilities. For example, UE might have different CSI prediction model implementations due to complexities, UE antenna numbers etc. The corresponding prediction period or prediction precision would be different. The performance requirement could be different and would lead to different channel models are applied, such as slow changing or fast changing. This further increases the number of model variants for the tests.

Observation 17: Different environments, UE antenna orientation, UE mobility or different UE capabilities might be quite relevant to an AI feature, and multiple CDL models with variable parameters would be needed to cover these conditions.

When we look at the single link, the channel richness is expected to be high enough to represent the real environment impact. It could be very critical for some very high layer or large bandwidth transmission. From CDL model perspective, it means the number of clusters should be as many as possible. It will directly increase the test complexity due to multiple links that need to be set up during demodulation tests. How to balance channel richness and test complexity should be studied according to simulation or tests. Of course, this issue also happens in non-AI condition, but AI condition would be much more sensitive to it regarding more CDL model variants are needed.

Observation 18: Channel richness might be important to some special features but would also impact the complexity of test instruments.

As we discussed above, the testability of CDL models would be quite challenging due to potentially many model profiles used in the tests to have better coverage and better reliability. To simplify the complexity of profile generation before tests, maybe formula-based methodology could be studied on top of some limited model profiles.

Furthermore, another critical issue for AI model tests is that the test data should not be used for training, otherwise the test data would be useless for performance verification since AI model could pass it easily. But the channel model used for demodulation is randomness fixed model which means the test signal could be easily created for training. This could be a general issue for all AI-based receiver tests. To avoid this situation, more discussions are needed in RAN4.

Observation 19: If CDL models are randomness fixed and could be easily for test signal creation, the performance test might be meaningful once test signals are used for AI model training.

Proposal 18: RAN4 to study following aspects when applying CDL models for demodulation requirements definition for AI-based receiver.

- Model variants depending on scenarios covered by AI feature

- Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

- Balance between channel model richness and test complexity.

- How to avoid test signals being used for AI model training.

## Aspects to be studied in RAN4 for UE based AI/ML feature

### Model generalization

Use of AI/ML models for beam management (BM), channel state information (CSI) prediction, mobility, and positioning were studied in Rel. 18 and Rel. 19 study items. Motivation to conduct study items on AI/ML based BM, CSI prediction, and BM was to enhance operational efficiency of the cellular network. For positioning, the idea was to further enhance the accuracy of the positioning measurements by using AI/ML models in densely cluttered environment such as indoor open office and indoor factory where non line of sight radio condition between the UE and network node has been observed to severely impact the accuracy of the NR positioning measurements. While the performance of AI/ML for these features were evaluated, it was observed that the AI/ML models perform well in the scenarios it is trained for, but the performance of AI/ML models degrade when the AI/ML model trained in one scenario is tested in another scenario for which it is not trained for. When the model is tested with the data from the scenario where it is trained for, it could be the case that the model is tailored to perform well in that specific scenario, hence a decent gain in terms measurement accuracy with reduced latency is observed. The performance of such AI/ML model drops significantly when tested with data from a different scenario that does not follow the noise pattern and the data fluctuation pattern of the trained data. This behaviour of the AI/ML models is identified as generalization in the AI/ML terms and concerns the ability of AI/ML model to apply the patterns it has learned from the training data to new or unseen data for inference. Where RAN4 is responsible to ensure a predictable and reliable UE behaviour and measurements reported by the UE to the network are decently accurate, generalization capability of AI/ML model deployed to support 6G use cases becomes of utmost importance. It is therefore important to study and verify if the performance of AI/ML model is predictable in scenarios that depict realistic deployments and if the model performance varies between scenarios then to it is important to understand to what degree the performance of the model varies and study/identify its impact on RAN4 specification.

Observation 20: For a given use case, adequate performance across scenarios/conditions (i.e., generalization) cannot be guaranteed.

During the 5G-A SI and WI reporting of applicability of AI/ML model was considered as one of the ways to overcome generalization issues. This approach was mainly considered for the use cases where the AI/ML model is hosted at the UE and governs a solution in terms of signaling from the UE to the network node indicating if the model at UE is applicable for inference in the scenario where the UE is configured to report the inferred measurements by the network. When the model is deemed inapplicable by the UE, UE may fallback to non-AI/ML measurement procedure or operation to report the configured measurements to network. In one hand this fallback procedure provides some degree of robustness to measurement operation, specifically in cases where the AI/ML models become inapplicable, but in another hand will induce extra reporting delay as UE will need to switch its operation from AI/ML based to non-AI/ML based to report measurements configured by the network. The applicability reporting framework largely depends on the UE capability to determine when its model becomes inapplicable and how quickly it can fallback to non-AI/ML based operation. To ensure predictable and reliable applicability reporting is performed by the UE, requirements (accuracy requirement could be one such candidate) for such applicability reporting may need to be defined. In this regard, it is important to study and understand how well UEs can reliably determine applicability of its AI/ML model.

Observation 21: Applicability reporting framework, as introduced/discussed in 5G-A SI/WI for AI/ML based features risk inducing additional reporting delay when UE is configured to report the inferred measurement in scenarios where the UE deems its AI/ML model as inapplicable.

Observation 22: Applicability reporting framework largely depends on the UE capability to accurately determine when its model becomes inapplicable and how quickly it can fallback to non-AI/ML based operation.

Traditionally for UE features such as CSI reporting or RRM measurements, RAN4 assumes UE applies the same algorithm regardless of the test scenarios (e.g., antenna configuration, SNR, channel model). Based on this assumption, RAN4 usually defines one set of test cases to verify the UE feature. This approach however may not be reasonable to test AI/ML based feature as the performance of AI/ML model heavily depends on the scenario it is trained for. To test AI/ML based performance of a UE consideration of non-static test scenarios/conditions and propagation conditions for testing can be one of the approaches. Test metric for such a testing consideration is however not straight forward. An alternative approach could be an approach where a UE that can infer measurement needs to be tested in multiple candidate scenarios that depict realistic deployments. In this approach a UE capable of using AI/ML model for inferencing the measurement should then be able to meet the minimum performance requirements in all the scenarios identified for testing. This approach means that AI/ML based UE feature needs to be tested in multitude of test scenarios making testing a cumbersome process for both TE and DUT compared with the traditional non-AI/ML-based tests. Given the rising interest in using AI/ML model to enhance operational efficiency of many features of 6G cellular network, generalization of AI/ML model performance becomes a key aspect that needs a careful consideration by RAN4 as RAN4 is the leading group that is responsible to define acceptable performance for the AI/ML based feature and identify test cases for the validation of UE capability of producing reliable and predictable inference by using the AI/ML model.

Observation 23: Testing UE based AI/ML feature in multitude of test scenarios makes testing a cumbersome procedure for UE and TE vendors.

Proposal 19: RAN4 to study generalization of UE based AI/ML functionality for 6G and identify its impact on RAN4 specification.

### Performance monitoring

As discussed above, performance of the AI/ML model can be largely limited to the scenario it is trained in. It is therefore important to monitor the performance of the AI/ML model. The performance monitoring of the UE sided model can either be done by the UE or done by the network. For the UE sided monitoring, the performance metric used by the UE may not be reported to the network. For the network sided monitoring, UE needs to report the monitoring metric to the network node. Based on the UE reported monitoring metric, if network determines the performance of the AI/ML model has degraded and is not reliable and useful, network may configure UE to fall back to non-AI/ML procedure so that AI/ML model is not used for measurement inferencing or retrain its model and or to update its AI/ML model so that the consistency in AI/ML model performance is maintained. For network sided monitoring the accuracy of the monitoring metric and the quality of the reported monitoring metric is critical. If the monitoring metric is not accurate enough or the quality of the monitoring metric is not good enough, then the network node cannot configure the UE to take the necessary corrective action(s).

Requirements for monitoring becomes even more relevant when the AI/ML model is up to the UE implementation as this leads to the scenario where different flavors of AI/ML models will be deployed by different UE vendors to support the same feature. Since the network may not know the actual AI/ML inference quality at UE, it is important to ensure that the UE reports the correct performance accuracy indication, so that the network can take appropriate action to properly activate/deactivate the AI/ML functionality at UE. The consistency among the quality and accuracy of monitoring metrics reported for different AI/ML implementations is therefore vital to ensure a uniform action, based on the reported monitoring metric, can be taken by the network. It is evident that the monitoring metric may be different for different AI/ML based use cases, but the fundamental question on what such a metric should look like and how to define potential requirement for such a metric need to be first understood and therefore needs to be studied by RAN4. Once this fundamental issue is addressed, further discussion on need of use case specific monitoring metric can be conducted when the AI/ML based use cases for 6G are identified to be studied.

Observation 24: Type of AI/ML model to be deployed for AI/ML based feature can be up to UE implementation.

Observation 25: For the same feature different flavors of AI/ML model may be deployed by different UE vendors.

Proposal 20: RAN4 to study performance monitoring aspects related to UE based AI/ML functionality and identify its potential impact on RAN4 specification.

### Interoperability

More 6G features are expected to be supported by AI/ML models that are deployed at the UE or at the network node. For some of those features, some part of the operation may have to performed by the model at the UE and the other by the model at the network node. In all these scenarios interoperability due to the implementation of AI/ML model becomes a key issue. To better elaborate the scenarios that may arise due to the implementation of AI/ML to facilitate 6G features, the further discussion is focused on issues arising from the one-sided and two-sided model deployments.

#### One-sided model deployment

During the 5G-A SI and WI it has been observed that one-sided model deployment is feasible to support use cases such as RRM measurement prediction, positioning, and CSI prediction. Similar assumption can also be made when it comes to supporting these features by deploying AI/ML model in 6G. In these use cases, when the model is deployed at the UE, the UE is configured by the network node to infer RRM/positioning/CSI measurements and report to the network. In scenarios where the network needs to support a mix of UEs that can use AI/ML model to infer measurements and those which do not support AI/ML for measurement inferencing it is important to ensure that the accuracy of the predicted measurement and measured measurement are within the same error threshold. This is particularly important to ensure consistent decision can be made by the network node irrespective of whether the UE can support the feature based on AI/ML or not. This aspect is important to ensure that the AI/ML based operation and non-AI/ML based operation are interoperable and equivalent from the network point of view. Besides, as highlighted in the generalization section above, it is worth mentioning here that if the model performance varies between different UEs and their models, then the network needs to know which UE it is talking to, when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs it is managing will compromise interoperability and increase the complexity when it comes to deploying the AI/ML based functionality in 6G.

Observation 26: In 6G, for the same functionality, network needs to support a mix of UEs that can use AI/ML model to infer measurements and those which do not support AI/ML for measurement inferencing.

Observation 27: If the AI/ML model performance varies between different UEs and their models, then the network needs to know which UE it is talking to, when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs will compromise interoperability and increase the complexity of deploying the AI/ML based functionality in 6G.

Proposal 21: Interoperability for one-sided model deployment refers to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature.

Proposal 22: RAN4 to study how to ensure consistent behaviour between UEs supporting AI/ML and UEs not supporting AI/ML for the same functionality.

#### Two-sided model deployment

In contrast to one-sided model, two-sided model deployment concerns a scenario or a use case where the AI/ML models to support a functionality is deployed at the UE and the network node. In simpler terms, the AI/ML operation to support a specific functionality is split into two parts where one part of the AI/ML operation is conducted by the model hosted at the UE and the other part of the AI/ML operation is conducted by the model hosted at the network node. For such an operation interoperability between the AI/ML models deployed in two different nodes in the cellular network needs to be ensured. One example of such a use case is CSI compression which is being specified as a part of 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

Observation 28: Interoperability for two-side model is being addressed for CSI compression in 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

### LCM and post deployment testing

To support AI/ML based feature, AI/ML model needs to be deployed at the UE which needs to undergo different stages of LCM while being hosted at the UE. Where LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model, it imposes complexity on testing procedures. Depending on the deployment choice, a trained or an untrained model can potentially be deployed at the UE. When a trained model is deployed at the UE, it can be assumed that the performance of the model is validated before the model is being deployed. Meaning that the UE capable of supporting the AI/ML based feature is tested and its performance is verified before it is deployed to the field. When the model is updated after deployment, then the question remains how to validate the performance of the updated AI/ML model. This is particularly important to ensure that the predication accuracy does not degrade after model is updated. A similar situation also arises when an untrained AI/ML model is deployed at the UE. The untrained model, in this scenario, first needs to be trained before it can be used for inferencing. For training, UE may perform data collection procedure to train its model. Since the model is trained after being deployed, the question, like validation of the performance of an updated model, needs to be answered. Impact on RAN4 requirement due to LCM procedure can potentially be discussed after the AI/ML based features to be studied for 6G are identified, but the discussion on testing complexity can already start in RAN4.

Observation 29: LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance.

Observation 30: LCM procedures like model training, model retraining, and model update impose complexity to testing procedures.

Proposal 23: RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment.

### Testability of AI/ML based features

Conventionally testing approach in RAN4 has been to evaluate the performance of a UE under the propagation condition that depicts a typical radio link between the network node and the UE. The RAN4 testing procedure, thus far, has been designed to validate the link level performance of UE under the assumption that the UE that can meet the requirements in RAN4 defined test scenarios should also be able to consistently perform well in the real field. This is a fair assumption to make when it comes to testing of measurement accuracy for non-AI/ML based feature that relies on the quality of the received radio signal.

When it comes to AI/ML based feature, validating the performance of model under certain radio condition and expecting the model behavior to remain consistent regardless of the scenario may not hold true. As highlighted earlier in this paper and observed during the 5G-A SIs, the performance of AI/ML based feature observed in one scenario may not necessarily hold true in another scenario. This behavior of AI/ML based feature makes testing more complex and time-consuming procedure and therefore mandates a careful consideration from the beginning of the 6G study item. Moreover, 6G is expected to also operate in new range of frequencies from 7 GHz to 15 GHz. This is new frequency range and testing in this new range of frequencies may have their own challenges also mandating a need to study testability of AI/ML based feature from the beginning of the 6G study item. Besides the possibility of AI/ML model being trained on real world data cannot be neglected. Testability and testing procedures for models that are trained with real world data and tested in a lab setup should also considered as a part of RAN4 study for 6G features anticipated to be supported by AI/ML models deployed at UE. In this regard, it is important to ensure that the test conditions are closely aligned with real-world parameters.

Observation 31: Performance of AI/ML based feature observed in one scenario may not necessarily hold true in another scenario. This behavior of AI/ML based feature makes testing more complex and time consuming requiring a careful consideration from the beginning of the 6G study item.

Observation 32: 6G is expected to also operate in new range of frequencies from 7 GHz to 15 GHz. Testing in this new range of frequencies may have their own challenges mandating a need to study testability of AI/ML based feature from the beginning of the 6G study item.

Observation 33: Test conditions need to be closely aligned with real-world parameters to verify the performance of AI/ML based feature where the model is trained with real world data.

### RRM requirement

RAN1 aims to identify AI/ML based use cases by RAN1#123 meeting and RAN2 is starting the discussion on general framework for AI/ML from RAN2#131bis meeting. To ensure AI/ML based feature identified by RAN1 and RAN2 is supported in 6G, RAN4 role will be to define the RRM requirements for those use cases. Moreover, unlike in 5G-A SI and WI, RAN4 should involve from the beginning of evaluation work where RAN4, based on its expertise, should support by providing inputs in terms of realistic impairment models that have an impact on AI/ML performance and needs to be considered when feasibility of AI/ML based feature is being studied by RAN1 or RAN2 working group. In this regard, depending on the progress made by RAN1 and RAN2, RAN4 might have to start discussions to identify the impact of AI/ML use cases driven by other working groups on RAN4 specification earlier than planned timeline which is RAN4 meetings after Q1 2026.

Observation 34: To ensure AI/ML based feature identified by RAN1 is supported within the framework defined by RAN2, RAN4 role will be to define the RRM requirements for those use cases.

Observation 35: Depending on the progress made by RAN1 and RAN2, RAN4 may have to start discussions to identify the impact of AI/ML use cases driven by other working groups on RAN4 specification earlier than planned timeline which is RAN4 meetings after Q1 2026.

Observation 36: Unlike in 5G-A SI and WI, RAN4 should involve from the beginning of evaluation work where RAN4, based on its expertise, should support other working groups by providing inputs in terms of realistic impairment models that needs to be considered when feasibility of AI/ML based feature is being studied.

# Summary

In this paper discussion on AI/ML use cases, CDL model derivation for demodulation requirements, and the scope of AI/ML study in RAN4 are presented. The following observations and proposals summarize the discussion presented in this paper.

# AI/ML for UL post-distortion compensation

Observation 1: PA should operate in a linear region to ensure reliable signal reception. For PA to be linear, UE needs to back-off power which affects UL coverage and energy efficiency.

Observation 2: Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off.

Proposal 1: RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion.

Observation 3: AI-DPoD at the BS receiver offers solution to compensate UE RF non-linearity to reduce UE’s MPR for higher order and thus improve UL high data rate availability.

Observation 4: The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions.

Proposal 2: RAN4 could consider adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit.

Proposal 3: RAN4 should study AI based UL-Post distortion compensation at BS receiver and understand the feasibility of creating RAN4 core and performance requirements.

Proposal 4: UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

Observation 5: AI/ML-based uplink PA digital post-distortion (DPoD) could in certain conditions achieve performance similar to that of ideal PA without power backoff, resulting in a potentially higher UE output power and/or more power efficient operation for higher order modulations.

Proposal 5: RAN4 to evaluate the potential MPR reduction enabled by AI-DPoD and its impact on UL coverage and UE transmit energy efficiency.

Observation 6: In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits.

Proposal 6: RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station.

Observation 7: The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance.

Observation 8: Accurate modeling of UEs PA is necessary for this feature. However, transmitter characteristics have many variations due to other RF impairments, frequency range etc. Therefore, UE RF front-end needs to be evaluated by RAN4, so that reliable and representative data from the RF front-end can be obtained to effectively train the AI/ML model.

Proposal 7: RAN4 needs to study feasibility on how to train AI/ML algorithms with variety or subsets of UE transmitter models.

Observation 9: To characterize multiple UE transmitters, either a fine-tuned offline model or less precise and adaptive online approach can be feasible, considering the UE power consumption needed for model training.

Observation 10: Offline trained AI/ML models may need to be updated in a certain time period to include new UE transmitter models being implemented by UEs.

Proposal 8: RAN4 may study which AI/ML algorithms are feasible for modeling UE transmitter characteristics, and which type of model training would be feasible.

- Proposal 9: RAN4 to study data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region. Both simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs) could be evaluated. Other RF impairments should be considered and included.

- Proposal 10: RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable AI/ML model for UL post-distortion studies.

- Proposal 11: The AI/ML model used for DPoD should be trained effectively to handle the UE front-end variation.

Proposal 12: RAN4 to evaluate UE PA non-linearity and other related impairments across both FR1 and FR2 for AI based UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

- Proposal 13: RAN4 to study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature.

Proposal 14: RAN4 to evaluate channel models for link-level simulation with AI-DPoD feature, considering their impact on test metrics under higher UE transmit power.

Proposal 15: RAN4 to evaluate AI/ML DPoD performance with higher modulation orders focusing on high-SNR conditions.

Proposal 16: RAN4 to evaluate on how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations.

Observation 11: As part of the study, RAN4 should decide the appropriate complexity level of the reference AI/ML model.

# AI/ML receiver for DM-RS reference signal overhead reduction

Observation 12: RAN1 proposes to study the use case of AI/ML receivers for PxSCH reception with substantially lower DMRS overhead than in NR.

# AI/ML receiver for CSI-RS reference signals overhead reduction

Observation 13: RAN1 proposes to study the AI/ML use case of CSI-RS overhead reduction with UE-side model.

# CDL model derivation for demodulation requirements

Observation 14: Channel models used for non-AI-based receiver might not be suitable for AI-based receiver due to AI model would be trained by data experienced real and more complicated channels.

Observation 15: Using TDL models might get misleading performance results from AI models trained by different types of data including field data, lab data and system level simulation (with CDL models) data.

Proposal 17: RAN4 to study the performance impact by using CDL models for AI/ML based receiver.

Observation 16: Different scenarios might be covered by an AI feature and multiple CDL models would be needed to cover these scenarios.

Observation 17: Different environments, UE antenna orientation, UE mobility or different UE capabilities might be quite relevant to an AI feature, and multiple CDL models with variable parameters would be needed to cover these conditions.

Observation 18: Channel richness might be important to some special features but would also impact the complexity of test instruments.

Observation 19: If CDL models are randomness fixed and could be easily for test signal creation, the performance test might be meaningful once test signals are used for AI model training.

Proposal 18: RAN4 to study following aspects when applying CDL models for demodulation requirements definition for AI-based receiver.

- Model variants depending on scenarios covered by AI feature

- Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

- Balance between channel model richness and test complexity.

- How to avoid test signals being used for AI model training.

# Aspects to be studied in RAN4 for UE based AI/ML feature

Observation 20: For a given use case, adequate performance across scenarios/conditions (i.e., generalization) cannot be guaranteed.

Observation 21: Applicability reporting framework, as introduced/discussed in 5G-A SI/WI for AI/ML based features risk inducing additional reporting delay when UE is configured to report the inferred measurement in scenarios where the UE deems its AI/ML model as inapplicable.

Observation 22: Applicability reporting framework largely depends on the UE capability to accurately determine when its model becomes inapplicable and how quickly it can fallback to non-AI/ML based operation.

Observation 23: Testing UE based AI/ML feature in multitude of test scenarios makes testing a cumbersome procedure for UE and TE vendors.

Proposal 19: RAN4 to study generalization of UE based AI/ML functionality for 6G and identify its impact on RAN4 specification.

Observation 24: Type of AI/ML model to be deployed for AI/ML based feature can be up to UE implementation.

Observation 25: For the same feature different flavors of AI/ML model may be deployed by different UE vendors.

Proposal 20: RAN4 to study performance monitoring aspects related to UE based AI/ML functionality and identify its potential impact on RAN4 specification.

Observation 26: In 6G, for the same functionality, network needs to support a mix of UEs that can use AI/ML model to infer measurements and those which do not support AI/ML for measurement inferencing.

Observation 27: If the AI/ML model performance varies between different UEs and their models, then the network needs to know which UE it is talking to, when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs will compromise interoperability and increase the complexity of deploying the AI/ML based functionality in 6G.

Proposal 21: Interoperability for one-sided model deployment refers to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature.

Proposal 22: RAN4 to study how to ensure consistent behaviour between UEs supporting AI/ML and UEs not supporting AI/ML for the same functionality.

Observation 28: Interoperability for two-side model is being addressed for CSI compression in 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

Observation 29: LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance.

Observation 30: LCM procedures like model training, model retraining, and model update impose complexity to testing procedures.

Proposal 23: RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment.

Observation 31: Performance of AI/ML based feature observed in one scenario may not necessarily hold true in another scenario. This behavior of AI/ML based feature makes testing more complex and time consuming requiring a careful consideration from the beginning of the 6G study item.

Observation 32: 6G is expected to also operate in new range of frequencies from 7 GHz to 15 GHz. Testing in this new range of frequencies may have their own challenges mandating a need to study testability of AI/ML based feature from the beginning of the 6G study item.

Observation 33: Test conditions need to be closely aligned with real-world parameters to verify the performance of AI/ML based feature where the model is trained with real world data.

Observation 34: To ensure AI/ML based feature identified by RAN1 is supported within the framework defined by RAN2, RAN4 role will be to define the RRM requirements for those use cases.

Observation 35: Depending on the progress made by RAN1 and RAN2, RAN4 may have to start discussions to identify the impact of AI/ML use cases driven by other working groups on RAN4 specification earlier than planned timeline which is RAN4 meetings after Q1 2026.

Observation 36: Unlike in 5G-A SI and WI, RAN4 should involve from the beginning of evaluation work where RAN4, based on its expertise, should support other working groups by providing inputs in terms of realistic impairment models that needs to be considered when feasibility of AI/ML based feature is being studied.

# Appendix

Table 1: Common training and evaluation assumptions for AI-DPoD.

| Parameters | Value |
| --- | --- |
| Waveform | DFT-S-OFDM |
| Carrier frequency | 30 GHz |
| PA backoff values | 4 dB, 6 dB, 8 dB |
| PA type | CMOS (memoryless) [3] |
| Number of RBs | 60 |
| Subcarrier spacing (kHz) | 120 |
| Link adaptation | Active with MCS index table 2 in TS 38.214 |
| BS antenna configuration | [4,8,2] |
| FFT size | 1024 |

Table 2: Varying parameters used for evaluation and training dataset generation of AI-DPoD.

| Parameters | Evaluation dataset | Training and validation dataset |
| --- | --- | --- |
| Number of slots | 10000 | 200000 |
| Channel model | CDL-B, C, D | CDL-A |
| Delay spread (ns) | 100 | 30-150 |
| UE speed (km/h) | 3-30 | 3-30 |
| MCS | LA (10% target BLER) | MCS-17 (64QAM) MCS-22 (256QAM) |
| SNR (dB) | -6:2:20 | -7:5 (64QAM) 1:20 (256QAM) |

# References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator).

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair (Apple).

- R4-165901, “Further elaboration on PA models for NR”, source Ericsson.