3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513062

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Inputs to RAN4 6G study on general aspects

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.2 General aspects and others [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#109, the RAN led 6G Study objectives were updated in [1] adding the RAN4 related topics. In this contribution, we share our views on the general aspects related to UE RF, its requirements, and target performance improvements.

# Discussion

## 2.1 6G SI objectives relevant to UE RF

The portion of the 6G SI objective most relevant to the UE RF requirements is reproduced below in italics and highlighted in yellow.

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

- Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

- Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

6GR core and performance requirements

- General RF aspects [RAN4]

- Intra-3GPP co-existence studies

- Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

- BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

- Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

- UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

- Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

- RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

- Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

- Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

- Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

- Handling irregular channel bandwidths including the definition [RAN4, RAN1]

- Definition of ‘frequency range(s)’[RAN4]

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

- PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

- Aspects of integration with communication services [RAN1]

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In this contribution, we will cover the general aspects as per the RAN4 chairman note associated with the corresponding agenda item:

NOTE: Companies can use this agenda to share their general view on 6G from RAN4 perspective and any aspects which are not covered in AI8.3-8.13.

## 2.2 UERF requirements principles for 6G

5G specification complexity issues are related to:

- The clause structure which requires to go to every clause for a given feature which are in separate documents

- The lack of UE types creates ambiguity about the features and unnecessary repetitions

- Clauses are added for combination of features like CA and MIMO or different modes of operation like unlicensed, V2X. ATG, UAV. In many cases with cross references.

- Similarly, the separation of TN and NTN does not allow to make new features timely available for NTN.

- This also make it more complex to verify the completeness of spectrum or non-spectrum features.

- Some of the specification like MSD, A-MPR are related to spectrum aspects that may not be relevant in a real or regional deployment (frequency, range, channel bandwidths) or same requirements are duplicated (MSD test points at different positions or channel bandwidth verifying the same UE impairments)

- It is very difficult to verify that fallback

- There is both over specification and holes in the specifications.

Also, with a per-feature approach, unnecessary limitations are created in 5G:

- Some features being limited to some frequency range or power class or TN vs NTN, RedCap vs other UEs.

- Artificial limitations are put on the UE like with inter-band power class, boosting opportunities.

The specification is filled with lengthy tables with no or duplicated information:

- CA bands that are duplicated in the configuration tables

- Delta T, Delta R tables

- Coexistence tables

- Infinite list of band combinations, BCS for NRCA, ENDC, NEDC sometimes without coherence.

  - For example, why listing FR1+FR2 combination while there is no specific requirements for such combinations.

  - Similarly, for >3band DL combinations

- Some of these could be simplified or covered by requirements per band groups or band group combinations.

Some naming conventions that imply a sequence or order like frequency ranges or power classes make it difficult to create new “in-between” cases.

Proposal on UERF RAN4 specification and structure:

- RAN4 specification should consider features to be self-contained in a clause and introduce UE types to avoid ambiguity for the implementation.

  - No separate file for one feature.

  - In its simplest implementation, all single band requirements should be in one place in the specification, then followed by intra-band and inter-band combinations

  - Requirement could then be across UE types and applications

- TN and NTN application should not be separated, as it can be identified by different bands and UE types

- Simplify spectrum requirements by creating band-group level requirements (at least as a default)

- Rather than using frequency ranges, requirements should be separated between individual antennas/connectors/conducted measurements versus antenna arrays/beamforming/OTA measurements

- In that case, the associated frequency ranges could overlap within the 7-20GHz region: For 0.4 to 52GHz, two overlapping frequency ranges may prove sufficient.

- Simplify band and band-combination requirements (Emissions, REFSENS, MSD, blocking) with a default set of requirements per band groups and band group combinations.

- Favor equation-based requirements and parameters.

It is also important to make the specification scalable per UE types and fit to the purpose.

Proposal on UE types and categories and their requirements:

- Variable requirement support depending on UE types:

  - Not necessarily in terms of objects like smartphones, wearables or CPE/FWA, vehicles, trains etc..

  - Rather in terms of size, antenna quality/directivity/quantity/integrated/external, mobility/speed, country vs regional vs WW support.

  - This partition could then drive the channel BW, performance, nRx, nTx, MIMO, band and band combination support

- Avoid across the board mandatory requirements, avoid BCS and leave more to the UE declaration or based on UE types.

- Avoid unnecessary limitations (like maximum output power, BW support)

- Avoid overspecification (multiples ways to do the same thing, duplications, same UE behavior tested multiple times, in multiple configurations, testing configurations that are not deployed)

  - For example: no intra-band CA if the spectrum can be covered with a single carrier with an 8K FFT based CBW

  - But still ensure completeness (avoid holes, missing fallbacks).

- Avoid names implying an order use explicit value instead (power class, bandwidth classes, frequency ranges)

  - For example: Use PC23 instead of PC3 or even add #Tx like 1T23, 2T29.

## 2.3 UERF performance improvements for 6G

For 5G FR1, most of the performance parameters ended up being the same as 4G with only a few exceptions:

- Some improved duplexer performance for new FDD bands (almost uniquely for n71 and n105), overwise the 4G values were copied for the same band. And consistency across bands is not there due to averaging, different assumptions.

- Improved UL impairments and UL QAM support: 64QAM mandatory support for NR and 28dB impairments

- Improved DL performance (QAM support, number of Rx in some bands)

- Increased CBW and spectrum utilization at the expense of too many channel bandwidths and SCS that are unused but yet tested.

At the same time, the part related to band combinations exploded with the accumulation of ENDC, NRCA and NEDC and FR1, FR2 and FR1+FR2 cases, not to mention V2X.

With 6G targeting all UE types and applications from the beginning and a larger timespan for implementation and migration, it is essential to provide a step in performance on UE RF parameters beyond the new 6G features. Also, it is essential that RAN spends more time on new features and targeting the right performance level rather than treating zillions of combinations.

Proposal on improved CBW and BW parts support:

- Support of 2x 5G maximum CBW in the same band thanks to 8K FFT and single SCS per band/band-group.

- Enable variable BW support by design such that any CBW (1MHz granularity?) can be supported but only a limited set is measured.

Proposal on improved SU: Especially, there is opportunity for:

- FR1 30kHz and 60kHz SCS since a single SCS per band/band-group removes the RB alignment rule that penalize NRB for SCS other than the lowest.

- FR2-1 where the SU is fixed versus CBW at 95% (like for 4G where it was fixed at 90%) while in FR1 SU improves with increased CBW

  - For example, FR2 SU for 200MHz and 60kHz SCS is 95% while FR1 SU for 100MHz and 30kHz SCS is 98.3%)

- Higher CBW beyond the 4K FFT range assuming 8K FFT is the default.

- DFT-s-OFDM SU which is limited by the 2^x*3^y*5^z limitation.

  - For example, although for 20MHz CBW, 5G SU is larger in 5G compared to 4G for CP-OFDM with 106RBs, it is the same for DFT-s-OFDM at 100RB. Considering the addition of exponents of 7 would, for example allow 105RBs for DFT-s-OFDM.

Proposal on improved UL and DL UE performance:

- Target improved UL impairment and support: support of 256QAM for smartphone UEs with related improved impairments like carrier and image rejections that will improve EVM, IBE, MPR, A-MPR and self-desense for small duplex distance FDD bands.

- Target improved DL impairment and support: Support of 1024QAM for smartphones and associated improved image and IM2 responses and assuming better EVM on BS side.

# Conclusion

In this contribution, we provide our views on the Release 20 RAN4 6G SI general aspects.

Proposal on UERF RAN4 specification and structure:

- RAN4 specification should consider features to be self-contained in a clause and introduce UE types to avoid ambiguity for the implementation.

  - No separate file for one feature.

  - In its simplest implementation, all single band requirements should be in one place in the specification, then followed by intra-band and inter-band combinations

  - Requirement could then be across UE types and applications

- TN and NTN application should not be separated, as it can be identified by different bands and UE types

- Simplify spectrum requirements by creating band-group level requirements (at least as a default)

- Rather than using frequency ranges, requirements should be separated between individual antennas/connectors/conducted measurements versus antenna arrays/beamforming/OTA measurements

- In that case, the associated frequency ranges could overlap within the 7-20GHz region: For 0.4 to 52GHz, two overlapping frequency ranges may prove sufficient.

- Simplify band and band-combination requirements (Emissions, REFSENS, MSD, blocking) with a default set of requirements per band groups and band group combinations.

- Favor equation-based requirements and parameters.

Proposal on UE types and categories and their requirements:

- Variable requirement support depending on UE types:

  - Not necessarily in terms of objects like smartphones, wearables or CPE/FWA, vehicles, trains etc..

  - Rather in terms of size, antenna quality/directivity/quantity/integrated/external, mobility/speed, country vs regional vs WW support.

  - This partition could then drive the channel BW, performance, nRx, nTx, MIMO, band and band combination support

- Avoid across the board mandatory requirements, avoid BCS and leave more to the UE declaration or based on UE types.

- Avoid unnecessary limitations (like maximum output power, BW support)

- Avoid overspecification (multiples ways to do the same thing, duplications, same UE behavior tested multiple times, in multiple configurations, testing configurations that are not deployed)

  - For example: no intra-band CA if the spectrum can be covered with a single carrier with an 8K FFT based CBW

  - But still ensure completeness (avoid holes, missing fallbacks).

- Avoid names implying an order use explicit value instead (power class, bandwidth classes, frequency ranges)

  - For example: Use PC23 instead of PC3 or even add #Tx like 1T23, 2T29.

Proposal on improved CBW and BW parts support:

- Support of 2x 5G maximum CBW in the same band thanks to 8K FFT and single SCS per band/band-group.

- Enable variable BW support by design such that any CBW (1MHz granularity?) can be supported but only a limited set is measured.

Proposal on improved SU: Especially, there is opportunity for:

- FR1 30kHz and 60kHz SCS since a single SCS per band/band-group removes the RB alignment rule that penalize NRB for SCS other than the lowest.

- FR2-1 where the SU is fixed versus CBW at 95% (like for 4G where it was fixed at 90%) while in FR1 SU improves with increased CBW

  - For example, FR2 SU for 200MHz and 60kHz SCS is 95% while FR1 SU for 100MHz and 30kHz SCS is 98.3%)

- Higher CBW beyond the 4K FFT range assuming 8K FFT is the default.

- DFT-s-OFDM SU which is limited by the 2^x*3^y*5^z limitation.

  - For example, although for 20MHz CBW, 5G SU is larger in 5G compared to 4G for CP-OFDM with 106RBs, it is the same for DFT-s-OFDM at 100RB. Considering the addition of exponents of 7 would, for example allow 105RBs for DFT-s-OFDM.

Proposal on improved UL and DL UE performance:

- Target improved UL impairment and support: support of 256QAM for smartphone UEs with related improved impairments like carrier and image rejections that will improve EVM, IBE, MPR, A-MPR and self-desense for small duplex distance FDD bands.

- Target improved DL impairment and support: Support of 1024QAM for smartphones and associated improved image and IM2 responses and assuming better EVM on BS side.

# References

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN4#109