3GPP TSG-RAN WG4 Meeting #116bis	R4-2513066

# Prague, Czech Republic, 13th – 17th October 2025

Agenda item:			5.11.3

Source:	Nokia

Title:	On 4940-4990 MHz BS requirements

Document for:	Approval

# Introduction

During RAN4#116, the co-existence study phase for NR TDD 4.9GHz Band for US Operation has been concluded with the following agreement:

  - RAN4 commences work on defining requirements for the 4.9 GHz band with the following conclusion for the study phase

    - The existing environment has a broad diversity of incumbent 4.9 GHz and adjacent spectrum coexistence scenarios. These scenarios include non-3GPP systems, which are beyond the scope of RAN4 coexistence studies.

      - No 3GPP work beyond incorporating local regulation is required to address co-existence with non 3GPP out-of-band incumbents.

      - No 3GPP work beyond incorporating local regulation is required to address co-existence with in-band non 3GPP incumbents.

    - 3GPP relies on FCC descriptions of the band manager responsibility with respect to the coexistence aspects

  - With the above observations, RAN4 agrees to conclude the study phase.

This document addresses BS specification impact due to introduction of new 4.9GHz band.

# Discussion

This document provides impact to 38.104 due to introduction of new 4.9GHz band. It should be noted system parameters are not discussed in this document and addressed in [1]. Furthermore, it is assumed RAN task work on BS co-existence and co-location will progress in Q4 2025 with conclusion there will be no further impact to BS co-existence and co-location requirements whenever new band is introduced.

Taking into account aspects above and power limits allowing to define requirements for Local Area and Medium Range BS only, the only changes expected to 38.104 are to address regional regulations below:

§ 90.1215 Power limits.

Except as provided in paragraph (f) of this section, the transmitting power of stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this section.

(a)

(1) For base, mobile, and temporary fixed operations, the maximum conducted output power must not exceed:

| Table 1 to Paragraph (a)(1) |  |  |
| --- | --- | --- |
| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

(2) High power devices are also limited to a peak power spectral density of 21 dBm per one MHz. High power devices using channel bandwidths other than those listed above are permitted; however, they are limited to peak power spectral density of 21 dBm/MHz. If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi. However, high power point-to-point and point-to-multipoint operations (both fixed and temporary-fixed rapid deployment) may employ transmitting antennas with directional gain up to 26 dBi without any corresponding reduction in the maximum conducted output power or spectral density. Corresponding reduction in the maximum conducted output power and peak power spectral density should be the amount in decibels that the directional gain of the antenna exceeds 26 dBi.

(b) Low power devices are also limited to a peak power spectral density of 8 dBm per one MHz. Low power devices using channel bandwidths other than those listed above are permitted; however, they are limited to a peak power spectral density of 8 dBm/MHz. If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi.

(c) The maximum conducted output power is measured as a conducted emission over any interval of continuous transmission using instrumentation calibrated in terms of an RMS-equivalent voltage. If the device cannot be connected directly, alternative techniques acceptable to the Commission may be used. The measurement results shall be properly adjusted for any instrument limitations, such as detector response times, limited resolution bandwidth capability when compared to the emission bandwidth, sensitivity, etc., so as to obtain a true maximum conducted output power measurement conforming to the definitions in this paragraph for the emission in question.

(d) The peak power spectral density is measured as conducted emission by direct connection of a calibrated test instrument to the equipment under test. If the device cannot be connected directly, alternative techniques acceptable to the Commission may be used. Measurements are made over a bandwidth of one MHz or the 26 dB emission bandwidth of the device, whichever is less. A resolution bandwidth less than the measurement bandwidth can be used, provided that the measured power is integrated to show total power over the measurement bandwidth. If the resolution bandwidth is approximately equal to the measurement bandwidth, and much less than the emission bandwidth of the equipment under test, the measured results shall be corrected to account for any difference between the resolution bandwidth of the test instrument and its actual noise bandwidth.

(e) The ratio of the peak excursion of the modulation envelope (measured using a peak hold function) to the maximum conducted output power shall not exceed 13 dB across any 1 MHz bandwidth or the emission bandwidth whichever is less.

(f) The transmitting power of permanent fixed point-to-point and point-to-multipoint stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this paragraph (f). Moreover, applicants should request no more power than necessary for a particular use.

(1) The maximum equivalent isotropically radiated power (EIRP), as referenced to an isotropic radiator, must not exceed 55 dBW (85 dBm).

(2) For path lengths shorter than 17 kilometers, the EIRP shall not exceed the value derived from the following equation: New EIRP limit = 55 dBW—40*log(17/B) dBW, where B = the actual path length in kilometers.

Furthermore, to guarantee adjacent spectrum protection, the following emissions masks (L for low power and M for high power transmitters) apply [3]:

Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB.

(6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

# Conclusion

This document addresses BS specification impact due to introduction of new 4.9GHz band. It is proposed to agree on the expected changes to 38.104 provided in this document.

# Reference

[1] R4-2514046; Discussion on System parameters for US 4.9 GHz band; Nokia