3GPP TSG-RAN WG4 Meeting # 116bis                                                         R4-2513080

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:	6.11.4

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

# Introduction

In this contribution, we provide our views on core requirements of AI/ML BM use case.

# Discussion

## 2.1 Prediction report delay

Last meeting, RAN4 reached the following agreement on prediction report delay:

Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement

There are two remaining issues regarding prediction report delay. One is the value of inference delay. Another is the value of “T” in the so-called observation period for BM case 2.

The value of inference delay:

In RAN1#121, RAN1 reached the following agreement for DCI triggered semi-persistent or aperiodic report. The timeline of DCI triggered CSI report is shown in Fig. 1. In our understanding, Z3’+d’ is the inference delay required. No matter the report is aperiodic or semi-persistent or periodic, the inference delay is the same. So for inference delay, we propose to align with RAN1 agreement, i.e., inference delay = Z3’+d’.

Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively - Detailed values of d and d’ can be further discussed in UE feature.

Fig. 1

Proposal 1: For AI/ML BM, inference delay in the is Z3’+d’ as agreed and to-be-defined in RAN1.

The value of “T” in the observation period for BM case 2

It is not clear to us which observation period is to define here. For BM case 2, the input is measured results of set B of several occasions during observation window. The length of observation window is up to UE implementation. After UE receives the configuration of Set B and Set A, UE starts the measurement on Set B. Assuming the model input is RSRP results of 4 occasions, UE may not be able to make the prediction before the 1st report occasion as shown in Fig. 2 due to UE hasn’t gotten enough RSRP measurement results for the model input. In the 2nd report occasion, UE can report the valid predicted results for Set A and in the following report occasions, UE will update the predicted results based on the latest measurement results. It can be observed that there may be two kinds of delay to define. One is the delay of the first valid prediction report. Another is the updating period of the predicted results. It is up to UE implementation on the dimension of model input, i.e., length of observation window. When UE hasn’t measured enough occasions of Set B, UE can report invalid codepoints for the predicted occasions in the report until UE has measured enough occasions of Set B. For legacy L3 measurement, to guarantee measurement accuracy, RAN4 defined the measurement period requirements. But for AI/ML BM case 2, UE may adjust the observation window based on UE speed. We don’t think it is a good way to define the length of observation window. Then it is not possible to define the delay requirements for the first valid prediction report. The observation period to be defined is actually the updating period of prediction results. For the prediction, sliding window can be used. So we propose T=1. Then the observation period is exactly the same as legacy measurement period defined in 9.5.4 in 38.133. We don’t think we need a new chapter to define the observation period.

Fig. 2

Proposal 2: For AI/ML BM case 2, the length of observation window is up to UE implementation. No need to define the delay for the first valid prediction report for periodic and semi-persistent report.

Proposal 3: In the observation period agreed last meeting, T = 1 for AI/ML BM case 2.  No need to have a new chapter to capture the requirement of observation period as it is the same as legacy measurement period defined in 9.5.4 in 38.133.

## 2.2 Known TCI state condition

In the past few meetings, RAN4 discussed known TCI state conditions and reached the following progress.

Issue 2-3: TCI State Handling Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

The intention is to define known TCI state conditions which is captured in 38.133. Then when UE receives TCI state switch command, the switch delay will be different pending on the TCI state is known or not. For unknown TCI state, additional Rx beam sweeping during TCI state activation is needed. The current known conditions for TCI state are copied here for information.

38.133 8.10.2 Known conditions for TCI state The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.

According to the above conditions, a TCI state is known if the timeline is as below and SNR of the beam is not too low:

Since in the definition, there is no limit that L1-RSRP report is only for measurement. We think the above definition is applicable for the two cases “use SSB beams to predict CSI-RS beams” and “use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off ”.

But for the case “Use part of SSB beams to predict all the SSB beams”, the above definition should be updated as the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam. In this case, as long as UE reports the capability that Rx beam of predict beam is known, then the predicted beam is known. Last meeting, there were several versions on how to capture known TCI state conditions for AI/ML BM in the spec. For the sake of spec readability, we prefer to have a separate paragraph to capture the known TCI state conditions for the case “the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam”.

Proposal 4: If the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, then legacy known TCI state conditions are applicable.

Proposal 5: Use a separate paragraph to capture the known TCI state conditions for the case “the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam”. For this case, known TCI state conditions can be defined as

-	UE reports [TCI state known] in Capability X, and

-	TCI state switch command is received within 1280 ms upon transmitting the predicted L1-RSRP report for the target TCI state

-	The TCI state remains detectable during the TCI state switching period, and

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3 dB

      - 2.3 Impact of measurement error

### 2.3.1 General analysis

In previous meeting, the simulation settings were further discussed to evaluate the impact of measurement error in R4-2505152. In our simulations, Option 2 in R4-2505152 is adopted to get the distribution of BB error under different SNRs, i.e., TDL-C channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the mean and variance of BB measurement error under SNR= [-24, 10] dB with step 3dB at first (as in Fig. 3).

Fig.3

During inference, the way to add measurement error to the inputs of model inference is shown in Fig. 4:

- In SLS simulation, SINR of each Tx-Rx beam pair is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 10dB which is as guided in TR38.843 Table 6.3.1-1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam pair, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

Fig. 4 Method of adding measurement error

### 2.3.2 Simulation results using our dataset

As guided in R4-2508081, we focus on the scenario with 100% outdoor UEs to check the performance of different metrics. The simulation results are summarized as in Table 1 and 2. To check the predicted RSRP accuracy, regression model is adopted in the simulations. We also increased the size of training dataset to larger than 100,000 to check the impact of the size of training dataset.

Table 1. Simulation results for Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement

| error consideration |  |  | case 1 (6300) | case 3 (6300) |  |  | case 1 (105000) | case 3 (105000) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KPI | K | X | N.A. | BB error only | RF error only | BB+RF error | N.A. | BB+RF error |
| KPI 1 | 1 | 0 | 68 | 65 | 52 | 51 | 73 | 53 |
|  | 2 | 0 | 84 | 83 | 73 | 72 | 87 | 75 |
|  | 3 | 0 | 91 | 90 | 84 | 82 | 92 | 84 |
|  | 4 | 0 | 94 | 93 | 90 | 88 | 95 | 89 |
|  | 5 | 0 | 95 | 95 | 92 | 92 | 96 | 92 |
|  | 1 | 1 | 75 | 73 | 59 | 58 | 82 | 61 |
|  | 1 | 2 | 81 | 80 | 67 | 65 | 87 | 69 |
|  | 1 | 3 | 86 | 85 | 74 | 72 | 90 | 75 |
| KPI 2 | 1 | N.A. | 4.9 | 5.7 | 7.1 | 7.5 | 4 | 6.3 |
|  | 3 |  | 5.4 | 6 | 7.3 | 7.8 | 4.4 | 7 |
|  | 5 |  | 5 | 6.2 | 7.2 | 8 | 4.6 | 7 |

Table 2. Simulation results for Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B

| error consideration |  |  | case 1 (6300) | case 3 (6300) |  |  | case 1 (105000) | case 3 (105000) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KPI | K | X | N.A. | BB+RF error | RF error only | BB+RF error | N.A. | BB+RF error |
| KPI 1 | 1 | 0 | 82 | 79 | 57 | 55 | 85 | 57 |
|  | 2 | 0 | 94 | 92 | 79 | 76 | 95 | 78 |
|  | 3 | 0 | 96 | 96 | 87 | 86 | 98 | 87 |
|  | 4 | 0 | 98 | 98 | 92 | 92 | 99 | 92 |
|  | 5 | 0 | 98 | 99 | 95 | 94 | 99 | 94 |
|  | 1 | 1 | 90 | 88 | 65 | 62 | 94 | 66 |
|  | 1 | 2 | 97 | 93 | 72 | 70 | 97 | 73 |
|  | 1 | 3 | 98 | 96 | 79 | 76 | 98 | 80 |
| KPI 2 | 1 | N.A. | 2.5 | 2.7 | 5.3 | 5.6 | 1.9 | 4.9 |
|  | 3 |  | 3.5 | 4.1 | 7.2 | 7.6 | 2.4 | 6.8 |
|  | 5 |  | 4.4 | 5.2 | 9 | 8.7 | 3.1 | 7.8 |

It can be observed that measurement error has big impact on the performance. Both BB error and RF error should be considered when define the requirements and test case.

Proposal 6: Both BB error and RF error should be considered when define the requirements and test case.

The prediction accuracy is improved a bit by increasing the training dataset size from 6300 to 105,000.

Observation 1: The prediction accuracy is improved a bit by increasing the training dataset size.

      - 2.4 Comparison with other dataset

We also simulated the performance using vivo’s dataset. As vivo’s dataset size is much larger than ours. So we simulated the results with equal size (6300, same size of ours) and full size (use all the data). In the simulation, we only compared the results not adding measurement error. The simulation results are summarized as in Table 3.

Table 3

| KPI | K | X | MTK’s dataset | reference dataset |
| --- | --- | --- | --- | --- |
| KPI 1 | 1 | 0 | 73 | 70 |
|  | 2 | 0 | 87 | 86 |
|  | 3 | 0 | 92 | 92 |
|  | 4 | 0 | 95 | 95 |
|  | 5 | 0 | 96 | 96 |
|  | 1 | 1 | 82 | 80 |
|  | 1 | 2 | 87 | 86 |
|  | 1 | 3 | 90 | 90 |
| KPI 2 | 1 | N.A. | 4 | 3.4 |
|  | 3 |  | 4.4 | 4.2 |
|  | 5 |  | 4.6 | 4.2 |
| KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB KPI 2: Absolute RSRP accuracy - 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams |  |  |  |  |

Comparing the results, it can be observed that Top-K beam prediction accuracy are similar using our dataset and vivo’s dataset with full size. But Top-1 + XdB, the prediction accuracy using vivo’s dataset is higher than ours. The absolute RSRP accuracy using vivo’s dataset is also better than ours. By comparing the distribution of RSRP difference between different beams (1th gap = Best beam – 2nd best; 3th gap = best beam – 4th best, etc.) in vivo’s dataset and ours, it can be observed that RSRP difference of the best and 2nd best in vivo’s dataset is smaller than ours. We think that may be the reason why the prediction accuracy of Top-1 + XdB using vivo’s dataset is higher than ours.

RAN4 need to discuss how to handle the performance divergence between different datasets. There can be several solutions. E.g., pick the metric with the least divergence or add enough margin or define a common dataset. We are not sure which way is better.

| vivo | MTK |
| --- | --- |

Observation 2: Using different dataset, for some metric, the accuracy performance are similar, while for some other metric, the accuracy performance may have some divergence.

      - 2.5 Metrics

### 2.5.1 Metric: Top-K plus xdB margin

In RAN4#114bis, RAN4 agreed to use Top-K plus xdB margin as the metric for beam ID prediction. The value of K and x is supposed to be discussed further.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%): the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Meanwhile, RAN1#120bis agreed to use Top-K/M for Type 1 option 2 monitoring.

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

As shown above, the metric agreed in RAN4 is not totally aligned with the metric agreed for monitoring in RAN1. One of the questions is whether to align with the metric for monitoring. We prefer to have aligned metric as monitoring, then the defined accuracy requirements and test cases can be used as a reference for NW to decide whether the model is working well. Otherwise, the to-be-defined accuracy requirements and test cases can only be used to test the performance of model. Given that there is slightly difference between the metrics agreed in RAN4 and RAN1. We prefer the defined accuracy requirements and test cases can be used as a reference for NW to decide whether the model is working well, i.e., the metric is also for monitoring. As RAN1 has concluded the metric for monitoring, we propose to align the metric discussed in RAN4 with RAN1.

Proposal 7: Prefer that the metric to be used in RAN4 is also used for monitoring, i.e., x shall be 0.

### 2.5.2 Metric: absolute/relative RSRP requirements

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

Last meeting, RAN4 reached the above agreement on absolute/relative RSRP requirements for predicted RSRP. One of the remaining issue is how to set SNR side condition. The legacy L1 measurement requirements are defined at SNR= -3dB and the requirements are applicable under the side condition that SNR≥-3dB. In the LLS simulation, we can set SNR to -3dB and get the accuracy requirements under SNR= -3dB. But for AI BM, we are confused on how to set SNR side condition as SLS simulations are performed. We checked SNR distribution of the strongest beam in set A and found that there is no data with SNR of the strongest beam in set A equal to -3dB in our generated dataset as shown in Fig.5. We once tried to decrease the RSRP artificially to make SNR of the strongest beam in set A equal to -3dB. But we received some offline comments that this may lead to a different scenario and is not a good way. Considering the above fact, we think the RSRP accuracy requirements for the strongest beam can be defined by filtering out the data with SNR of the strongest beam in set A no less than -3dB.

Fig. 5

Proposal 8: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the data with SNR of the Top-1 beam in set A no less than -3dB.

For none Top-1 beams, it can be observed that the absolute accuracy of each predicted beam varies as shown in Table 1 (KPI 2). We checked the CDF of the RSRP/SINR difference between the (x+1)th strongest beam and the strongest beam (E.g., 1th gap: strongest beam - 2nd strongest beam, 3th gap: strongest beam - 4th strongest beam) in Fig. 6. As shown by figure on the right in Fig. 6, there is a 60% probability that SINR of the 2nd strongest beam is lower than the strongest beam, i.e., SINR difference >0dB. Such a SINR difference will lead to the difference in predicted absolute RSRP accuracy of top-1 beam and other beams.

Observation 3: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams.

Fig. 6

Due to limited time, we don’t have time to check the predicted absolute RSRP accuracy of none top-1 beams if excluding those datasets with low SNR. We can further check it to see whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions, e.g., SNR≥-3dB.

Proposal 9: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions.

      - 2.6 Model monitoring

In the past meetings, there were some discussions on whether and how to define accuracy requirements for UE-assisted performance monitoring. For UE-assisted performance monitoring, UE reports the beam prediction accuracy RS-PAI (prediction accuracy indicator) Np, where Np is the total count of prediction instances that meets the requirement among N monitoring occasions. Based on the predicted and measurement results at N monitoring occasions, it is possible that UE reports any Np value which is no larger than N. If UE reports “Np=0”, we can not say it is not accurate. Maybe it is due to the model in use is not suitable. If UE reports “Np=0”, we can not conclude it is accurate. It just reflects that the model in use works well in the current scenario. So it is not possible to define the accuracy requirements for model monitoring for AI/ML BM.

Someone may argue that RAN4 may test whether the report value is correct or not. We don’t think it is necessary. The report value is derived by comparing the measured results with predicted results. Both the measurement and prediction are performed at UE. UE just does some comparison. We don’t think UE will report a wrong value. That is, even RAN4 design a test case, no UE will fail the test case.

Observation 4: The report result for model monitoring is to reflect whether the model in use works well in the current scenario.

Proposal 10: It is not possible to define the accuracy requirements for model monitoring for AI/ML BM.

Proposal 11: It is not necessary to define test cases for model monitoring for AI/ML BM, as no UE will fail the test.

      - 2.7 Analysis on Multiple AoA Test setup for AI/ML BM

RAN4#116 reached the following progress on test system for AI/ML BM. Although we still think single AoA test system is more workable, we can focus on the evaluation of multi-AoA test system this meeting as guided.

Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing) Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. Take CDL-C Umi as one example Other channel models can be further discussed

We believe that companies are supposed to simulate using the final agreed simplified CDL channel if confirmed workable. To carry out the simulations, RAN4 needs to discuss how to simulate the simplified CDL. We think at least the following aspects should be discussed.

How to simulate the channel “from BS to clusters” and the channel “from clusters to UE”

In the simulation, it is the channel from BS to UE simulated. As shown in Fig.7, the channel between BS and UE includes two parts in CDL channel model: BS  Clusers and Clusters  UE.  As enhanced IFF system with 4 probes is used as baseline and there are far more than 4 clusters in the CDL channel model, it is obvious that how to simply the part “from clusters to UE” should be discussed. In R4-2511597, several methods have been proposed on this. But it seems that the contribution focused on how to simplify the channel “from clusters to UE” and didn’t touch the part “from BS to clusters”. To get the whole channel, how to simulate the part from BS to clusters should also be discussed.

Fig.7

Observation 5: CDL channel includes two parts “from BS to clusters” and the channel “from clusters to UE”.

Proposal 12: Discuss how to simulate the channel “from BS to clusters” in the simplified CDL-channel as well.

How to emulate UE movement?

In general, the channel that the chamber simulates is link level channel, which is just one realization, i.e., for UE at a certain location. For AI/ML BM test, it is necessary to check the performance at different UE locations in a cell. Especially for AI/ML BM case 2 (temporal prediction), it is necessary to emulate UE movement which is important for BM case 2 during the test. In our understanding, there may be two methods. One is through UE rotation. But according to the information provided by TE vendors [2-5], it may be difficult and costly to support frequent and flexible UE rotation, i.e., rotation 2o, 5o. Now, the time needed to rotate UE is about 1s, which is not acceptable during one iterative test.

Another method may be through adjusting TE transmission power at each probe, which is similar as single AoA test system. Using this method, RAN4 needs to conclude on the methodology of simplifying the CDL-channel. In the simulation, the procedures will be as the below:

- Cast the UE in the cell randomly.

- Generate the channel between BS and UE according to 38.901, including path-loss, shadowing, and small-scale fading.

- Simplify small scale fading between BS and UE based on the to-be-agreed simplification method.

- Calculate received RSRP of each beam.

Proposal 13: Discuss how to emulate UE movement in multiple AoA test systems.

Observation 6: It is not workable to emulate UE movement through rotating UE during the test as the time needed to rotate UE is about 1s.

Proposal 14: Evaluate whether it is workable to emulate UE movement through adjusting TE transmission power at each probe in multiple AoA test systems.

Proposal 15: The procedures to emulate UE movement using simplified CDL channel in the simulation will be:

- Cast the UE in the cell randomly.

- Generate the channel between BS and UE according to 38.901, including path-loss, shadowing, and small-scale fading.

- Simplify small scale fading between BS and UE based on the to-be-agreed simplification method.

- Calculate received RSRP of each beam.

How to get training dataset?

Another issue is how to get the training dataset. In computer simulations, a pre-defined UE radiation pattern can be used to generate the training dataset and inference dataset. But in the test, we are not clear about how to get the training dataset. We think there may be two ways. One is collecting the training data in the chamber for each test UE before training. This method is time-consuming and not preferred. Another is to generate the training dataset assuming a certain UE radiation pattern. We evaluated the performance of models trained with a UE antenna radiation pattern (case 2) and inference on UEs with varied antenna radiation patterns (case 1, 3, 4) as shown in Fig. 8. It can be observed that AI/ML BM Tx beam prediction model generalize well for different antenna radiation patterns. Therefore, we think it is enough to use a specified UE radiation pattern during the test. We guess, even simplified CDL channel is adopted, we may have similar observation.

Fig. 8

Proposal 16: Discuss how to get training dataset if to use multiple AoA test systems:

- Alt 1: collect the training data in the chamber for each test UE before training

- Alt 2: generate the training dataset assuming a certain UE radiation pattern through simulation

Dynamic range

In early releases, dynamic range of IFF and DFF test systems had been analysed. Since several years have passed, there may be some new progress. But we are not experts, so here we just copy the previous results in TR38.810 here for information.

Table 4

| Test methods | Wanted signal (dBm/SCS) at reference point: upper bound | SNR at baseband (dB): upper bound | SNR at baseband (dB): upper bound – 30dB |
| --- | --- | --- | --- |
| IFF, peak fine beam | -83.5 | 25.1 | -4.9 |
| IFF, peak rough beam | -83.5 | 18.1 | -11.9 |
| IFF, non-peak fine beam | -83 | 13 | -17 |
| IFF, non-peak rough beam | -83 | 6 | -24 |
| DFF, peak fine beam | -83.8 | 24.8 | -5.2 |
| DFF, peak rough beam | -83.8 | 17.8 | -12.2 |
| DFF, non-peak fine beam | -83.3 | 12.7 | -17.3 |
| DFF, non-peak rough beam | -83.3 | 5.7 | -24.3 |

If to use multiple AoA test system, the Rx beam will be tuned by UE and it is not possible to always use the peak fine beam. So for multiple AoA test system, we only need to check the highlighted rows in Table 4. It can be observed that the highest SNR is only 13dB, which is very low.

Observation 7: Multiple AoA test systems cannot use peak fine beam direction only.

Considering the upper bound of SNR with rough beam are much lower than that with fine beam as shown in Table 1, fine beam should be used in AI/ML BM test to get a larger dynamic power range. Next we will only focus testing using fine beam.

Observation 8: The upper bound of SNR with multiple AoA test system is 13dB.

Proposal 17: Take the upper bound of SNR with multiple AoA test system into consideration during simulation.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: For AI/ML BM, inference delay in the is Z3’+d’ as agreed and to-be-defined in RAN1.

Proposal 2: For AI/ML BM case 2, the length of observation window is up to UE implementation. No need to define the delay for the first valid prediction report for periodic and semi-persistent report.

Proposal 3: In the observation period agreed last meeting, T = 1 for AI/ML BM case 2.  No need to have a new chapter to capture the requirement of observation period as it is the same as legacy measurement period defined in 9.5.4 in 38.133.

Proposal 4: If the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, then legacy known TCI state conditions are applicable.

Proposal 5: Use a separate paragraph to capture the known TCI state conditions for the case “the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam”. For this case, known TCI state conditions can be defined as

-	UE reports [TCI state known] in Capability X, and

-	TCI state switch command is received within 1280 ms upon transmitting the predicted L1-RSRP report for the target TCI state

-	The TCI state remains detectable during the TCI state switching period, and

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3 dB

Proposal 6: Both BB error and RF error should be considered when define the requirements and test case.

Observation 1: The prediction accuracy is improved a bit by increasing the training dataset size.

Observation 2: Using different dataset, for some metric, the accuracy performance are similar, while for some other metric, the accuracy performance may have some divergence.

Proposal 7: Prefer that the metric to be used in RAN4 is also used for monitoring, i.e., x shall be 0.

Proposal 8: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the data with SNR of the Top-1 beam in set A no less than -3dB.

Observation 3: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams.

Proposal 9: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions.

Observation 4: The report result for model monitoring is to reflect whether the model in use works well in the current scenario.

Proposal 10: It is not possible to define the accuracy requirements for model monitoring for AI/ML BM.

Proposal 11: It is not necessary to define test cases for model monitoring for AI/ML BM, as no UE will fail the test.

Observation 5: CDL channel includes two parts “from BS to clusters” and the channel “from clusters to UE”.

Proposal 12: Discuss how to simulate the channel “from BS to clusters” in the simplified CDL-channel as well.

Proposal 13: Discuss how to emulate UE movement in multiple AoA test systems.

Observation 6: It is not workable to emulate UE movement through rotating UE during the test as the time needed to rotate UE is about 1s.

Proposal 14: Evaluate whether it is workable to emulate UE movement through adjusting TE transmission power at each probe in multiple AoA test systems.

Proposal 15: The procedures to emulate UE movement using simplified CDL channel in the simulation will be:

- Cast the UE in the cell randomly.

- Generate the channel between BS and UE according to 38.901, including path-loss, shadowing, and small-scale fading.

- Simplify small scale fading between BS and UE based on the to-be-agreed simplification method.

- Calculate received RSRP of each beam.

Proposal 16: Discuss how to get training dataset if to use multiple AoA test systems:

- Alt 1: collect the training data in the chamber for each test UE before training

- Alt 2: generate the training dataset assuming a certain UE radiation pattern through simulation

Observation 7: Multiple AoA test systems cannot use peak fine beam direction only.

Observation 8: The upper bound of SNR with multiple AoA test system is 13dB.

Proposal 17: Take the upper bound of SNR with multiple AoA test system into consideration during simulation.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2411177, Testability and interoperability issues for beam management, Korea Testing Laboratory, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 # 112, Aug. 2024.