3GPP TSG-RAN WG4 Meeting #116bis				                 R4-2513123

Prague, Czech, Oct. 13-17, 2025

Source:	Sony

Title:	Initial views on general and UE RF of 6GR

Agenda item:	8.4

Document for:	Approval

# Background

In RAN4#116, the following scope for the general and UE RF of 6G was proposed [1]: The scope includes non-spectrum and non-AI UE RF, spectrum aggregation framework, UE RF-related coverage and energy efficiency, and other joint UE-BS RF issues.

Our high-level view of the 6GR system parameters and device type has been discussed in the companion paper [2]. To avoid duplicate discussion here, detailed capability/parameters for each device type are captured in the appendix of the paper, where we propose three device types as a starting point, and use those terminologies in the rest of the paper:

- Massive IoT device (e.g., tracker, wearable devices, meters)

- Broadband device with a limited form factor (e.g., smartphone, glass-type XR)

- Ultra Broadband devices (e.g., FWA/CPE, HMD XR)

In this contribution, we focus on sharing some selected issues that are particularly related to the UE RF aspect of 6GR.

# SAW-less HD-FDD on massive IoT front-end assumptions

From the duplex aspect, HD-FDD is the most important mode for 6G massive IoT devices, which offers improved power efficiency, reduced device complexity, and enhanced global scalability by eliminating band- or regional-specific SAW/BAW duplexers/filters from the RF front end of the devices.

However, based on the lessons learned from 5G NR (e)Redcap implementation, the full advantages of HD-FDD are not realised since band-specific SAW filters would still be required to fulfil the band-to-band protections (-50 dBm), especially for selected bands in sub 1GHz where the offset between the aggressor band and victim band are small (below 12 MHz-ish). Therefore, achieving a true SAW-less HD-FDD design is a key consideration for reducing RF domain complexity in 6G massive IoT [4][5], which can offer:

- A true single stock-keeping unit (SKU) design for global operations is possible, allowing for true economies of scale.

- Additionally, a fully SAW-less HD-FDD design can achieve a reduced chip footprint and a smaller IoT module.

- Last, but not least, the insertion loss on Tx and Rx chains can be reduced, and the power efficiency of the system can be enhanced.

The design of a true SAW-less device is illustrated in Fig. 1. A similar design principle has been widely adopted in 4G IoT commercial production, e.g., cat-M1 and NB-IoT, thanks to 4G IoT's even smaller CBW. However, it is not the case in 5G NR as larger BW needs to be supported, even for (e)Redcap devices, which is one of the factors that creates a significant implementation feasibility difference between 4G IoT and 5G (e)Redcap devices in terms of SAW-less aspect.

Proposal 1: 6G massive IoT design shall target a truly SAW-less design as baseline.

Fig. 1. Comparison of massive IoT device with SAW filter/duplexer design vs a fully SAW-less design.

The impact on the standard to achieve a true SAW-less implementation is still pending on the basic system parameters required for 6G massive IoT devices. For example, if the device only needs to support a small bandwidth comparable to legacy devices, e.g., 3MHz, it can be designed naturally as a true SAW-less design without any further impact. If a large bandwidth needs to be supported, especially in the uplink direction, other approaches, e.g., limited uplink RB length, allowed power backoff for certain bands and/or certain RB allocations, may need to be considered instead. Overall, we see that the impact is very limited and would not jeopardize the overall 6GR design, especially for 6GR broadband services.

Observation 1: The impact to enable a true SAW-less design is very limited and will not jeopardize the overall 6GR design.

Observation 2: with a limited uplink bandwidth or RB length, e.g., 3MHz, the 6G massive IoT devices can be implemented as SAW-less natively, while some power backoff may be needed if larger uplink bandwidth needs to be supported.

Proposal 2: Depending on the system parameters choice, RAN4 shall study the most proper approach to address the design of true SAW-less.

# EVM relaxation

EVM relaxation is one of the items that has been agreed upon in RAN#109 to be studied under the 6G track but can possibly be applied to 5G NR if feasible [4]. The aim of EVM relaxation is to allow UE to transmit on each modulation order with relaxed EVM to lower the required MPR. It is worth pointing out that such a scheme is mostly effective with high-order modulation schemes, e.g., 256QAM and 1024QAM, as the UE MPR is dominated by EVM rather than emission level.

Observation 3: EVM relaxation is effective in reducing MPR for high-order modulations.

From the UE perspective, the necessary EVM relaxation needs to be investigated with a certain MPR reduction value. Considering that such relaxation would inevitably place a greater burden on network decoding, it must guarantee a sufficient MPR reduction gain from the UE side. Therefore, RAN4 needs to identify how much EVM relaxation could result in a sufficiently high-performance gain in terms of MPR first. In our view, at least a 1.5 dB MPR reduction shall be ensured; otherwise, it may not be meaningful to consider any EVM relaxation.

Proposal 3: RAN4 shall study the EVM relaxation level needed to achieve a meaningful MPR reduction, e.g., larger than 1.5 dB.

From the BS side, how to demodulate the uplink data with a relaxed EVM level needs to be studied as well. It is agreed that RAN4 will study both AI-based and non-AI-based solutions. Considering the overall AI framework of 6GR is still in its very early stage, it is proposed that RAN4 focuses on non-AI-based solutions at this stage. It is necessary to clarify how BS can decode the data with a relaxed EVM value so that proper requirements and tests can be specified.

Proposal 4: RAN4 focuses on a non-AI-based approach and aims to achieve a common understanding of how BS can decode data with a relaxed EVM, so that proper requirements and tests can be specified

Supposing it is feasible to support a relaxed EVM for a given modulation order, RAN4 may consider defining the minimum requirements for EVM in 6GR from the beginning, aiming for a more relaxed value than 5G NR, to enable better uplink coverage for high order modulations.

Proposal 5: If approved as feasible, RAN4 may consider relaxing the EVM requirement for higher-order modulation schemes in 6GR compared to 5G NR.

# Antenna configuration in “FR3”

One of the main differences between 5G NR and 6GR is the adoption of spectrum between FR1 and FR2, which is commonly referred to as “FR3” for now. As a transition spectrum from sub-6 GHz to mmWave, the antenna architecture, especially on the UE side, has not been agreed upon. Currently, in FR1, multiple single antennas are integrated to achieve high spatial diversity. On the other hand, multiple antenna arrays are used in FR2, mainly to target a high gain to mitigate the high pathloss in mmWave.

In FR3, for devices like smartphones, considering the form factor limitation, it will be very difficult to fit any array structure. Therefore, multiple single-antenna systems, as in FR1, are a more promising antenna architecture to consider, at least for broadband devices with limited form factors. Regarding the number of antennas, it needs to be balanced between the implementation limits and the system performance. From the UE implementation aspect, it needs to be taken into account that existing antennas, at least for the current FR1 bands, need to be supported. A schematic drawing of a typical antenna topology in FR1 is shown in Fig. 2. It can be observed that adding additional antennas is very challenging due to the limited space. The possibility of adding more antennas for FR3 can be achieved by creating more resonant structures in higher frequencies on existing antennas or reusing some existing structures in the mobile phone as antennas.

Observation 4: The space in a smartphone for placing additional antennas in FR3 is extremely limited; therefore, reusing the physical structure of the existing antenna for FR3 might be necessary.

Fig. 2: a typical antenna topology for a smartphone in FR1

Meanwhile, from a system perspective, based on the investigation captured in Section 6.3.1 of TR38.820, the typical rank of the channel at 15 GHz is in the range of about 2-4 under an indoor environment as shown in Fig. 3. In general, it can be expected that the rank of the channel will be reduced with higher frequencies, considering the higher beamforming gain on the BS side and also the increased shadowing losses in the channel. Therefore, from a system perspective, a UE with 4 antennas can almost achieve optimal system performance.

Observation 5: The achievable rank at 15 GHz would be in the range of 2-4, implying that a UE with 4 antennas can almost exploit the full rank of the channel.

Fig .3: The measurement of channel rank with the UE antennas prototype at 15 GHz in TR38.820

Therefore, considering the factors above, it is proposed to four antennas in FR3 with an FR1-like antenna architecture for a broadband device with a limited form factor, e.g., smartphones.

Proposal 5: It is proposed to consider four antennas in FR3 with an FR1-like antenna architecture for a broadband device with a limited form factor, e.g., smartphones, as the starting point.

# Improved PA modeling

In 5G NR, a common PA model has been typically used to derive the MPR for smartphones. However, as already identified by companies [5], this model can be improved based on the commercial device's performance. Therefore, it is proposed that RAN4 study an improved PA model in the 6GR study, at least for smartphone-type devices.

Proposal 6: RAN4 shall study an improved PA model for 6GR smartphone-type devices.

In addition, considering the diverse device types that 6GR needs to support, the PA models may differ between different device types. For example, Massive IoT PA may be optimized against a narrower bandwidth, while a smartphone PA needs to cover a much wider range of bandwidths. In addition, more advanced semiconductor technologies, e.g., GaN PA, may be considered for FWA/CPE devices, which focus on high performance and high-power applications. Therefore, it is proposed that RAN4 should consider different PA models for different device types.

Proposal 7: RAN4 shall consider different PA models for different device types.

# LP-WUR

The low-power wake-up signal and receiver (LP-WUS and LR) were specified in 5G NR Rel-19. Energy efficiency and power saving are considered key motivations for the 6GR design, and it is proposed that the LP-WUS design and LR be considered from day-1 of 6GR, making it more likely to be deployed in the field. In addition, it is proposed to study if more RRM measurement can be offloaded to 6G LR design, to allow more usage scenarios of such a power saving feature.

Proposal 8: 6G to support LP-WUS and LR from day-1, with consideration of more RRM measurement being offloaded to 6G LR design

# Miscellaneous considerations in UE RF of 6GR

Among the lessons learned from 5G NR, UE behavior in terms of actual output power and the power control framework is one aspect that has been debated frequently in RAN4. Multiple factors in the power domain that a UE applies in the field are unknown to the network, for example, the A-MPR value. These factors may cause more complications and a more conservative design of the network scheduler. Therefore, RAN4 may need to collaborate with RAN1 to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power.

Proposal 9: RAN4 may need to collaborate with RAN1 in 6GR to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power.

The MSD framework for CA is also one area that can be improved in the 6GR design as well. Currently, RAN4 spends a significant amount of time defining the MSD for every band combination; however, the fact is that these values are usually far from the actual implementations. In 6GR, to make MSD useful for the network, a more dynamic MSD reporting mechanism could be studied. Whether to still define any MSD minimum requirements can also be further discussed.

Proposal 10: RAN4 shall study improving the MSD requirement framework with possibilities to support a dynamic MSD reporting mechanism, making the feature useful for the network.

The HPUE framework is also something that warrants further consideration in 6GR. Currently, RAN4 defines requirements on a case-by-case basis to cover all possible implementations for HPUE, such as 1Tx, 2Tx …. or even multiple TX with different output power. In our view, such an HPUE framework should be reconsidered entirely in the 6GR design. For example, RAN4 can define unified requirements for each power class, which are agnostic to the UE Tx implementation. The feasibility of such an approach needs to be studied in RAN4.

Proposal 11: RAN4 shall study whether it is feasible to define a unified requirement for each power class that is agnostic to the UE Tx implementation, thereby simplifying the HPUE requirement framework.

# Conclusion

In this contribution, we share our intial views on selected topics regarding general and UE RF aspect in  6GR study

Observation 1: The impact to enable a true SAW-less design is very limited and will not jeopardize the overall 6GR design.

Observation 2: with a limited uplink bandwidth or RB length, e.g., 3MHz, the 6G massive IoT devices can be implemented as SAW-less natively, while some power backoff may be needed if larger uplink bandwidth needs to be supported.

Observation 3: EVM relaxation is effective in reducing MPR for high-order modulations.

Observation 4: The space in a smartphone for placing additional antennas in FR3 is extremely limited; therefore, reusing the physical structure of the existing antenna for FR3 might be necessary.

Observation 5: The achievable rank at 15 GHz would be in the range of 2-4, implying that a UE with 4 antennas can almost full rank of the channel.

Proposal 1: 6G massive IoT design shall target a truly SAW-less design as baseline.

Proposal 2: Depends on the system parameters choice, RAN4 shall study the most proper approach to address the design of true SAW-less.

Proposal 3: RAN4 shall study the EVM relaxation level needed to achieve a meaningful MPR reduction, e.g., larger than 1.5 dB.

Proposal 4: RAN4 focuses on a non-AI-based approach and aims to achieve a common understanding of how BS can decode data with a relaxed EVM, so that proper requirements and tests can be specified

Proposal 5: If approved as feasible, RAN4 may consider relaxing the EVM requirement for higher-order modulation schemes in 6GR compared to 5G NR.

Proposal 6: RAN4 shall study an improved PA model for 6GR smartphone-type devices.

Proposal 7: RAN4 shall consider different PA models for different device types.

Proposal 8: 6G to support LP-WUS and LR from day-1, with consideration of more RRM measurement being offloaded to 6G LR design

Proposal 9: RAN4 may need to collaborate with RAN1 in 6GR to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power.

Proposal 10: RAN4 shall study improving the MSD requirement framework with possibilities to support a dynamic MSD reporting mechanism, making the feature useful for the network.

Proposal 11: RAN4 shall study whether it is feasible to define a unified requirement for each power class that is agnostic to the UE Tx implementation, thereby simplifying the HPUE requirement framework.

# References

- R4-2511652		Plan on RAN4 6G discussion and meeting arrangement in Rel-20	RAN4 Chair (Apple)

- R4-2513122		Initial views on 6GR system parameters	Sony

- RP-252912		Revised SID: Study on 6G Radio		NTT DOCOMO, CMCC, AT&T, Vodafone.

- RP-252952 	  	New WID: UE RF enhancements for NR FR1, Phase 5	Moderator (vivo)

- R4-2506896		Observation on UE simulation assumption for A-MPR	KDDI

# Appendix 6GR device type and associated device capability

Table I. 6GR device type and associated device capability

|  | Massive IoT device, e.g., tracker, wearable devices, meters | Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices | Ultra broadband device e.g., FWA/CPE, Vehicle, HMD type XR devices. |
| --- | --- | --- | --- |
| Design considerations | - Aim for devices capability above 4G IoT below 5G NR, - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap) | - Improved data rate without adding - Limited complexity additional to legacy devices (e.g., 5G eMMB) - considering the formfactor limitation | - Aim to focus on the implementation assumption can support fullfil the extraordinary performances - no major constraints due to device formfactor with realistic implementation limitations |
| Frequency range | Focus on frequency bands < 3 or 4 GHz | FR1, FR2-1, “FR3” | FR1, FR2-1, “FR3” |
| Maximum UE CBW | 3MHz | 3MHz in FR1 | 3MHz in FR1 |
| Maximum UE CBW (single CC) | UL: 3MHz/ DL: TBD | - 100 MHz in FR1 (same as 5G NR) - 200 MHz in FR3 - 400 MHz in FR2-1(same as 5G NR) | - 200 MHz in FR1 - 400 MHz in FR3 - 400 MHz in FR2-1 |
| Duplex mode | Focus on HD-FDD, FDD/TDD can be supported | FDD/TDD | FDD/TDD |
| Number of antennas | 1T/1R | FR1&FR2-1 antenna assumption to be the same as in 5G NR, 2T/4R in FR3. | 2T/8R in FR1 and FR3 FR2-1 antenna assumption to be the same as in 5G NR. |
| Default power class | 23 dBm | 23 dBm in FR1 26 dBm in FR3 Considering enhancement on PC3 in FR2-1 | 26 dBm in FR1 31 dBm in FR3 PC4 in FR2-1 |
| Mobility | Idel mode mobility as baseline | Connected mode mobility | Connected mode mobility |
| Maximum Modulation order | 64QAM UL/64QAM DL | 256 QAM UL/1024 QAM DL in all frequency range | 512 QAM UL/1024 QAM DL in all frequency range |