3GPP TSG-RAN WG4 Meeting #116bis			                        R4-2513129

Prague, CZ, October 13rd – 17th, 2025

Agenda item:	8.5

Source: 	CMCC

Title: 	Discussion on BS RF and coexistence

Document for:	Discussion

# 1. Introduction

In RAN plenary #108 meeting, the SID for 6G has been approved. 6G-related RAN4 work will officially begin at this meeting. According to the agenda released by the RAN4 chair, the 6G activities have been split into multiple parts.

This contribution will focus on BS RF aspects and coexistence for future 6G work.

- Discussion

### 2.1 General consideration on 6GR BS

During last meeting, RAN has achieved the following agreement:

Agreement - Study and identify the lessons learned from NR duplex modes - On 6GR duplexing study, RAN1 considers at least following duplex types - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD - Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded

gNB semi-static SBFD has been one of the duplex types to be considered in 6GR. Over the past releases and in the R20 Work Item, SBFD has been a key topic and has received supports from many operators and vendors. The study in R18 verified the uplink gain and feasibility of SBFD. In Rel-19, requirements for SBFD BS in SBFD-TDD scenarios were defined, and in Rel-20 it is proposed to specify BS TX and RX RF requirement for SBFD-capable BS for the scenario of SBFD BS to SBFD BS adjacent channel coexistence.

Observation 1: Research related to SBFD BS has been continuously conducted in recent releases and has received supports from numerous operators and BS vendors.

For operators, the deployment of SBFD macro stations relies on large bandwidth frequency bands. A part of objectives of the study on 6G Scenarios and requirements are listed as below:

e) Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. f) Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

It is easy to observe that all the frequency bands within the above frequency range have relatively large bandwidths, especially the frequency around 7 GHz, e.g. n104. Therefore, taking into account the existing technological advancements as well as the spectrum of 6GR, SBFD BS should be supported in the 6G day 1 version.

Observation 2: Frequency around 7 GHz has the large bandwidth, which is beneficial for the deployment of SBFD BS.

Proposal 1: SBFD BS should be defined in the 6G Day1.

### 2.2 RF requirements for 6GR BS

For 6GR BS, since the decision on the new IMT frequencies in WRC-27 has not been finalized yet, the discussion on the 6GR band may currently focus on the frequency band around 7GHz and some bands that have already deployed LTE or NR. For the frequency bands that have already deployed LTE or NR, when refarming them to 6GR, it can be considered to conduct the discussion based on the existing NR requirements as the baseline.

Proposal 2: The RF requirements for 5G NR BS could be set as starting point for the 6GR refarming bands.

Although n104 has been identified as an IMT spectrum regional at WRC-23, it has not actually been deployed for NR BSs in this band. Therefore, it can be discussed together with other 6GR frequencies. Therefore, it is recommended to conduct research on the 6GR BS RF requirements using the frequency around 7GHz. Considering the introduction of new scenarios for 6GR, Existing RF requirements should be enhanced to meet the demands of new deployment scenarios.

Proposal 3: Conduct research on the 6GR BS RF requirements using the frequency around 7GHz, and consider enhancing the existing RF indicators, e.g. EVM.

Regarding the types of 6GR BS, for FR1, the current 1-C, 1-H and 1-O can be considered as a starting point. Among these BS, the 1-H BS defines two sets of RF requirements: conduction requirements and OTA requirements. From the perspective of cost reduction and BS energy saving, the conduction requirements for 1-H can be appropriately relaxed to create more flexibility for the selection of BS components. Similarly, from the perspective of flexibility, for 6G pico stations, it can be considered to reuse the components of Wi-Fi devices.

Proposal 4: Conduction requirements relaxation could be considered in 6G day-1.

### 2.3 MSR for 6GR BS

With the completion of the 6GR day1 study, the deployment of 6GR BSs will also begin gradually. For many regions, during the deployment period of 6GR, LTE and NR BSs will still exist to ensure communication service, especially NR BSs. To ensure the communication experience of users during the 6GR deployment period, while maintaining the flexibility of spectrum usage and reducing the deployment costs of operators, it is better to introduce MSR (Multi-Standard Radio) for 6GR BSs. The introduction of MSR can accelerate the construction of 6GR networks, avoid duplicate BS deployment and coverage gaps, and also provides hardware support for MRSS.

Observation 3: MSR can provide hardware support for MRSS.

For operators, the demand for MSR between NR and 6GR is more necessary. On the one hand, the large-scale deployment of NR base stations and the deployment of future 6GR base stations can be advanced simultaneously. At the same time, considering the current important frequency bands of NR, such as n41, n78, and n79, all of these bands have a bandwidth of arould 200 MHz. These bands are more likely to be refarmed to 6GR bands in the future. The priority of MSR between 6GR and NR is relatively high.

Observation 4: Some NR bands, such as n41, n78, and n79 have a bandwidth of about 200 MHz, are more likely to be refarmed to 6GR bands in the future.

Proposal 5: 6GR BS should support MSR, and MSR between 6GR and NR has high priority.

Proposal 6: BS RF requirements related to MSR should be studied.

### 2.4 Coexistence study

In addition to considering coexistence with LTE and NR, studies on coexistence with other technologies should also be conducted within the scope of 6GR study scope, such as coexistence with SBFD or NTN. Take SBFD as an example. Although R19 conducted some coexistence research in some scenarios and defined related RF requirements, the introduction of new 6GR frequencies and scenarios may be able to further enhance these requirements. The coexistence research in 6G should be conducted based on the actual deployment scenarios in the future. Therefore, it is recommended to revisit whether the current only coexistence research can be applicable to the network deployment of 6G.

Proposal 7: Legacy coexistence requirements should be revisited in 6G day1.

# 3. Conclusions

Observation 1: Research related to SBFD BS has been continuously conducted in recent releases and has received supports from numerous operators and BS vendors.

Observation 2: Frequency around 7 GHz has the large bandwidth, which is beneficial for the deployment of SBFD BS.

Proposal 1: SBFD BS should be defined in the 6G Day1.

Proposal 2: The RF requirements for 5G NR BS could be set as starting point for the 6GR refarming bands.

Proposal 3: Conduct research on the 6GR BS RF requirements using the frequency around 7GHz, and consider enhancing the existing RF indicators, e.g. EVM.

Observation 3: MSR can provide hardware support for MRSS.

Proposal 4: Conduction requirements relaxation could be considered in 6G day-1.

Observation 4: Some NR bands, such as n41, n78, and n79 have a bandwidth of about 200 MHz, are more likely to be refarmed to 6GR bands in the future.

Proposal 5: 6GR BS should support MSR, and MSR between 6GR and NR has high priority.

Proposal 6: BS RF requirements related to MSR should be studied.

Proposal 7: Legacy coexistence requirements should be revisited in 6G day1.

# 4. Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)