# 3GPP TSG-RAN WG4 Meeting #116bis                                                                   R4-2513136

Prague, Czech Republic, Oct. 13-17, 2025

Source:                Tejas Networks

Title:                    Multi-RAT spectrum sharing for migration, mobility between 5G NR and 6G

Agenda Item:      8.10

Document for:    Discussion

# Introduction

DSS has been a key enabler of 5G NR deployment since Release 15, supporting efficient coexistence and migration from LTE to NR on shared spectrum. Release 17 introduced cross-carrier scheduling enhancements [1], and Release 18 further advanced DSS by enabling narrowband deployments and dynamic resource optimization [2]. DSS continues to offer a stable framework for multi-RAT operations. As 3GPP begins 6G studies under 6GR [3], MRSS between NR and 6GR will be essential for seamless migration, mobility, and spectrum efficiency within existing 5G bands. In this contribution, we propose key RAN4 considerations for MRSS in 6G. In the last RAN4 meeting, there was discussion around planning for 6G based on the 6G SID, and the RAN4 6G plan and meeting arrangement were approved [4].

This contribution focuses on spectrum sharing aspects relevant to MRSS.

# Discussion

The 6GR study (revised SID RP- 252912, approved at RAN#109) [3] aims to define the overall 6G radio access network framework, with explicit scope for multi-RAT spectrum sharing to support migration and mobility between 5G NR and 6GR. This includes operating 6GR in existing NR spectrum allocations without disrupting 5G services, enabling operators to dynamically allocate resources based on traffic demands. RAN4’s role in such sharing mechanisms is critical for ensuring real-world feasibility, as it defines RF performance requirements, RRM procedures, and conformance testing for UEs and base stations.

## Evolution of DSS

The DSS feature allows LTE and NR to operate on the same carrier. It enables operators to introduce NR without major spectrum reallocation. DSS dynamically partitions resources, improving spectrum use and reducing interference, such as LTE control region effects on NR PDSCH. The focus remains on interference mitigation, power control, and conformance testing in shared environments. Release 15 established the foundation for LTE-NR coexistence in FR1 bands. RAN4 defined RF requirements, including ACLR and spurious emissions, to limit inter-RAT interference. RRM enhancements ensured right UE measurements, such as cell search, on DSS carriers. Conformance tests confirmed demodulation performance under LTE-heavy loads, enabling initial DSS deployments. Release 16 introduced optimizations for higher NR traffic ratios. RAN4 specified refined channel models for interference and power backoff limits to prevent NR signal degradation. These updates supported multi-band deployments and improved spectrum efficiency across diverse scenarios. Release 17 brought cross-carrier scheduling [5]. It allowed PDCCH on an SCell to schedule PDSCH or PUSCH on an sPCell in DSS configurations. RAN4 addressed RF impacts, including scaled PDCCH monitoring and inter-carrier interference tests, to support NR-heavy traffic on shared carriers. Release 18 targets narrowband DSS below 5 MHz [2]. It introduces sidelink enhancements for time-division multiplexing based sharing in co-channel LTE and NR, such as in band n47.

## RAN4 Considerations for MRSS in 6G

Smooth migration from 5G to 6G requires addressing challenges beyond what DSS handled in 5G. MRSS must support mmWave frequencies, ultra-wide bandwidths, advanced beamforming techniques such as holographic MIMO, and dynamic interference management. For RAN4, key focus areas include defining RF coexistence requirements for NR-6GR, specifying RRM procedures to support mobility in shared spectrum environments, and setting up conformance testing frameworks tailored to MRSS scenarios.

Proposal 1: Initiate RAN4 studies on RF coexistence for NR-6GR MRSS; define extended ACLR for wide bandwidths, interference models for advanced beamforming such as holographic MIMO, and AI/ML-based adaptive emission control to minimize interference-related performance degradation.

Proposal 2: Define RRM procedures for MRSS that enable continuous measurements, support conditional handovers, and allow cross-RAT spectrum aggregation such as NR CA with 6GR secondary cells. Include timing alignment requirements and ensure robust mobility handling even at extremely high speeds, with a strong emphasis on supporting high handover reliability.

Proposal 3: Establish MRSS conformance testing guidelines; incorporate radiated tests for 6GR beam interference on NR UEs, AI/ML validation for interference prediction, and scenarios simulating partial 6G rollout in NR-dominant spectrum.

# Conclusion

Proposal 1: Initiate RAN4 studies on RF coexistence for NR-6GR MRSS; define extended ACLR for wide bandwidths, interference models for advanced beamforming such as holographic MIMO, and AI/ML-based adaptive emission control to minimize interference-related performance degradation.

Proposal 2: Define RRM procedures for MRSS that enable continuous measurements, support conditional handovers, and allow cross-RAT spectrum aggregation such as NR CA with 6GR secondary cells. Include timing alignment requirements and ensure robust mobility handling even at extremely high speeds, with a strong emphasis on supporting high handover reliability.

Proposal 3: Establish MRSS conformance testing guidelines; incorporate radiated tests for 6GR beam interference on NR UEs, AI/ML validation for interference prediction, and scenarios simulating partial 6G rollout in NR-dominant spectrum.

# Reference

- 3GPP TR 21.917, Release 17 Description; Summary of Work Items.

- 3GPP TR 21.918, Release 18 Description; Summary of Work Items.

- RP- 252912, Revised SID: Study on 6G Radio.

- R4-2511652, RAN4 6G Plan and Meeting Arrangement

- RP-211345, “Revised WID on NR Dynamic spectrum sharing (DSS)”, Ericsson