3GPP TSG RAN4 Meeting #116bis															R4-2513145

Prague, Czech Republic, 13th – 17th Oct, 2025

Title: 	6GR General RF and UE RF

Agenda Item:	8.4

Source: 	Nokia

Document for:	Approval

# 1				Introduction

Under 6G system General RF and UE RF topic following items are to be discussed during the Q4 2025 [1].

- Non-spectrum and non-AI UE RF

- Spectrum aggregation

- UE RF related coverage and energy efficiency.

In this paper we give our initial views for each of these topics.

# 2	Discussion

2.1	Non-spectrum and non-AI UE RF

### 2.1.1	UE Power Class

In recent years PC2 has become de-facto default power class for 5G NR TDD bands. Introduction of PC2 power amplifiers PA’s made possible to do PC1.5 with two PC2 PAs. Therefore, it is natural that 6G baseline is PC2 for TDD bands and perhaps also FDD bands in lower frequencies if PC2 low band PA’s are in market.

When we say that PC2 is default power class we mean that it is the only power class that is mandatory to be defined for new 6G band and it naturally is the first power class to be defined. Additional HPUE power classes can be defined of course also but for eMBB use PC3 may not be defined at all.

Proposal 1: For 6G the default power class is PC2 (i.e. 26 dBm)

Proposal 2: For 6G bands PC2 is the first power class to be defined.

For 5G interband UL CA, 3GPP specified UL CA power classes in a table manner for PC3 and PC2. After multi-PA designs became common and UEs had combinations of PC3 and PC2 PAs it was decided to introduce higherPowerLimit-r17 capability where instead of defining new UL CA power class in a table the UE output power capabilities are determined from single band operation power class indication by calculating 10 log10 ∑ pPowerClass,c. This way for inter-band UL CA/DC UE is allowed for each band to transmit up-to single band power class capabilities.

higherPowerLimit-r17

Indicates whether UE supports increase in maximum output power above the power class indication for inter-band UL CA and NR-DC band combinations as defined in clause 6.2A of TS 38.101-1

Proposal 3: In 6G RAN4 do not define interband UL CA power classes. UL CA output power capabilities are determined by constituent single bands power classes. Approach is similar as NR higherPowerLimit feafure.

### 2.1.2	MPR for eMBB UE types

Here with eMBB we mean “basic UEs” which have requirements specified without a clause suffix in 36.101 or 38.101-1. In 4G and 5G different MPRs have been specified for different power classes but MPRs are quite similar between different power classes.

In 6G we think that there is a need to define MPR separately to battery opereated mobile terminals and non-battery-operated terminals such as FWA and CPE. Reason being that in battery operated terminals current consumption is the key parameter that needs to be managed and that causes a trade of between PA linearity and UE current consumption where suitable balance always needs to be found. In non-battery-operated UE current consumption is not as critical, and PA linearity can be prioritized. This means that need from MPR and A-MPR is reduced for non-battery operated UEs compared to battery-operated UEs.

Dedicated MPRs are defined for battery-operated UEs and non-battery operated UEs.

UE type indication is needed to be able to separate battery-operated UEs and non-battery operated UEs.

#### 2.1.2.1	PA dimensioning/modelling

In 6G we are proposing a modest PA linearity update for eMBB battery operated UE compared to NR. More specifically we propose to use in MPR simulations a PA dimensioning: MPR = 0 for CP-OFDM QPSK Inner. This is the PA calibration point which is fixed and determines MPRs for rest of the modulations. Some of the modulations and waveforms would have negative MPR which could be seen as built in boosting capability as output power would exceed Ppowerclass.

Proposal 4: PA dimensioning for battery operated eMBB UEs is MPR = 0 for CP-OFDM QPSK Inner

#### 2.1.2.2	Modulator impariments

Modulator impariments should be based on what is actually needed for most demanding modulation order. In 5G it is imposible to meet UL 256-QAM EVM requirement with the nominal 28 dBc LO lekage and IQ-image requirements, instead much more stringent modulator impairment values are budgeted for EVM of 256-QAM compared to lower order modulations. This is not very technical approach. Hence baseline for 6G modulator requirements should be atleast what is needed for 256-QAM and if higher modulation orders are considered then modulator requriemets need to be derived from those. Improvement of modulator impairments will directly impact to MPR and A-MPR numbers and increase UE Tx power.

Proposal 5: Modulator impariments should be based on what is actually needed for most demanding modulation order to meet EVM requirement.

### 2.1.3	Pcmax

Upon studying the technical specification 38.101-1 [1], one can observe that the configured transmitted power for 5G is fairly complex which is understandable as there is legacy carried from 4G and new requirements added by different releases of 5G. We think that for 6G there are opportunities to optimize the equations and parameters for PCMAX,f,c

The configured maximum output power PCMAX,f,c is set within the following bounds for single carrier 5G in 38.101-1:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass –++w3 ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

Regarding 6G, we think that the same framework can still be utilized, albeit in a simplified and optimized form. From our view the necessity of incorporating ΔTIB,c or ΔPPowerBoost into the equation needs reconsideration. The parameter ∆TC,c, may be updated to allign with technological advancements available today or within 6G timeframe. The upper limit of PCMAX_H,f,c, should be constrained only by term PEMAX,c,  to allow UE to transmit its full capability. Furthermore, although it does not influence the equation, the application of ΔPPowerClass  in SRS scenarios could be re-evaluated.

Proposal 6: Investigate the simplification of PCMAX,f,c equations and the optimization of associated parameters

### 2.1.4	SEM, ACLR and Spurious emissions

Traditionally General SEM is defined by 3GPP and additional SEM’s have been added based for some regional regulation. ALCR (and ACS) comes from co-existance simulations between 3GPP systems and is performed by RAN4. Spurions emissions are coming from ITU recommendations. This framework can be used also for 6G.

Framework for defining SEM, ACLR and Spurious emissions can be re-used  from NR and LTE for 6GR.

### 2.1.5	UL Switching

The initial value proposition of UL Tx switching in 5G was aimed at improving data rates and reducing latency by utilizing one of the UE transmitter chains for a new carrier when UE is operating in 2TX TDD mode during the DL phase. However, practical challenges, such as switching gaps, complicated PCell control plane processes and the outcome was a reduction in anticipated benefits. UEs indicate switching period capabilities of {35, 140, 210} microseconds, which the network uses to schedule temporal gaps by blanking symbols, leading to UL throughput penalties of 1, 2, or 3 symbols depending on the UE's capability. By the time switching period capabilities were established for Rel-16, durations were rooted in legacy implementation. Considering modern RF chip technology and leveraging insights from 5G, from our perspective it is feasible to avoid utilizing at least the longest capability of 210us. Even today there is a special case of 0 microseconds switching time for SUL (TS 38.101-1, 5.2C, Note1), applicable to UEs configured with both NR UL and SUL carriers requiring simultaneous Rx/Tx operations. Currently 5G specifications mandates DL interruptions when UL switches from one carrier to another which is having detrimental impact on throughput. The need for the interruptions in modern RF chips may be reevaluated.

Proposal 7:  Re-evaluation of switching period lengths to match modern RF chip technology

Proposal 8:  Re-evaluation of the need for DL interruptions considering modern RF chip technology

### 2.1.6	DL Switching

In WID for the LB CA switching, there was a recommendation to utilize the Rel-16 TX switching framework as much as possible. This can be seen problematic as the reasons why UL switching framework ended up to the form it is having now are because of the dependencies that are unique for UL implementation.

For DL, there are more HW resources available that can be activated at specific time instances without necessitating the use of the same HW for two carriers during the switch event. The factors influencing the required time for DL carrier switching differ from those for UL. RFFE reconfiguration for DL switching is more straightforward than for UL and is typically performed concurrently with the transceiver. Unlike dynamic UL switching, DL CA switching operates based on a semi-static RRC pattern which makes planning for timing predictable.

The switching periods for DL CA are specified as {35,70,140}us. The network is tasked with scheduling resources for a UE, taking into account the signaled capability and total timing advance (TTA). UL transmission at the UE side precedes DL reception by the TTA amount. If a UE shares an antenna with the two carriers to be switched, or if the UL band is close to the DL band, simultaneous UL/DL operation is prohibited. At late phase of Rel19 a new requirement emerged that UL transient periods cannot overlap with the switching periods in time.

Figure 1 FDD to SDL UL time mask options with TTA =0us. The number of blanked UL symbols is dependent on the position of the UL transient period with respect to the switching period.

Figure 2 SDL to FDD UL time mask options with TTA =0us. The number of blanked UL symbols is dependent on how much the UE can tolerate the UL and switching gap overlap.

As the network schedules the switching gap based on the number of symbols, the longest switching period results in the blanking of one “extra” symbol for UL when TTA=0.

UL switching framework is ill-suited for DL.

Transient periods and timing advance are adversely affecting the DL throughput.

### 2.1.7		REFSENS

In the current specification, REFSENS is defined in absolute numbers for two antenna ports, except for n7, n38, n41, n48, n77, n78, n79, and n104 for which four RX antenna ports is assumed, in Table 7.3.2-1a. This is for PC3. Then in Table 7.3.2-1b, REFSENS is defined for TDD, SDL, and FDD bands with variable duplex operation as an equation. Then there are two delta tables Table 7.3.2-1c and Table 7.3.2-1d, which specifies RSD for PC3 to PC2, with and without TX diversity. The RSD with TX diversity is much severe compared to without TX diversity. Then there are two more delta tables Table 7.3.2-2a and Table 7.3.2-2a for four RX (ΔRIB,4R) and eight RX (ΔRIB,8R). Regarding 1 RX (e)RedCap devices, there is a flat delta ΔR1R defined in Table 7.3I.2-1, which some companies think is a mistake.

The uplink configuration for validation of the REFSENS requirement is another big table with barely any justification of why and how those numbers came into existence. There are bands for which due to reasons varying from small TX-RX separation, harmonics, etc., smaller uplink configuration is utilized instead of full channel allocation. However, all these reasoning are hidden behind absolute numbers, thus making the specification very unintutive.

Current specification with only absolute numbers is unintutive.

For many bands, due to a myriad of reasons, neither the delta values nor the absolute REFSENS values scale with channel bandwidth. As a result, specifying only absolute numbers obscures the underlying complexity and rationale behind how the specification was defined. This presents a significant challenge for future-proofing the specification, as it embeds hidden assumptions and impairments and for any change required, one must must first reverse engineer the original impairment calculations, then recalculate the values using new assumptions.

Curent way of specifying REFSENS is not future proof.

To make specifications future proof, it would be better to document the underlying assumptions, impairment factors, and calculation methods explicitly in the normative specifications.

Another issue with 5G REFSENS specification is related to the values assumed for the NF and IM. The NF number of 9.0 dB was first proposed back in 1999 in meeting#1[5]. Since then, there has been leaps and bound improvement in the device technology, external LNAs are the norm providing a significant improvement in the noise performance of the UE. However, these improvements are not visible in the UE RF specification.

UE NF of 9.0 dB was first proposed in 1999.

Implementation margin (IM) of 2.5 dB for REFSENS calculation is another parameter that has been stuck in the past. Further, it is a black box of margin as there is no consensus on what exactly it covers, why it is same for TDD and FDD bands. This unfortunately is another ambiguous number with no clear description.

Implementation margin of 2.5 dB as a catch all margin is ambiguous and contributes to the unintutiveness of the specification.

Proposal 9: A unified equation covering various cases such as different duplexing, multiple receivers, etc., shall be utilized for defining the REFSENS requirements.

Proposal 10: A better NF assumption shall be agreed for 6GR keeping the technology advancement of last two decades.

Proposal 11: Study the need and possible value for implementation margin.

### 2.1.8	ACS and Blocking

3GPP has defined several UE blocking requirements. These are inband blocking (IBB), out of band blocking (OOB) and narrowband blocking (NBB) which have been alsmost unchanged for a quite a long time, infact there are virtually no or just small differences between 4G and 5G requirements.

These blocking requirements have worked well and there may not be any need to make big (or any) adjustments to levels of blockers or ranges of interferers. Obviously blocking requirements for 6G need discussion and changes are possible but there would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker levels and interferer offsets.

There would need to be very strong need and justifications to alter the current blocking requirements for 6G in terms of blocker level and interferer offsets.

As was discussed we do not necessarily see a need for changing the blocker levels and interferer offsets but we see a need to restructure how we write the spec and especially how we do the tables. In our view especially how the IBB requirement is defined needs improvements. In our view major reason why IBB bloking requirement is obscure and hard to decipher from tables is that IBB1 is with fixed offset, similarly as ACS and IBB2 is a range similarly as OOB. It would make quite a lot of sense to re-organize ACS and IBB requrements such that ACS and IBB1 are combined togeter as ACS1 and ACS2, levels are kept the same. IIB2 would become the only IBB requirement see Fig 2.1.8-1.

Figure 2.1.8-1: Possible 6G UE ACS and Blocking requirements

How the UE blocking requirements are defined/organized should be discussed and not just copied from NR.

## 2.2	Spectrum aggregation

In general, RAN4 have considered all aggregation of spectrum as carrier agregation. However, the topic is treated here in a more general manner as the startring poing is the individual UE channel bandwidths.

### 2.2.1	UE channel bandwidth adaptation

Despite its initial motivations to support diverse UE bandwidth capabilities and enhance power saving, the 5G NR BWP design presents several significant challenges. These include considerable signaling overhead and complex configuration changes, primarily because BWP switching involves unnecessarily heavy Radio Resource Control (RRC) configuration, designed to support impractical use cases such as different Subcarrier Spacings (SCSs) per BWP.

Another issue is the poor robustness and long delays in BWP switching. For instance, NR design for BWP switching may result in severe error cases, and BWP switching delays often assume worst-case scenarios.  There was also a notable lack of RAN4 involvement in NR BWP, which particularly affects uplink coverage. This is because many UL BWP configurations allow unnecessarily large Maximum Power Reduction (MPR) or Additional MPR (A-MPR), thereby lowering the minimum requirement of UE transmit power. Finally, the coupled Uplink (UL) and Downlink (DL) BWP in TDD is problematic. 5G TDD mandates that UL and DL BWPs share the same center frequency, a restriction that results in configuration and scheduling limitations, suboptimal UL coverage, and prolonged switching times.

Observation: The 5G NR BWP design, despite aiming for UE power saving and diverse bandwidth support, is hampered by excessive signaling overhead, poor switching robustness and latency, a lack of RAN4 involvement leading to large MPR/A-MPR, and restrictive coupled UL/DL BWPs in TDD.

Need for 6G BW adaptation

For 6G Bandwidth Adaptation, while addressing 5G's shortcomings, 6G must continue to support varying UE bandwidth configurations and capabilities. It is crucial to define the necessity of bandwidth adaptation without reintroducing complexity. In downlink, bandwidth adaptation is useful for UE power saving, supporting UEs with limited BW capabilities, and effective load balancing. Whether UE power saving in DL is achieved with BW adaptation, PDCCH adaptation, or both may need further investigation. In uplink, BW adaptation is critical from a coverage perspective and necessitates active RAN4 involvement. For example, proper placement of LO along with BW adaptation needs attention to minimize A-MPR.  In addition to dynamic bandwidth adaptation supporting also load balancing scenario, 6GR needs to support initial BWP functionality, which can be seen as a key enabler for less capable UEs.

Since the 6G target is to enable leaner BW adaptation by solving the lessons learned from 5G, 6G could reconsider the terminology for BWP. One option could be to call it as a scheduling range.

Another issue is that 6G may support channel bandwidths up to 200MHz or 400MHz. While NR supports intra-band contiguous carrier aggregation (CA) to utilize such bandwidths, in 6GR primary focus should be in optimization of single wideband carrier to handle contiguous spectrum allocations at least for channel BWs up to 200MHz.

Proposal 12:  Study 6GR bandwidth scalability by early cooperation with other working groups with the following aspects:

- Simplify Bandwidth Adaptation:

  - Reduced configuration overhead

  - Reduced number of bandwidth adaptation options compared to NR.

  - Support only a single Subcarrier Spacing (SCS) per carrier.

  - Minimize the number of BW adaptation types and BW-specific parameters.

- Separate DL and UL BW Adaptation:

  - Consider DL and UL bandwidth adaptation independently to optimize each direction.

  - Assume separate LO for DL and UL BW adaptation in TDD

- Improve Robustness, Reduce Latency and minimize interruptions in BW adaptation:

  - Enhance the reliability and speed of bandwidth adaptation – the procedure must support robust failure recovery.

- Study to optimize RF/RRM switching delays and MPR/A-MPR with better assumptions related to LO placement in bandwidth adaptation

- Streamline UE Capabilities:

  - Define concise set of UE capabilities for BW adaptation and identify the essential operation to ensure good baseline feature support for BW adaptation operation.

### 2.2.3	Flexible UL & DL pairing

DL/UL Decoupling has been a research topic since 5G study phase and in the context of 3GPP it is often associated with SUL. Considering UE RF, while SUL original promise is to improve coverage of a UE there are also other opportunities flexible DL/UL concept could be advantageous. Switching framework for UL and DL CA allows UE to switch carriers dynamically or semi static ways. As UEs today are supporting several carriers, and combinations of those it is possible to develop means for a UE to be able to use optimum carrier frequency in a given point of time to enable optimum resource usage in a network. One measure of improved performance can be MSD avoidance. By flexible UL/DL pairing UL may schedule resources across carriers so that MSD can be completely avoided.

From RAN1#122 several companies presented benefits to combine uplink and downlink carriers of bands, which are not related in a TDD or FDD style of 3GPP. The aim is to get advantages such as uplink coverage, by combining a lowband uplink carrier with a downlink carrier of typical larger channel bandwidth of the bands above 1.7GHz, no matter if its FDD or TDD.

Key considerations for flexible pairing:

- Scenarios

  - Mid-band + Low-band (DL/UL available on both carriers)

  - Mid-band + SUL-only.

  - Low-band + SDL

  - SDL+SUL

- Initial access may be allowed via low-band or mid-band.

- Low band UL carriers may be shared across multiple DL carriers.

- Real time interaction across the schedulers needs considerations.

- Within a certain band combination, possibilities of DL and UL combinations (DL CA, UL CA, flexible pairing)

- Timing requirements across the carriers.

Figure 2.2.3-1: Flexible UL and DL pairing

Flexible UL and DL pairing is interesting idea in concept level but can be very difficult from RAN4 point of view. Discussion is needed how flexible it needs to be. Flexibility comes with a trade of not knowing the performance.

## 2.3	UE RF related coverage and energy efficiency

### 2.3.1	UL scheduling range (Petri)

For 6G, one of the goals is to reduce unwanted emissions and lower UE power consumption. These were also the goals on 5G BWP but did not materialize in RAN4 specs. In Rel-19 RAN4 worked on a single carrier MPR reduction scheme which allows in some limited cases convert outer allocations into inner allocations, but benefits of this feature are rather limited especially as it does not address A-MPR cases.

In 6G we would like to take Rel-19 work on single carrier MPR enhancement further and make it more flexible and also applicable for A-MPR. Basic principle is shown in figure 3.2.1-1.

Figure 3.2.1-1: UL scheduling range

The UL scheduling range would have the following properties

- UL scheduling range is configured to UE.

- UL scheduling range bandwidth is smaller or at maximum the same as the UE and/or BS channel bandwidth

- UE places the local oscillator (LO) in the middle of the UL scheduling range (bandwidth)

- UE may set UL baseband processing and RF filters according to UL scheduling range bandwidth and location in frequency.

- Bandwidth of intermodulation distortion (IMD) interference generated in the power amplifier (PA) due to allocation, LO leakage and IQ-image are directionally proportional to the bandwidth of the UL scheduling range i.e. the smaller the UL scheduling range the less interference

- UE Tx current consumption decreases when UL scheduling range bandwidth decreases as baseband and D/A-conversion clock rates can be reduced.

Impact to MPR reduction is achieved by proper LO placement and reduction of UL scheduling range bandwidth into what is needed instead of what is UE channel bandwidth. In R19 RAN4 WI this approach was already partially defined but the flexibility will be constrained, and hence there is room for improvement.

Figure 2.3.1-1: UL scheduling range impact to MPR

UL scheduling range impact to A-MPR reduction will be significant as in some cases A-MPR can be fully avoided. This is because the IMD created in power amplifier by the IQ-Image, LO leakage impairments and the actual transmission are not spread as far in out-of-band region comparing the case when UL bandwidth is same as UE channel bandwidth.

Figure 2.3.1-2: UL scheduling range impact to A-MPR

Proposal 13: RAN4 needs to consider bandwidth adaptation (UL scheduling range) from the beginning, to minimize MPR and A-MPR.

UL scheduling range is configured by the network for each UE individually. When UL scheduling range is configured to the UE, then UE set’s LO in the middle of the scheduling range and adapts filters as necessary. LO and filter are unchanged until scheduling range is reconfigured. UE UL scheduling instances happen within the UL scheduling range bandwidth.

### 2.3.2		FDSS-SE

Effect of lower PAPR waveform to coverage is due to increased UE maximum transmission power, however there may be some loss in link performance. Hence the actual effect to link budget can be described using net gain, which was defined in Rel-18 coverage enhancement WI. Net gain can be defined simply as:

Net Gain [dB] = Tx power gain – link loss

This is relevant as it is not sufficient to look at the additional Tx power that maybe achieved if the impact to the link quality and the SNR requirement at the receiver is not taken into account.

Tx power gain is Tx power difference between the baseline case and introduced low PAPR waveform enhancement. Best way to obtain this is using RF-simulations that take into account RF-requirements such as EVM, SEM, inband and out of band emissions etc. This approach provides much more accurate results than simply estimating the power difference using e.g. PAPR or cubic metric (CM) difference.

Proposal 14: Net gain is used to evaluate coverage gain of waveform enhancements.

Proposal 15: RF-simulations are used to obtain Tx power gain for net gain evaluation.

A low PAPR method used already in 5G is frequency domain spectrum shaping (FDSS). FDSS is a method where spectrum of DFT-s-OFDM signal is shaped by utilizing frequency domain shaping filter resulting to low complexity and good reduction in PAPR. FDSS was specified already in Rel-15 for pi/2 BPSK and later in Rel-18 with slightly different requirements for both pi/2 BPSK and QPSK. In Rel-18 timeframe also frequency domain spectrum shaping with spectrum extension (FDSS-SE) was discussed. FDSS-SE provides more enhancement compared to FDSS by adding excess bandwidth where data is copied from inband, see figure 2.3.2-1 below, where the symmetric extension stage copies the edges of the nominal frequency allocation to the extended allocation before the frequency domain spectrum shaping stage.

Spectrum shaping filtering could be done for FDSS/FDSS-SE either in transparent or non-transparent way. In transparent approach the same filter is used for both DMRS and data, hence the effect of the filter gets compensated in the receiver and the receiver does not need to be aware of the filter parameters. There is no need to specify anything related to filter except relaxed spectrum flatness requirement as done in NR ([2], sections 6.4.2.4.1 and 6.4.2.4.2). Spectrum flatness is a requirement for Tx signal defined by RAN4 to facilitate frequency domain equalization in base station receiver. Specifying filter parameters would require extensive discussions in 3GPP as the optimum filter depends also on implementation. Together with the fact that there are no clear benefits of non-transparent approach we believe that transparent approach should be followed also in 6G.

Both FDSS and FDSS-SE should be supported in 6G to provide enhanced coverage compared to 5G.

Figure 2.3.2-1. FDSS-SE

Proposal 16: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio.

Proposal 17: Transparent filtering approach is used for FDSS and FDSS-SE in 6G Radio.

Net gains of FDSS and FDSS-SE are shown in Figure 2.3.2-2 and Figure 2.3.2-3 for outer and inner RB allocations. In these results Tx power gains have been evaluated by RF simulations. Link simulation assumptions including MCSs used are as agreed in Rel-18 work item (RAN1 #111). As the results show, the net gains of FDSS-SE are much higher than FDSS. This is especially true for the inner allocations where the FDSS gains are modest.

Similar results obtained in Rel-18 work item and averaged over all the companies results can be found in [3]. Despite differences in different companies results the average gains seem to be in the same ballpark as in Figures 2 and 3.

Figure 2.3.2-2. Net gains of FDSS and FDSS-SE for outer RB allocations

Figure 2.3.2-3. Net gains of FDSS and FDSS-SE for inner RB allocations

### 2.3.3	Tx EVM relaxation

RAN #109 approved new WI for NR in [4]. Tx EVM relaxation was discussed in RAN as part of the WI, however there was a decision to continue the work in RAN4 6G study context, hence there is a note in WID:

Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

According to the note above RAN4 should discuss the scope and other details including whether there is network control for the feature. In our view there should definitely be network control/configuration, it is difficult to understand how could the feature be specified without. Network should have possibility to operate also without the relaxed UE Tx EVM. Also scope of the work and all assumptions should be clarified. Scope and assumptions should include at least following:

- Scenarios including waveform and frequency range(s)

- Receiver assumptions

- Evaluation assumptions

- RAN4 specification impact

Proposal 18: Network control would be needed for the relaxed UE Tx EVM feature

Proposal 19: RAN4 to discuss the scope and assumptions for the relaxed UE Tx EVM feature

EVM is intended as a general measure of the transmitter accuracy. Anomalies such as IQ imbalance, phase noise, nonlinear degradation, filter passband ripple and group delay, and thermal noise are all factors that contribute to EVM. The total EVM attributed to the various degradation effects is the sum of the square of EVMs due to each source of degradation.

The below table [TS38.101-1, Clause 6.4.2.1] for the UE transmitter lists the 5G EVM requirements after correcting for sample timing offset, RF frequency offset and carrier leakage:

Table 6.4.2.1-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

3GPP defines the MPR requirements in uplink as maximum allowed power reduction such that UE transmit power shall satisfy the minimum RF requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

With PA non-linearity (or other impairments) compensation, the receiver can basically estimate or learn the impairments of the PA and compensate those in the receiver. General architecture of the digital post distortion (DPoD) model is shown in .

Figure 2.3.3-1: UL / DL general architecture including DPoD block.

As shown in figure 2.3.3-2, for 256QAM EVM is always the bottle neck, while for lower orders (e.g., 64QAM) it is mainly either IBE (in the inner region) or ACLR (in the outer region). Also for 256QAM, if EVM was relaxed, IBE would become the gating factor. For this reason, relaxed TX EVM could be suitable mainly for FR1 in the case of a high modulation scheme, e.g. >=256 QAM where the EVM is the gating factor. In other words, gain potential only for 256QAM or higher unless also IBE/ACLR is relaxed. The result below shows only the gating factor for each modulation but does not show the level where the next possible RF requirement would become a gating factor in case the requirement was changed for the current gating factor. Hence it does not show anything about the possible gain potential for each modulation. Gain potential should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

There would be gain potential mainly for 256QAM where EVM is always the bottle neck, while for lower orders (e.g., 64QAM) it is mainly either IBE (in the inner region) or ACLR (in the outer region)

Proposal 20: Gain potential for Tx EVM relaxation should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

Figure 2.3.3-2: Gating factors, 100MHz, 30 KHz

IBE is already limiting the UL maximum power in 5G NR in many cases such as for all waveforms, power classes, most modulation orders and RB allocations, etc.

5G NR Rel-16 allowed Network to configure UEs in FR2 serving cells to omit IBE requirements completely and boost UL power since UEs are clearly spatially separated. However, this solution is not fully applicable in lower frequency ranges due to wider beam and/or UE beamforming architecture. Thus partial IBE relaxation, as depicted in Figure 2.3.3-3, by shifting IBE edge from active RBs can be considered in lower frequency ranges. The partial IBE relaxation allows to achieve up to 2.5 dB power gain even without Tx EVM relaxation for modulation orders below 256 QAM and assist Tx EVM relaxation to achieve higher gains for high modulation order such as 256QAM.

- Figure 2.3.3-3: Partial IBE relaxation for higher UL maximum power

Partial and full IBE relaxation can increase UL power for all waveforms and most modulation orders in all frequency ranges and can increase Tx EVM relaxation gain for high modulation orders such as 256 QAM.

Proposal 21: RAN4 to consider NW controlled partial and full IBE relaxation without Tx EVM relaxation for modulation order up to 64QAM, and with Tx EVM relaxation for higher modulation order.

# 3	Conclusion

In this contribution we have made following proposal which are further justified with a number of observations and explanations in the contribution.

Proposal 1: For 6G the default power class is PC2 (i.e. 26 dBm)

Proposal 2: For 6G bands PC2 is the first power class to be defined.

Proposal 3: In 6G RAN4 do not define interband UL CA power classes. UL CA output power capabilities are determined by constituent single bands power classes. Approach is similar as NR higherPowerLimit feafure.

Proposal 4: PA dimensioning for battery operated eMBB UEs is MPR = 0 for CP-OFDM QPSK Inner

Proposal 5: Modulator impariments should be based on what is actually needed for most demanding modulation order to meet EVM requirement.

Proposal 6: Investigate the simplification of PCMAX,f,c equations and the optimization of associated parameters

Proposal 7:  Re-evaluation of switching period lengths to match modern RF chip technology

Proposal 8:  Re-evaluation of the need for DL interruptions considering modern RF chip technology

Proposal 9: A unified equation covering various cases such as different duplexing, multiple receivers, etc., shall be utilized for defining the REFSENS requirements.

Proposal 10: A better NF assumption shall be agreed for 6GR keeping the technology advancement of last two decades.

Proposal 11: Study the need and possible value for implementation margin.

Proposal 12:  Study 6GR bandwidth scalability by early cooperation with other working groups with the following aspects:

- Simplify Bandwidth Adaptation:

  - Reduced configuration overhead

  - Reduced number of bandwidth adaptation options compared to NR.

  - Support only a single Subcarrier Spacing (SCS) per carrier.

  - Minimize the number of BW adaptation types and BW-specific parameters.

- Separate DL and UL BW Adaptation:

  - Consider DL and UL bandwidth adaptation independently to optimize each direction.

  - Assume separate LO for DL and UL BW adaptation in TDD

- Improve Robustness, Reduce Latency and minimize interruptions in BW adaptation:

  - Enhance the reliability and speed of bandwidth adaptation – the procedure must support robust failure recovery.

- Study to optimize RF/RRM switching delays and MPR/A-MPR with better assumptions related to LO placement in bandwidth adaptation

- Streamline UE Capabilities:

  - Define concise set of UE capabilities for BW adaptation and identify the essential operation to ensure good baseline feature support for BW adaptation operation.

Proposal 13: RAN4 needs to consider bandwidth adaptation (UL scheduling range) from the beginning, to minimize MPR and A-MPR.

Proposal 14: Net gain is used to evaluate coverage gain of waveform enhancements.

Proposal 15: RF-simulations are used to obtain Tx power gain for net gain evaluation.

Proposal 16: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio.

Proposal 17: Transparent filtering approach is used for FDSS and FDSS-SE in 6G Radio.

Proposal 18: Network control would be needed for the relaxed UE Tx EVM feature

Proposal 19: RAN4 to discuss the scope and assumptions for the relaxed UE Tx EVM feature

Proposal 20: Gain potential for Tx EVM relaxation should be studied with RF-simulations taking into account all the relevant RF-requirements, such as EVM, IBE, ACLR, OBW, SEM, and spurious emission.

Proposal 21: RAN4 to consider NW controlled partial and full IBE relaxation without Tx EVM relaxation for modulation order up to 64QAM, and with Tx EVM relaxation for higher modulation order.

# 4	References

[1] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

[2] TS 38.101-1 “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, 3GPP

[3]	RP-230864, “Uplink coverage enhancements”, Nokia, Nokia Shanghai Bell

[4] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5”

[5] TSGW4#1(99)005, MS receiver sensitivity in UTRA FDD mode, Nokia, RAN4#1, Espoo, Finland, 21st – 22nd January, 1999.