3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513164

# Prague, CZ, 13th – 17th Oct, 2025

Title: 	Discussion on demodulation aspects for 6G

Source: 	CATT

Agenda item:	8.8

Document for:	Discussion

# Introduction

The study for 6G Radio Access Technology[2] aims to develop one non-backward compatible radio access technology, and the motivation is deeply rooted in the growing demands and technological evolution expected in the coming decade.

In this contribution, we provide our initially views on 6G demodulation study.

# Discussion

As per RAN4 6G Plan and Meeting Arrangement[1], the remaining issues concerning 6G demodulation aspect are copied as below:

The demodulation performance requirements are to specify the ability of BS, UE etc. to correctly demodulate signals in various conditions and configurations.

### 2.1 Other PHY signal/channel/procedure related demod

The “signal” to demodulate is dependent on PHY signal/channel/procedure for 6G. Since the ISAC is 6G vision, for ISAC BS, the physical layer must demodulate not only communication signals but also signals reflected from environmental objects for perception and positioning. This will generate entirely new “sensing reference signals” and corresponding demodulation process. So RAN4 may study the demodulation for ISAC for 6G.

Proposal 1: RAN4 to study the demodulation for ISAC for 6G.

### 2.2 Channel model for demodulation

Before 5G Rel-19, the TDL propagation conditions and antenna correlation matrix as channel model were used for demodulation. In Rel-19, the spatial channel model (SCM) was studied and agreed to be used for SU-MIMO demodulation, and the study of spatial channel model (SCM) for MU-MIMO was agreed in RAN#109. So for 6G demodulation, the spatial channel model (SCM) can be used.

Proposal 2: RAN4 use spatial channel model (SCM) as one candidate channel model for demodulation for 6G.

### 2.3 Detector types for demodulation

In 5G demodulation, the linear MMSE and linear MMSE-IRC detector (LMMSE-IRC) were used for demodulation. The major benefit for the usage of widely linear MMSE-IRC(WLMMSE-IRC) [3,4] is to increase the detector’s degrees of freedom from NR to 2NR. Particularly, this holds strictly if both desired and interfering signals employ a real-valued modulation scheme. As a consequence, the interference suppression capability of linear interference-aware detector can be significantly enhanced, from NR -1 to 2NR -1, leading to an improved performance in interference limited scenarios.

Proposal 3: Widely linear MMSE-IRC (WLMMSE-IRC) detector can be considered as one candidate detector which benefits from network coordination of modulation type.

# Conclusion

This contribution discussed the demodulation aspects in 6G and presented following proposals and observation:

Proposal 1: RAN4 to study the demodulation for ISAC for 6G.

Proposal 2: RAN4 use spatial channel model (SCM) as one candidate channel model for demodulation for 6G.

Proposal 3: Widely linear MMSE-IRC (WLMMSE-IRC) detector can be considered as one candidate detector which benefits from network coordination of modulation type.

# Reference

[1]    R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair, RAN4#116

[2]     RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN#108

[3]     R4-131791, Receiver structure feasibility for LTE Rel-12, Renesas Mobile Europe Ltd, RAN4#66bis

[4]     R4-131793, Further considerations on Advanced Receivers, Renesas Mobile Europe Ltd, RAN4#66bis