3GPP TSG RAN WG4 Meeting #116bis		R4-2513165

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on LB-LB CA via switching

Agenda Item:	4.8.1

Source: 	CATT

Document for:	Approval

1 Introduction

In this August meeting, RAN2 has agreed the CR [1] for TS 38.331 to introduce network signalling for Rel-19 LB-LB CA via switching.

In this August meeting, RAN1 has reached the following agreements and conclusion based on the RAN1#122 chair notes [2].

Agreement For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, a UE does not receive the SSB that collides with the LB CA switching pattern - Note: a SSB collides with the LB CA switching pattern if - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap Conclusion For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support the following restrictions for the semi-static switching pattern - It is RAN1 understanding that at least one SSB in PCell is available every 160ms for meeting existing RAN4 requirements - At least one SSB in SCell is available every 160ms

In the RAN4#116 meeting, an approved WF [3] captured some open issues, e.g. UL/DL RMC, to be further discussed in the performance phase of this work item.

Issue 1: Downlink reference measurement channels Proposed Agreement: - RAN4 can continue to discuss how to specify the DL RMC for this feature as part of the Performance phase of the work item - The following options for the RMC have been proposed: - Option 1: 10ms periodicity with switching period placed in slots 4 and 8 - Option 2: Increase period of RMC to 40 slots and use the LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’ - Option 3: Bitmap (0000000000, 1111111111, 0000000000, 1111111111) - Option 4: other options are not precluded Issue 2: Uplink reference measurement channels Proposed Agreement: - RAN4 can further discuss whether an UL RMC is needed for this feature Issue 3: Reference measurement channels: general aspects Agreement: - In RMC, NTA=0 is considered.

In this paper, considering the latest outcomes of RAN1 and RAN2 for this work item, we’d like to further discuss UL/DL reference measurement channels and refine our DL RMC proposal of the contribution [4].

2 Discussion

2.1 Test methodology for the DL RMC

As a new switching pattern between the PCell and the SCell will be indicated by network IE LBCA-SwitchingPattern referring to clause 24 of TS 38.213, current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching. That means current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for this case.

The clause 7.3A.2.3 of TS 38.101-1: 7.3A.2.3 Reference sensitivity power level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6. For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2. The reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation and power class 2 or power class 3 CA intermodulation interferences are applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

Observation 1: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching.

Apart from REFSENS requirements, it’s very important to verify the DL receiving performance for UE before and/or after the Switching gap. Alternatively, RAN4 can design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Observation 2: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Based on RAN1’s conclusion on the restrictions for the semi-static switching pattern, one SSB in PCell and one SSB in SCell is available every 160ms.

- It is RAN1 understanding that at least one SSB in PCell is available every 160ms for meeting existing RAN4 requirements - At least one SSB in SCell is available every 160ms

Thus, the DL RMC for the LBCA-SwitchingPattern should include at least one SSB in PCell and one SSB in SCell. Option 1 only including the SSB in PCell without SSB in SCell can’t align the RAN1’s understanding on the restrictions for the semi-static switching pattern.

Observation 3: the DL RMC for the LBCA-SwitchingPattern should include at least one SSB in PCell and one SSB in SCell and option 1 in the WF R4-2511862 can’t align the RAN1’s understanding on the restrictions for the semi-static switching pattern.

In order to verify the DL REFSENS requirements and DL receiving performance for UE before and/or after the Switching gap, it’s proposed to schedule the DL switch-from slots and switch-to slots for the DL RMC.

Considering the worst case and simplifying the specification, it’s proposed to configure two symbols PDSCH mapping Type B for DL RMC referring to the table 5.1.2.1-1 of TS 38.214. It’s better to configure the same length of symbols in all the DL switch-from slots and switch-to slots.

Table 5.1.2.1-1: Valid S and L combinations PDSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; {0,1,2,3} (Note 1); {3,…,14}; {3,…,14}; {0,1,2,3} (Note 1); {3,…,12}; {3,…,12}. Type B; {0,…,12}; {2,4,7}; {2,…,14}; {0,…,10}; {2,4,6}; {2,…,12}. Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3.

For the LBCA-SwitchingPattern, it’s proposed to design the DL RMC as ‘00011 00110 01100 11001 11100 11001 10011 00110’ in order to increase DL samples (18 DL Slots for PCell and 18 DL Slots for SCell). Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots. In addition, one symbol PDCCH is scheduled at the first symbol in each DL slot except for SSB slots. The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown below.

Figure 1 The diagram about DL RMC for Low-Low bands inter-band CA via switching

Proposal 1: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

1-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

1-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

1-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

1-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 1.

Based on the following formulas for switching gap calculations, it’s proposed to design the following timing relationship and values.

FDD to SDL switch:

when FDD UL trailing end transient period - TTA * 106 > 0: actual switching gap length ≥ ceil[ (Tswitch + RTDP2S + FDD UL trailing end transient period- TTA * 106)/(1/14*103)] when FDD UL trailing end transient period - TTA * 106≤0: actual switching gap length ≥ ceil[(Tswitch + RTDP2S)/(1/14*103)] where the RTDP2S is the receiving timing difference among the closest DL slot timing boundaries between PCell and SCell, with the range of negative MRTD to positive MRTD. And the MRTD = 3us assuming that the FDD and SDL carriers are colocated. UL transient period is assumed as 10us.

SDL to FDD switch

Actual switching gap length ≥ ceil[ ( Tswitch + TTA * 106+ RTDS2P + FDD UL leading transient period ) / (1/14*103) ] Where RTDS2P is the receiving timing difference among the closest DL slot timing boundaries between SCell and PCell, with the range of negative MRTD to positive MRTD. And MRTD = 3us assuming that the FDD and SDL carriers are colocated. In addition, RTDS2P = - RTDP2S. UL transient period is assumed as 10us.

Proposal 2: to design the following timing relationship and values according to the updated formulas for switching gap calculations.

| RTDP2S | µs | 0 |
| --- | --- | --- |
| RTDS2P | µs | 0 |
| NTA | Tc | 0 |
| NTA,offset | Tc | 25600 |

| swithcingPeriodForFDD-SDL (Tswitch) | µs | 35 | 70 | 140 |
| --- | --- | --- | --- | --- |
| gapDurationPCelltoSCell-r19 | in units of 15KHz SCS symbol | 1 | 1 | 2 |
| gapDurationSCelltoPCell-r19 | in units of 15KHz SCS symbol | 1 | 2 | 3 |

2.2 Test methodology for the UL RMC

Based on the WF [3] in the RAN4#116 meeting, RAN4 need to discuss whether an UL RMC is needed for this feature or not.

Issue 2: Uplink reference measurement channels Proposed Agreement: - RAN4 can further discuss whether an UL RMC is needed for this feature

Referring to the clause A.2.1 of TS 38.101-1, only TDD active uplink slots are specified for TDD bands considering UL/DL TDD pattern. For FDD bands, there is no pattern assumed for UL testing. However, for LB-LB CA via switching, a semi-static pattern with 40bits map should be specified for UL requirements testing, especially for the UL time mask.

Observation 4: For LB-LB CA via switching, a semi-static pattern with 40bits map should be specified for UL requirements testing, especially for the UL on/off time mask.

Referring to the clause A.2.2 of TS 38.101-1, the UL RMCs with PUSCH mapping Type A (14 PUSCH symbols in one UL slot) are applicable to both FDD band and TDD band. For LB-LB CA via switching, the switching gap should be considered in the UL switch-from slot of the PCell FDD band, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively.

Observation 5: For LB-LB CA via switching, the switching gap should be considered in the UL switch-from slot of the PCell FDD band, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively.

Generally, it’s very important to verify the UL transmitting signal’s performance for UE before and/or after the Switching gap. RAN4 could explore whether there is a test methodology to verify the UL transmitting signal’s performance for UE before and/or after the Switching gap(s).

Proposal 3: to specify the following aspects in order to complete the UL RMC for LB-LB CA via switching:

3-1: to specify LBCA-SwitchingPattern for UL RMC of LB-LB CA via switching.

3-2: to specify the UL RMC in the UL switch-from slot of the PCell FDD band considering the switching gap, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively.

3-3: RAN4 explore the test methodology which is used to verify the UL transmitting signal’s performance for UE before and/or after the Switching gap(s).

3 Summary

2.1 Test methodology for the DL RMC

Observation 1: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching.

Observation 2: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Observation 3: the DL RMC for the LBCA-SwitchingPattern should include at least one SSB in PCell and one SSB in SCell and option 1 in the WF R4-2511862 can’t align the RAN1’s understanding on the restrictions for the semi-static switching pattern.

Proposal 1: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

1-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

1-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

1-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

1-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 1.

Proposal 2: to design the following timing relationship and values according to the updated formulas for switching gap calculations.

| RTDP2S | µs | 0 |
| --- | --- | --- |
| RTDS2P | µs | 0 |
| NTA | Tc | 0 |
| NTA,offset | Tc | 25600 |

| swithcingPeriodForFDD-SDL (Tswitch) | µs | 35 | 70 | 140 |
| --- | --- | --- | --- | --- |
| gapDurationPCelltoSCell-r19 | in units of 15KHz SCS symbol | 1 | 1 | 2 |
| gapDurationSCelltoPCell-r19 | in units of 15KHz SCS symbol | 1 | 2 | 3 |

2.2 Test methodology for the UL RMC

Observation 4: For LB-LB CA via switching, a semi-static pattern with 40bits map should be specified for UL requirements testing, especially for the UL on/off time mask.

Observation 5: For LB-LB CA via switching, the switching gap should be considered in the UL switch-from slot of the PCell FDD band, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively.

Proposal 3: to specify the following aspects in order to complete the UL RMC for LB-LB CA via switching:

3-1: to specify LBCA-SwitchingPattern for UL RMC of LB-LB CA via switching.

3-2: to specify the UL RMC in the UL switch-from slot of the PCell FDD band considering the switching gap, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively.

3-3: RAN4 explore the test methodology which is used to verify the UL transmitting signal’s performance for UE before and/or after the Switching gap(s).

Reference

[1] R2-2506533, “Introduction of low NR band carrier aggregation via switching”, Apple, Telus, Nokia

[2] Chair notes RAN1#122.

[3] R4-2511862,”WF on UE RF requirements for LB CA via switching”, Apple

[4] R4-2509246, Discussion on UE RF requirements for low band CA via switching, CATT

[5] TR 38.768 v2.0.0 on Low NR band carrier aggregation via switching Apple