3GPP TSG RAN WG4 Meeting #116bis		R4-2513170

Prague, Czech,  13 – 17 October 2025

Title: 	Discussion on non-concurrent n5 DL and n8 UL in Rel-19 to enable UL CA_n5-n8

Agenda Item:	5.9.2

Source: 	CATT

Document for:	Approval

1 Background

Referring to the draft LS [1] in last meeting, all the background information about CA_n5-n8 was listed below.

For carrier aggregation involving a low-low band combination where the UL of one band partially overlaps with the DL of the other band, such as CA_n5-n8, RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872 [2].: 1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands. In particular, Option 1 has already been specified in Rel-18, while RAN4 has agreed in RAN4#112bis to specify only Option 2 to support full band frequency range of each band in Rel-19 (See WF R4-2417125 [3]) with Option 3 excluded in Rel-19 which enables simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions. Moreover, standardizing Option 2 in Release 19 may necessitate a future-proof signalling design to accommodate the introduction of simultaneous operation capabilities for additional NR FDD-FDD inter-band CA combinations. These combinations may involve partial UL and DL overlap between different FDD bands or face duplexer-related challenges, as requested in the original liaison statement R4-2420410 [4].

The timeline about the discussion of CA_n5-n8 can be found below.

As discussed so far, RAN4 need to answer the following questions for this topic.

Q1: Did RAN4 agree to introduce option 2 to enable the UL CA_n5-n8? What’s the reason behind the agreement?

Q1a: is there impact on RRM or other WGs beyond the introduction of UE capability?

Q2: Is it necessary to introduce a UE capability for the FDD-FDD inter-band CA_n5-n8 case that support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19

In this paper, we’d like to answer these questions to address companies’ concerns except for the RRM impacts which will be discussed in a separate agenda.

2 Discussion

2.1 Why did RAN4 agree to introduce option 2 to enable UL CA_n5-n8 in Rel-19

Q1: Did RAN4 agree to introduce option 2 to enable the UL CA_n5-n8? What’s the reason behind the agreement?

A1: According to the WF R4-2417125 [3], RAN4 agree to specify option 2 to enable UL CA_n5-n8.

<Way forward/Agreement> - RAN4 agrees to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2) - 2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL - Send LS to RAN2 to complete necessary signalling designs Stop discussion on Option 3 in Rel-19. If option 3 is discussed in Rel-20 or later, new band numbers are considered.

As we all knows, the operators have strong demands to enable UL CA_n5-n8 Since Rel-18, but option 2 and option 3 were dropped in Rel-18 due to the limited time budget. As operators only own 10MHz in band n5 and n8 respectively, there is demand to enable the UL CA_n5-n8 in the Rural scenario in order to achieve higher UL throughput.

Observation 1: the operators shows the strong demands to enable UL CA_n5-n8 Since Rel-18.

In last meeting, some companies suggest to consider this case together with FDD-SDL dynamic switching in Rel-20. As the WF [6] for RAN4 led REL-20 5G-A topics was approved, the scope for dynamic low-band CA switching is limited. The main scenario is still for FDD-SDL low-band CA switching as it focuses on Dual DL solution instead of Dual UL solution.

- Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109): - Further discussion on the possible scoping in RAN#109 based on the following two objectives - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration - Dual DL solution - RAN1/2 impact is expected and will be further discussed in RAN#109 - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26

Finally, one new WID on enhancement of low NR band carrier aggregation via switching for Rel-20 has been approved in the RAN#109 meeting without any RAN1/RAN2’s impacts. The scope is limited with to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

Observation 2: The CA_n5-n8 was not included into the Rel-20 dynamic LBCA switching WID due to the limited TU budget and scope.

This means that if UL CA_n5-n8 was not agreed to be introduced in the Rel-19, there is also less chance to enable UL CA_n5-n8 in the Rel-20.

Proposal 1: according to observation 1 and 2, RAN4 introduce option 2 based on the approved WF R4-2417125 in Rel-19.

2.2 Is there impact on other WGs beyond the introduction of UE capability?

In the WF [8], the proponent proposed the following WF to address companies’ concerns and check whether there is impact on RRM or other WGs beyond the introduction of UE capability.

<Way forward>: For CA_n5-n8, further consider the following solution with side conditions to support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19: Support case 1 and case 2 via RRC reconfiguration. - Case 1: 2UL/2DL configuration, where NW shall keep all the four carriers activated, allocate resource allocation for a dedicated user only to n5 UL, n8 UL and n8 DL (no n5 DL resource allocation to a dedicated user) and both two bands are collocated. - Case 2: 1UL/2DL configuration Companies are encouraged to study necessity of additional RRM spec or additional requirement(s) of other working groups. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19. Whether new capability is needed or not would be further discussed.

According to the RAN2’s LS [5], option 2 is feasible without RAN2’s spec impacts beyond the introduction of UE capability.

Regarding whether to make necessary modifications in RAN2 specification to enable the described functionality, RAN2 agreement is as below: RAN2 understands that for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact (i.e., the necessary scheduling restriction is up to network implementation)

Observation 3: According to the RAN2’s LS R2-2501572, option 2 is feasible without RAN2’s spec impacts beyond the introduction of UE capability.

In the RAN4#115 meeting, companies comment that more analysis is needed how this UL/DL switching works, including related impacts on ACK/NACK.

Switching Work:

Based on the WF R4-2511850 [9], DL n5 PDSCH can be scheduled under the RRC configuration case 2 (DL_n5-n8_UL_n5) or the RRC configuration case 1 (DL_n5-n8_UL_n5-n8), and UL n8 PUSCH is only scheduled under the RRC configuration case 1 (DL_n5-n8_UL_n5-n8). As RRC configuration case 1 and case 2 can be switched based on RRC configuration and UE has enough time to switch the RRC configuration, there is no RAN1’s spec impact.

Impacts on ACK/NACK

For RRC configuration case 2, the ACK/NACK of PDSCH in DL band n5 or n8 can be carried in UL band n5 as other DL CA band combinations without UL SCell.

For RRC configuration case 1, network doesn’t need to send ACK/NACK of PUSCH in UL band n5 or n8 to UE. If UL retransition is needed, network can schedule UE to transmit PUSCH again.

Observation 4: Based on our analysis on Switching Work and ACK/NACK, there is no RAN1’s impact observed.

Proposal 2: according to the observation 3 and 4 based on the WF R4-2507910, there is no impact on other WGs beyond the introduction of UE capability.

Proposal 3: As the performance of the solution in the WF R4-2507910 is not as good as the dynamic Low-band CA switching, it is not precluded to introduce the dynamic Low-band CA switching for CA_n5-n8 in Rel-20.

2.3 Is it necessary to introduce a UE capability for the FDD-FDD inter-band CA_n5-n8 option 2 in Rel-19?

Based on the WF [3], RAN4 may discuss option 3 in Rel-20 or later. Based on the outcome of TR 38.872, it’s possible for a UE to support CA_n5-n8 with simultaneous Rx/Tx in partial UL frequency range of band n8 by implementing dedicated filter in band n8. But this option may be discussed in Rel-20 or later.

Additionally, apart from CA_n5-n8, CA_n26-n106 may have the similar issue that band n26 DL and band n106 UL can’t support con-current operation referring to the contribution [10].

Observation 5: apart from CA_n5-n8, RAN4 also identify the other case which has the similar issue, e.g. CA_n26-n106.

Based on the WF [4], it’s agreed that simultaneous Rx-Tx is mandatory without signaling for FDD-FDD band combinations by default, but special case like CA_n5-n8 need to clarify the non-concurrent operation between band n8 UL and n5 DL. From network deployment perspective, it is not expected that the scheduling restriction (non-concurrent operation between band n8 UL and n5 DL) is only captured in the NOTE of RAN4 specification. Thus, from forward compatibility perspective, it’s necessary to specify a UE capability to indicate the non-concurrent operation between band n8 UL and n5 DL.

Proposal 4: Due to forward compatibility perspective, potential option 3 for CA_n5-n8 in Rel-20 and other similar case CA_n26-n106, it’s necessary to specify a UE capability to indicate the non-concurrent operation between band n8 UL and n5 DL.

3 Summary

Observation 1: the operators shows the strong demands to enable UL CA_n5-n8 Since Rel-18.

Observation 2: The CA_n5-n8 was not included into the Rel-20 dynamic LBCA switching WID due to the limited TU budget and scope.

Proposal 1: according to observation 1 and 2, RAN4 introduce option 2 based on the approved WF R4-2417125 in Rel-19.

Observation 3: According to the RAN2’s LS R2-2501572, option 2 is feasible without RAN2’s spec impacts beyond the introduction of UE capability.

Observation 4: Based on our analysis on Switching Work and ACK/NACK, there is no RAN1’s impact observed.

Proposal 2: observation 3 and 4 based on the WF R4-2507910, there is no impact on other WGs beyond the introduction of UE capability.

Proposal 3: As the performance of the solution in the WF R4-2507910 is not as good as the dynamic Low-band CA switching, it is not precluded to introduce the dynamic Low-band CA switching for CA_n5-n8 in Rel-20.

Observation 5: Apart from CA_n5-n8, RAN4 also identify the other case which has the similar issue, e.g. CA_n26-n106.

Proposal 4: Due to forward compatibility perspective, potential option 3 for CA_n5-n8 in Rel-20 and other similar case CA_n26-n106, it’s necessary to specify a UE capability to indicate the non-concurrent operation between band n8 UL and n5 DL.

Reference

[1] R4-2506064, draft reply LS to RAN2 on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT

[2] TR 38.872, Study on enhancement for 700/800/900MHz band combinations, v18.4.0

[3] R4-2417125, WF on UL CA_n5-n8, CATT, China Telecom, Huawei, HiSilicon

[4] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT

[5] R2-2501572, Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT

[6] RP-251879, WF for RAN4 led REL-20 topics, RAN4 Chair (Apple)

[7] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)

[8] R4-2507910, WF on simultaneous Rx/Tx requirements, Huawei

[9] R4-2511850, WF on CA_n5_n8, CATT, Huawei

[10] RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE

[11] R4-2502840, “WF on simultaneous Rx-Tx”, Huawei