3GPP TSG-RAN WG4 Meeting #116bis                                                                        R4-2513179

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Considerations on BS RF and coexistence for 6GR

Agenda item:	8.5

Document for:	Approval

1 Introduction

Study on 6G Radio in RAN4 should be started in this meeting, in this contribution, we focus on BS RF and coexistence issue, share some views and proposals.

2 Discussion

UE centric network

UE centric network is a comprehensive design, which involves many aspects across different layers, including, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling, etc. Whether UE centric network will impact on networking layout, BS to BS interface, BS type, BS RF requirements needs further study in 6G stage.

Proposal 1: Whether UE centric network will impact on networking layout, BS to BS interface, BS type, BS RF requirements needs further study in 6G stage.

Duplexing

SBFD (Sub-band Full Duplex) was studied and standardized in 5G late stage, it is continued on SBFD-SBFD deployment scenario in Rel-20 5G-A phase. For evolution of duplexing, it is natural the next step is overlapped Sub-band Full Duplex, or even full-band Full Duplex, this is the final destination for duplexing in the foreseeable future.

However, as the SBFD has not deployed in real network yet and Rel-20 5G-A still in the progress on finding the feasible and valuable commercial scenarios. The TX to RX interference in the same equipment or between different equipments is still formidable technically.  Therefore, it is reasonable that 3GPP should start the new duplexing study until SBFD is mature in product and ready to deploy in real network.

Proposal 2: 3GPP new study on full-duplex should be based on SBFD deployment experience in real network.

New frequency range

New frequency range between 7GHz and 24GHz is expected to be introduced in 6G, then the frequency range will be continued. This new frequency range may be introduced as FR3 or something else. For BS requirements related to testing method for different frequency ranges, it is summarized as below table. For the new frequency range between FR1 and FR2, which kind of requirements should be defined, or is there an exact frequency point to differentiate conductive and radiated requirements need further study.

Table 1 Frequency range and BS RF requirements type

|  | FR1 | FR2-1 | FR3 or something |
| --- | --- | --- | --- |
| Frequency range | 410~7125MHz | 24.25GHz ~ 52.6GHz | TBD 7.125~24.25GHz |
| BS RF requirements type | BS type 1-C, BS type 1-H, BS type 1-O | BS type 2-O | BS type 3-O and/or BS type 3-H and/or BS type 3-C? |

Co-existence in some new frequency was initially studied in TR38.922, but as 6GR is studying further, some simulation parameters may need to be updated and coexistence study may need to be updated too.

Another issue needs to be taken care for new frequency range is whether these BS could be co-located with FR1 BS. In current BS specification, FR1 BS will be co-located with other FR1 BS, and FR2 BS has no co-located requirements. Whether this new frequency range follow FR1 or FR2 rules or whether there is an exact frequency point to differentiate co-location requirements needs further study.

Proposal 3: For new frequency range between FR1 and FR2, some aspects need further study as below:

- Which type of BS requirements should be defined and tested,

- Co-existence study with new parameters, such as bandwidth,

- Co-location requirements with FR1 band,

- And other issues.

Energy efficiency

Energy consumption is a big problem that troubled operator all over the world, in RAN1 study, it is a big topic for 6GR. From RAN4 perspective, we may improve it in some aspects.

- Introduce an energy efficiency metric for BS, for example, evaluate the total power consumption per throughput, the throughput can be the total bits. Some traffic model for energy efficiency can be introduced, such as full buffer, medium buffer and low buffer. From OPEX perspective, high energy efficiency in low buffer is more valuable for operator.

- Investigate whether it is possible to revisit some RF requirements, such as ACLR, EVM, which can help BS for energy saving.

Proposal 4: Energy efficiency can be considered for BS from below aspects:

- Introduce an energy efficiency metric for BS, such as total power consumption per throughput under certain traffic model.

- Investigate whether it is possible to revisit some RF requirements, such as ACLR, EVM, which can help BS for energy saving.

- And other aspects.

TN/NTN Co-existence

In previous TN/NTN coexistence study in [2], some of the assumptions may need to be revisited with NTN development, such as:

- The lowest satellite altitude was 600km, lower altitude such as 300km may be interested by satellite operators.

- Antenna array was not considered for satellite, it may be considered for LEO in the near future.

- An isolation distance (as in Figure 1) of 1500m as 2*ISD was considered to reflect the NTN-TN selection algorithm at the border, assuming TN connection will always be prioritized over NTN. Is this assumption still reasonable in the future should be revisited.

Figure 1 Isolation distance in [2]

- New NTN bands n250, n251 and n253 were introduced in RAN4, the DL frequency range of these bands is adjacent with TDD band n50, which means there will be cross link interference between these bands.

Table 2 Frequency band of n50 and n253

| TN and NTN operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |
| n253 | 1668 MHz – 1675 MHz | 1518 MHz – 1525 MHz | FDD |

Table 3 Interference case

| No. | Combination | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | TN with NTN | NTN DL | TN UL |
| 2 | TN with NTN | TN UL | NTN DL |

Proposal 5：Coexistence for TN/NTN study should be revisited for some assumptions as:

- Lower satellite altitude such as 300km,

- Antenna array for satellite,

- An isolation distance of 1500m

- Cross link interference for NTN DL to TN UL and TN UL to NTN DL

Others

In order to further improve network performance, higher order modulation such as 4096QAM can be studied for DL, in terms of system performance and implementation feasibility.

As in previous 5G-A discussion, BS advanced receiver can be further studied to enable UE higher power through TX EVM relaxation, however if the study show that it will increase receiver complexity significantly or have impact on system delay or the UE power improvement is limited, RAN4 need to make a decision whether it should be introduced in 6GR.

Proposal 6: Higher order modulation such as 4096QAM can be studied for DL.

BS advanced receiver can be further studied to enable UE higher power through TX EVM relaxation, RAN4 need to make a decision whether it should be introduced in 6GR based on the study results of receiver complexity, system delay and the value of UE power boost.

3 Conclusion

This contribution provides our views on 6G BS RF and coexistence issue. The following proposals are concluded as follows:

Proposal 1: Whether UE centric network will impact on networking layout, BS to BS interface, BS type, BS RF requirements needs further study in 6G stage.

Proposal 2: 3GPP new study on full-duplex should be based on SBFD deployment experience in real network.

Proposal 3: For new frequency range between FR1 and FR2, some aspects need further study as below:

- Which type of BS requirements should be defined and tested,

- Co-existence study with new parameters, such as bandwidth,

- Co-location requirements with FR1 band,

- And other issues.

Proposal 4: Energy efficiency can be considered for BS from below aspects:

- Introduce an energy efficiency metric for BS, such as total power consumption per throughput under certain traffic model.

- Investigate whether it is possible to revisit some RF requirements, such as ACLR, EVM, which can help BS for energy saving.

- And other aspects.

Proposal 5：Coexistence for TN/NTN study should be revisited for some new assumptions as:

- Lower satellite altitude such as 300km,

- Antenna array for satellite,

- An isolation distance of 1500m,

- Cross link interference for NTN DL to TN UL and TN UL to NTN DL

Proposal 6: Higher order modulation such as 4096QAM can be studied for DL.

BS advanced receiver can be further studied to enable UE higher power through TX EVM relaxation, RAN4 need to make a decision whether it should be introduced in 6GR based on the study results of receiver complexity, system delay and the value of UE power boost.

4 References

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- TR 38.863