3GPP TSG-RAN WG4 Meeting #116bis                                                                        R4-2513183

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Considerations on coexistence for UAV

Agenda item:	7.7.3

Document for:	Approval

- Introduction

In RAN #109 meeting, a new WI [1] for UAV was set up and RAN4 work should be started in this meeting. In this contribution, we focus on the objectives in SID, share some views and proposals on co-existence issue.

- Discussion

The objectives of the WID are shown as below, some co-existence study related items are highlighted.

The core part of WI: - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected

  - Deployment scenario

The main application scenarios of UAV include low altitude logistics, travel, tourism, control and supervision, medical rescue, etc. Although UAV above a certain altitude should be supervision, UAV UE still may be anywhere so UAV BS may need to supply a continuous coverage, which may be not the same as ATG service. In some rural areas, UAV BS ISD may be the same as NR BS ISD, but in urban areas, UAV BS should be sparser than NR BS, but they should have similar network layout. Below figures depict the network layout of UAV and NR for non-co-located case and co-located case with ISDUAV =2*ISDNR.

Figure 1 NR BS and UAV BS non-co-located with ISDUAV =2*ISDNR

Figure 3 NR BS and UAV BS co-located with ISDUAV =2*ISDNR

Proposal 1: If coexistence study should be done, UAV BS network layout should be clarified first. It is proposed to reuse NR outdoor layout for UAV BS.

With above network layout assumption, we give some other deployment parameters as below table.

Table 1 Deployment parameters

| Parameter | Descriptions |
| --- | --- |
| BS deployment | Hexagonal grid, 3 sectors per site with wrap around, all the UAV and NR BS are activated in one snapshot BS height: 30m Inter-NR BS distance: [500m] Inter-UAV BS distance: [4km] The number of macro and UAV sites according to ISD of NR and UAV BS |
| Carrier frequency | 2GHz, 4GHz |
| CBW | 20MHz for 2GHz, 100MHz for 4GHz |
| UL power control | Yes |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| UE dropping | For NR UE only outdoor in the ground, with minimum distance 1m For UAV UE, the height between 600~1000m, uniformly distributed, with minimum distance [20m] of 3D UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| BS Inter-site distance | MCL of 70dB for NR BS MCL of 80dB for UAV BS |
| Pathloss model | NR BS to UE: NLOS and LOS UMa in TR38.901 UAV BS to UE: Propagation model between TN UE and ATG UE in TR 38.876 |

Proposal 2: If coexistence study should be done, it is proposed to adopt Deployment parameters in Table 1.

2.2 Coexistence interference analysis

According to SID, UL throughput and UL coverage is the key target compared with DL, which may mean the TDD configuration for UAV BS may not be the same as NR BS, and will bring cross-link interference as in below figure. Below Table gives the interference cases which should be considered in co-existence study among which case 4 and 5 should be pay more attention and may be the worst two cases in all interference cases.

D

D

D

U

D

D

D

U

NR

UAV

Table 2 Interference cases between NR and UAV

| Case | Aggressor | Victim | Aggressor → Victim | Note |
| --- | --- | --- | --- | --- |
| 1 | NR BS DL | UAV UE DL | NR BS Tx →UAV UE Rx | Both for 2GHz FDD and 4GHz TDD |
| 2 | UAV BS DL | NR BS DL | UAV BS Tx →NR UE Rx |  |
| 3 | NR UE UL | UAV BS UL | NR UE Tx →UAV BS Rx |  |
| 4 | UAV UE UL | NR BS UL | UAV UE Tx →NR BS Rx |  |
| 5 | UAV UE UL | NR BS DL | UAV UE Tx →NR UE Rx | Only for 4GHz TDD, case 7 and 8 depend on the TDD configuration |
| 6 | NR BS DL | UAV BS UL | NR BS Tx →UAV BS Rx |  |
| 7 | UAV BS DL | NR BS UL | UAV BS Tx →NR BS Rx |  |
| 8 | NR UE UL | UAV UE DL | NR UE Tx →UAV UE Rx |  |

Proposal 3: The interference cases between NR and UAV are proposed in table 1, whether there is cross link inference as below should be clarified and can be studied later (case 5~8).

- between UAV and NR

- between different UAV operators

- Conclusion

This contribution provides our views on coexistence for UAV and NR. The following proposals are concluded as follows:

Proposal 1: If coexistence study should be done, UAV BS network layout should be clarified first. It is proposed to reuse NR outdoor layout for UAV BS.

Proposal 2: If coexistence study should be done, it is proposed to adopt Deployment parameters in Table 1.

Proposal 3: The interference cases between NR and UAV are proposed in table 1, whether there is cross link inference as below should be clarified and can be studied later (case 5~8).

- between UAV and NR

- between different UAV operators

- References

- RP-252954,  New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV)